Additional file 5: Table S4. GC and HPLC analysis of the ee or de values of products.

| Product | Columna | Conditions | Retention time (min) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Substrate | (S)-P | (R)-P |
| P1 | BGB-174 | 110 oC, 5 oC/min, 160 oC; Inc./dec. 240 oC; helium | 6.6 | 7.7 | 7.9 |
| P2 | BGB-174 | 110 oC, 5 oC/min, 160 oC Inc./dec. 240 oC; helium | 7.8 | 8.8 | 9.2 |
| P3 | OJ-H | n-hexane/isopropanol (97:3, v/v); flow rate: 0.2 mL/min; 254 nm | 9.1 | 18.9 | 20.2 |
| P4 | OJ-H | n-hexane/isopropanol (97:3, v/v); flow rate: 0.2 mL/min; 254 nm | 10.3 | 19.4 | 21.2 |
| P5 | OJ-H | n-hexane/isopropanol (97:3, v/v); flow rate: 0.2 mL/min;254 nm | 13.3 | 25.0 | 41.7 |
| P6 | BGB-174 | 140 oC; Inc./dec. 240 oC; helium | 5.1 | 6.3 | 6.8 |
| P7 | BGB-174 | 140 oC; Inc./dec. 240 oC; helium | 3.5 | 5.8 | 6.1 |
| P8 | BGB-174 | 140 oC; Inc./dec. 240 oC; helium | 3.6 | 6.1 | 6.3 |
| P9 | OJ-H | n-hexane/isopropanol (95:5); flow rate: 0.2 mL/min; 254 nm | 9.6 | 11.4 | 12.1 |
| P10 | BGB-174 | 110 oC, 20 min, 5 oC/min, 160 oC; Inc./dec. 240 oC; helium | 10.8 | 24.4 | 23.9 |
| P11 | BGB-174 | 110 oC, 20 min, 5 oC/min, 160 oC; Inc./dec. 240 oC; helium | 6.2 | 19.3 | 18.9 |
| P12 | BGB-174 | 110 oC, 0.5 oC/min, 125 oC; Inc./dec. 240 oC; helium | 7.1 | 25.0 | 24.7 |
| P13 | BGB-174 | 110 oC, 25 min, 5 oC/min, 160 oC, 2 min; Inc./dec. 240 oC; helium | 9.3 | 34.6 | 34.4 |
| P14 | BGB-174 | 120 oC; Inc./dec. 240 oC; helium | 3.8 | 7.1 | 6.8 |
| P15 | AD-H | n-hexane/isopropanol (95:5); flow rate: 1.0 mL/min; 218 nm | 4.9 | 8.1 | 7.9 |
| P16 | ODS | acetonitrile/water (1:3, v/v); flow rate: 1.0 mL/min; 220 nm | 11.4 | 10.3 | 9.8 |
| P17 | ODS | acetonitrile/water (1:3, v/v); flow rate: 1.0 mL/min; 220 nm | 17.1 | 14.2 | 11.4 |

a BGB-174 (30 m × 0.25 mm × 0.25 µm; BGB Analytik); Chiralcel OJ-H column (2.1 × 150 mm; Daicel Chemical Ind. Ltd., Japan); Chiralpak AD-H column (4.6 × 150 mm; Daicel Chemical Ind. Ltd., Japan); Hypersil ODS column (4.6 mm × 250 mm, Thermo, USA).