1. Supplementary table

Supplementary Table S1. Mesh-convergence analysis based on the maximum implant von Mises stress calculated in Model 1 under Loading Scenario 1 (Force 1)

| Nominal element size (mm) | Maximum implant von Mises stress (MPa) | Relative change (%) | Convergence assessment |
| --- | --- | --- | --- |
| 0.4 | 22.135 | — | Reference mesh |
| 0.3 | 24.701 | 10.39 | Above criterion |
| 0.2 | 26.357 | 6.28 | Above criterion |
| 0.1 | 26.983 | 2.32 | Acceptable (<3%) |

Relative change (%) = |σᵢ − σᵢ₋₁| / |σᵢ| × 100, where σᵢ is the result from the finer mesh and σᵢ₋₁ is the result from the preceding coarser mesh.