# Additional file 1
Supplementary methods and tables

Anterior implant support across four ZAGA-informed pathways in hybrid zygomatic rehabilitation: a finite element analysis

Muhammet Yasin Pektaş
Department of Oral and Maxillofacial Surgery, Faculty of Dentistry, Kütahya Health Sciences University, Kütahya, Türkiye

This document reports the locked real-CAD workflow. Pathway denotes the conceptual ZAGA-informed condition and trajectory its implemented computational centreline. The editable results-and-traceability workbook contains the numerical record. Internal P1–P4 identifiers map to public Trajectories A–D and are retained only for source-file provenance.

S1. Geometry and CAD provenance

The maxilla and zygomatic complex were derived from 63 female Visible Human Project CT images (c_vf1110–c_vf1172). Local indices refer to this cropped stack, starting at zero. Rows [55,285) and columns [75,437) were retained; stored values ≥1451 defined bone. Morphological closing used a spherical radius-two-voxel element, followed by largest-component selection. Local slices 59 onward were removed. An ovoid ring was added over slices [56,69), centered at x/y = 88/65 mm, with outer radii 40/58 mm, inner radii 30/48 mm and cutoff y <92 mm. Coordinates used 0.488281-mm in-plane and 1-mm axial spacing, with negative z along the stack. The patient-right half was mirrored about x =88 mm before resampling; discrete bilateral symmetry and connected-component selection followed. Supplementary Fig. S6 shows segmentation, inferior modification and the retained H4–A bone domain. JD and NTA CAD remained the implant authorities; unsmoothed surface conditioning and filled abutment channels were retained for the bonded-continuum model. The fixed spatial cortical ROI used the union of 7.15-mm tubes around the four site-specific axes.

The 40-mm centreline was sampled at 401 equally spaced points before implant insertion. Bone fraction is occupied samples divided by 401; contiguous endpoint occupancy gives engagement lengths at 0.1-mm resolution. Shared-face area equals adjoining titanium–bone faces multiplied by 0.658² mm² and includes cortical and trabecular interfaces. It describes the discretized bonded interface, not native threaded contact area or histological bone–implant contact. The common loaded-side coronal point was (61.4,29.2,−66.0) mm. Inclinations to the superior–inferior axis were 52.70°,54.16°, 61.45° and 62.21° for A–D. Cylindrical bone-clearance intervals were absent in A, t=0.20–0.40 in B (radius 2.55 mm), t=0.12–0.72 in C (3.05 mm), and t=0.05–0.75 in D (3.45 mm); t=0 denotes the coronal end. These intervals correspond to nominal lengths 0, 8, 24 and 28 mm, not measured sinus lengths. No separately validated sinus-lumen mask was available. The 39.79–45.37-mm boundary distances are perpendicular apex-to-plane distances, not force-path lengths.

Supplementary Table S1. Manufacturer-CAD authority

| Component | Source file | Implemented designation | Source SHA-256 | Conditioning |
| --- | --- | --- | --- | --- |
| JD zygomatic fixture | ZSZY43400.stl | ZY43400; nominal Ø4.3 × 40 mm | b99782a11a06b3fd2d2d0cbb866ec5724c25332d9857ee20e2c386638b9f7a92 | Quadric decimation; no smoothing |
| JD angulated conical abutment | ZEVCA4545_45deg_H4p5_CLEANED.stl | EVCA4545C; 45°, H 4.5 mm | 5346585b6601024d5137a3dcc967bcf6d3c0c64870f50817671f76329dfc0793 | 0.04-mm filled-envelope reconstruction |
| NTA conventional fixture | FNM40S L10.stp | O-FNM 4010 S; nominal Ø4.2 × 10 mm | 9cb1e3088664cae9cc68339040a655e4d716a8c867fdc4948e39afe5b115d025 | STEP tessellation; quadric decimation; no smoothing |
| NTA straight MUA | O-NTMA(R) 5030.stp | Regular Ø4.8 mm; gingival height 3 mm | 3b1286dc8bed53212b86deec79b78a2aedcb130e4078a8c68a18c3e25923ec96 | STEP tessellation; 0.04-mm filled-envelope reconstruction |

Supplementary Table S2. Loaded patient-right trajectory geometry at 0.658 mm

| Trajectory | Length (mm) | Centreline bone fraction | Coronal engagement (mm) | Apical engagement (mm) | Shared-face area (mm²) |
| --- | --- | --- | --- | --- | --- |
| A | 40.0 | 0.516209 | 12.4 | 8.0 | 413.048 |
| B | 40.0 | 0.411471 | 8.1 | 8.2 | 352.866 |
| C | 40.0 | 0.391521 | 4.5 | 11.0 | 326.022 |
| D | 40.0 | 0.296758 | 1.8 | 9.9 | 211.719 |

S2. Materials, interfaces, boundary conditions, and loads

All materials were linear, homogeneous, and isotropic. Interfaces were fully bonded through shared nodes and faces. Filled abutment channels represented monolithic bonded envelopes; screws, preload, friction, separation, and micromotion were outside the model scope. The released analysis metadata assigned Co–Cr properties to the framework (218 GPa, ν = 0.33); this model input does not infer a manufacturer alloy grade from CAD. The same idealized titanium properties were assigned to all fixture and MUA domains. Superior-surface nodes were fixed in all translations on z = −1.974 mm; perpendicular plane-to-zygomatic-apex distances were 39.79, 40.60, 44.91, and 45.37 mm for Trajectories A–D.

Supplementary Table S3. Material properties

| Material | Elastic modulus (MPa) | Poisson ratio |
| --- | --- | --- |
| Cortical bone | 13700 | 0.30 |
| Trabecular bone | 1370 | 0.30 |
| Idealized titanium | 110000 | 0.35 |
| Modeled Co–Cr framework | 218000 | 0.33 |
| PMMA | 2700 | 0.35 |

Supplementary Table S4. Loading protocols

| Protocol | FDI | Force (N) | Patch radius (mm) | Direction |
| --- | --- | --- | --- | --- |
| Posterior vertical | 14 | 28 | 4.0 | Global vertical |
| Posterior vertical | 15 | 94 | 4.0 | Global vertical |
| Posterior vertical | 16 | 108 | 4.0 | Global vertical |
| Canine oblique | 13 | 93 | 3.5 | 45° between global vertical and local palatal-to-buccal direction |

S3. Discretization and numerical verification

The production mesh used 0.658-mm C3D8 elements. CalculiX assembled the scaled symmetric positive-definite system; SciPy conjugate gradients with a PyAMG smoothed-aggregation V-cycle solved the free equations at rtol = 1 × 10⁻⁹. Production acceptance required normal completion, finite fields, relative L2 residual ≤1.05 × 10⁻⁹, and force imbalance ≤1 × 10⁻⁸. Case metadata describe geometry/deck release and are not the authority for the external production solver; production solver status and hashes are recorded in the released solver-audit table.

Supplementary Table S5. Selected mesh-gate comparisons

| Sentinel | Metric | Coarse (mm) | Fine (mm) | Coarse value | Fine value | ¦Change¦ (%) | Gate |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H4-Trajectory A vertical | bone_fixed_pmax_p95_mpa | 0.658 | 0.638 | 7.92722 | 8.09356 | 2.098 | PASS |
| H4-Trajectory A vertical | bone_fixed_compression_p05_mpa | 0.658 | 0.638 | 11.0447 | 11.2616 | 1.964 | PASS |
| H4-Trajectory A vertical | zygoma_vm_p95_mpa | 0.658 | 0.638 | 67.5119 | 66.6873 | 1.221 | PASS |
| H4-Trajectory A vertical | framework_umax_mm | 0.658 | 0.638 | 0.419753 | 0.419193 | 0.133 | PASS |
| H6-Trajectory D oblique | bone_fixed_pmax_p95_mpa | 0.658 | 0.638 | 5.74172 | 5.55646 | 3.227 | PASS |
| H6-Trajectory D oblique | bone_fixed_compression_p05_mpa | 0.658 | 0.638 | 5.26561 | 5.31621 | 0.961 | PASS |
| H6-Trajectory D oblique | zygoma_vm_p95_mpa | 0.658 | 0.638 | 36.135 | 36.8467 | 1.970 | PASS |
| H6-Trajectory D oblique | framework_umax_mm | 0.658 | 0.638 | 0.32819 | 0.322461 | 1.746 | PASS |

Supplementary Table S6. Independent direct-solver verification on a 1.0-mm grid

| Outcome | AMG-PCG | SPOOLES direct | AMG vs direct (%) |
| --- | --- | --- | --- |
| Fixed-ROI cortical Pmax P95 (MPa) | 8.27826698 | 8.28174342 | -0.041977 |
| Fixed-ROI compression P05 (MPa) | 10.4093287 | 10.4090747 | +0.002440 |
| Loaded zygomatic-fixture VM P95 (MPa) | 67.5398388 | 67.5319028 | +0.011752 |
| Framework Umax (mm) | 0.419054694 | 0.419054713 | -0.000005 |

Supplementary Table S7. C3D8-to-C3D8I formulation comparison on a 1.0-mm grid

| Outcome | C3D8 | C3D8I | Change (%) |
| --- | --- | --- | --- |
| fixed anatomical cortical pmax P95 | 5.37644 | 5.34775 | -0.534 |
| fixed anatomical cortical pmax P99 | 7.29203 | 7.36613 | +1.016 |
| fixed anatomical cortical pmax MAXIMUM | 17.2797 | 15.9317 | -7.801 |
| fixed anatomical cortical compression P95 | 5.53605 | 5.5918 | +1.007 |
| fixed anatomical cortical compression P99 | 8.20397 | 8.2424 | +0.468 |
| fixed anatomical cortical compression MAXIMUM | 16.2365 | 14.9478 | -7.937 |
| loaded zygomatic fixture vm P95 | 21.8173 | 22.6348 | +3.747 |
| loaded zygomatic fixture vm P99 | 28.0587 | 29.3192 | +4.492 |
| loaded zygomatic fixture vm MAXIMUM | 32.3058 | 32.8448 | +1.668 |
| framework vm P95 | 15.4149 | 15.7879 | +2.420 |
| framework vm P99 | 20.7159 | 21.0785 | +1.750 |
| framework vm MAXIMUM | 27.9271 | 27.7629 | -0.588 |
| Global Umax | 0.3622 | 0.371208 | +2.487 |

Supplementary Table S8. Model-form sentinels

| Sentinel | Outcome | Reference | Comparison | Difference |
| --- | --- | --- | --- | --- |
| Cortical shell 2.0→1.0 mm | pmax p95 | 7.92722 | 11.5547 | +45.760% |
| Cortical shell 2.0→1.0 mm | pmax p99 | 11.3595 | 20.8323 | +83.391% |
| Cortical shell 2.0→1.0 mm | compression p05 | 11.0447 | 17.1301 | +55.098% |
| Cortical shell 2.0→1.0 mm | compression p01 | 18.1715 | 25.8831 | +42.438% |
| Bilateral symmetric-load noise | pmax p95 | 5.12156 | 5.1396 | 0.352% |
| Bilateral symmetric-load noise | pmax p99 | 8.41873 | 8.32369 | 1.129% |
| Bilateral symmetric-load noise | compression p05 | 16.458 | 16.4144 | 0.265% |
| Bilateral symmetric-load noise | compression p01 | 27.5968 | 27.6377 | 0.148% |

S4. Production contrasts and ROI sensitivity

Supplementary Table S9. Matched H6 versus H4 percentage changes

| Protocol | Trajectory | Outcome | H4 | H6 | Change (%) |
| --- | --- | --- | --- | --- | --- |
| vertical | A | Bone Pmax P95 | 7.92722 | 7.99827 | +0.896 |
| vertical | A | Bone compression P05 | 11.0447 | 11.0198 | -0.225 |
| vertical | A | ZI VM P95 | 67.5119 | 67.2722 | -0.355 |
| vertical | A | Framework Umax | 0.419753 | 0.421019 | +0.302 |
| vertical | A | Matched 12/22 fixture VM P95 | 6.22373 | 4.14017 | -33.478 |
| vertical | A | Matched 12/22 MUA VM P95 | 16.1146 | 12.2084 | -24.240 |
| vertical | A | Loaded zygomatic MUA VM P95 | 60.1147 | 61.3308 | +2.023 |
| vertical | A | Framework VM P95 | 25.6608 | 24.7982 | -3.361 |
| vertical | B | Bone Pmax P95 | 7.40381 | 7.46508 | +0.828 |
| vertical | B | Bone compression P05 | 10.7852 | 10.8323 | +0.436 |
| vertical | B | ZI VM P95 | 66.2153 | 66.0989 | -0.176 |
| vertical | B | Framework Umax | 0.417793 | 0.419077 | +0.307 |
| vertical | B | Matched 12/22 fixture VM P95 | 6.28981 | 4.16605 | -33.765 |
| vertical | B | Matched 12/22 MUA VM P95 | 16.3859 | 12.168 | -25.741 |
| vertical | B | Loaded zygomatic MUA VM P95 | 62.2893 | 61.2164 | -1.722 |
| vertical | B | Framework VM P95 | 26.2229 | 25.4257 | -3.040 |
| vertical | C | Bone Pmax P95 | 7.24582 | 7.33135 | +1.180 |
| vertical | C | Bone compression P05 | 10.8184 | 10.9426 | +1.148 |
| vertical | C | ZI VM P95 | 66.9865 | 66.8331 | -0.229 |
| vertical | C | Framework Umax | 0.425147 | 0.426256 | +0.261 |
| vertical | C | Matched 12/22 fixture VM P95 | 6.58452 | 4.50745 | -31.545 |
| vertical | C | Matched 12/22 MUA VM P95 | 16.7051 | 12.2513 | -26.662 |
| vertical | C | Loaded zygomatic MUA VM P95 | 67.5901 | 66.7167 | -1.292 |
| vertical | C | Framework VM P95 | 28.9887 | 27.963 | -3.538 |
| vertical | D | Bone Pmax P95 | 6.00382 | 6.03353 | +0.495 |
| vertical | D | Bone compression P05 | 12.1143 | 12.178 | +0.526 |
| vertical | D | ZI VM P95 | 77.6902 | 77.4701 | -0.283 |
| vertical | D | Framework Umax | 0.474181 | 0.475511 | +0.281 |
| vertical | D | Matched 12/22 fixture VM P95 | 6.88636 | 4.84512 | -29.642 |
| vertical | D | Matched 12/22 MUA VM P95 | 18.0674 | 12.9387 | -28.387 |
| vertical | D | Loaded zygomatic MUA VM P95 | 78.8399 | 78.1039 | -0.934 |
| vertical | D | Framework VM P95 | 29.0929 | 28.1147 | -3.362 |
| oblique | A | Bone Pmax P95 | 4.75218 | 4.71852 | -0.708 |
| oblique | A | Bone compression P05 | 5.27055 | 5.19607 | -1.413 |
| oblique | A | ZI VM P95 | 26.1009 | 26.3721 | +1.039 |
| oblique | A | Framework Umax | 0.297368 | 0.294122 | -1.091 |
| oblique | A | Matched 12/22 fixture VM P95 | 5.44284 | 4.24547 | -21.999 |
| oblique | A | Matched 12/22 MUA VM P95 | 18.4582 | 13.5626 | -26.523 |
| oblique | A | Loaded zygomatic MUA VM P95 | 31.9939 | 26.5193 | -17.111 |
| oblique | A | Framework VM P95 | 19.6485 | 17.4973 | -10.949 |
| oblique | B | Bone Pmax P95 | 4.75064 | 4.72254 | -0.592 |
| oblique | B | Bone compression P05 | 5.08112 | 5.03734 | -0.862 |
| oblique | B | ZI VM P95 | 25.6378 | 25.8373 | +0.778 |
| oblique | B | Framework Umax | 0.297689 | 0.294413 | -1.101 |
| oblique | B | Matched 12/22 fixture VM P95 | 5.51705 | 4.28715 | -22.293 |
| oblique | B | Matched 12/22 MUA VM P95 | 18.5374 | 13.622 | -26.516 |
| oblique | B | Loaded zygomatic MUA VM P95 | 32.7895 | 27.5086 | -16.105 |
| oblique | B | Framework VM P95 | 19.7543 | 17.5805 | -11.004 |
| oblique | C | Bone Pmax P95 | 5.00166 | 4.96661 | -0.701 |
| oblique | C | Bone compression P05 | 5.06996 | 4.984 | -1.695 |
| oblique | C | ZI VM P95 | 27.2971 | 27.5363 | +0.876 |
| oblique | C | Framework Umax | 0.306995 | 0.30343 | -1.161 |
| oblique | C | Matched 12/22 fixture VM P95 | 5.75032 | 4.53614 | -21.115 |
| oblique | C | Matched 12/22 MUA VM P95 | 18.7752 | 13.8135 | -26.427 |
| oblique | C | Loaded zygomatic MUA VM P95 | 34.982 | 29.9502 | -14.384 |
| oblique | C | Framework VM P95 | 19.9767 | 17.8953 | -10.419 |
| oblique | D | Bone Pmax P95 | 5.74887 | 5.74172 | -0.124 |
| oblique | D | Bone compression P05 | 5.29085 | 5.26561 | -0.477 |
| oblique | D | ZI VM P95 | 35.9793 | 36.135 | +0.433 |
| oblique | D | Framework Umax | 0.331494 | 0.32819 | -0.997 |
| oblique | D | Matched 12/22 fixture VM P95 | 5.63119 | 4.56405 | -18.951 |
| oblique | D | Matched 12/22 MUA VM P95 | 18.968 | 13.911 | -26.661 |
| oblique | D | Loaded zygomatic MUA VM P95 | 35.4403 | 29.7417 | -16.079 |
| oblique | D | Framework VM P95 | 20.0496 | 17.9489 | -10.478 |

Supplementary Table S10. Alternative-ROI Trajectory A-to-D changes in cortical Pmax P95

| Protocol | Support | ROI | A | D | A→D (%) |
| --- | --- | --- | --- | --- | --- |
| vertical | H4 | Equal-count | 9.4755 | 6.02434 | -36.422 |
| vertical | H4 | Dynamic 5 mm | 9.4755 | 5.88759 | -37.865 |
| vertical | H6 | Equal-count | 9.57721 | 6.00407 | -37.309 |
| vertical | H6 | Dynamic 5 mm | 9.57721 | 5.92373 | -38.148 |
| oblique | H4 | Equal-count | 5.159 | 6.61362 | +28.196 |
| oblique | H4 | Dynamic 5 mm | 5.159 | 6.11416 | +18.515 |
| oblique | H6 | Equal-count | 5.1269 | 6.54539 | +27.668 |
| oblique | H6 | Dynamic 5 mm | 5.1269 | 6.10334 | +19.045 |

S5. Production solver audit and figure provenance

Release status: PASSED. Production mesh: 0.658 mm; adjacent confirmation mesh: 0.638 mm. Maximum production relative L2 residual: 9.807e-10. Maximum force imbalance: 9.094e-11. Maximum selected primary mesh change: 3.227%. These values describe the original 2-mm baseline only; additional 1-mm-shell checks are reported separately in Tables S13–S15.

Released field figures use fixed class-specific scales: cortical Pmax 0–12 MPa, cortical compression 0–20 MPa, JD fixtures and MUAs 0–110 MPa, NTA fixtures 0–12 MPa, and NTA MUAs 0–45 MPa. Labels report the percentage above each ceiling; over-range values use the uppermost color. Component values are mapped only to the corresponding surface within one 0.658-mm cell diagonal. Interpolation, lighting, transparency, and smoothing are display-only. Figs. 1–2 contain no stress field. Fig. 3 combines implemented vectors, constrained surfaces, and mesh/ROI with nominal CAD context; sleeves and framework housings are illustrative, not solved interfaces. Numerical geometry and results remain unchanged.

Supplementary Table S11. Production solver audit

| Cell | Nodes | Elements | Iterations | Relative L2 residual | Force imbalance | Audit |
| --- | --- | --- | --- | --- | --- | --- |
| H4-A-V | 790985 | 609009 | 91 | 9.238e-10 | 7.820e-11 | PASS |
| H4-A-O | 790985 | 609009 | 92 | 8.664e-10 | 4.459e-11 | PASS |
| H4-B-V | 790841 | 608773 | 91 | 8.594e-10 | 7.977e-11 | PASS |
| H4-B-O | 790841 | 608773 | 92 | 8.304e-10 | 4.557e-11 | PASS |
| H4-C-V | 790317 | 607844 | 91 | 9.225e-10 | 7.561e-11 | PASS |
| H4-C-O | 790317 | 607844 | 91 | 9.720e-10 | 7.465e-11 | PASS |
| H4-D-V | 789800 | 606721 | 91 | 7.892e-10 | 8.462e-11 | PASS |
| H4-D-O | 789800 | 606721 | 91 | 9.807e-10 | 7.492e-11 | PASS |
| H6-A-V | 791129 | 609122 | 91 | 8.770e-10 | 9.094e-11 | PASS |
| H6-A-O | 791129 | 609122 | 92 | 8.155e-10 | 4.002e-11 | PASS |
| H6-B-V | 790985 | 608886 | 90 | 9.744e-10 | 7.002e-11 | PASS |
| H6-B-O | 790985 | 608886 | 92 | 9.405e-10 | 4.518e-11 | PASS |
| H6-C-V | 790461 | 607957 | 91 | 9.037e-10 | 8.890e-11 | PASS |
| H6-C-O | 790461 | 607957 | 92 | 8.105e-10 | 5.345e-11 | PASS |
| H6-D-V | 789944 | 606834 | 91 | 8.098e-10 | 9.052e-11 | PASS |
| H6-D-O | 789944 | 606834 | 92 | 8.421e-10 | 4.952e-11 | PASS |

Supplementary Table S12. Figure files

| File | Bytes | SHA-256 |
| --- | --- | --- |
| Figure_1.tif | 10312180 | ef0ae7694b7903e73419187e23feb79523a22f5afffb03992fd09d8a73055179 |
| Figure_2.tif | 6818936 | 59b120a55f5f69878465bb3d5eca3fe8b18f1b0062de12305c48b7d03611c199 |
| Figure_3.tif | 7435130 | 57a8d09ae9278a830a37fc81604f7b5debb347d9cfd7c67bdddf8ddb974344a4 |
| Figure_4.tif | 8014110 | 1c16532bf8b659e033e5566129500a9b07e930008874e4bd2f10b48d9a808fe5 |
| Figure_5.tif | 7951136 | a38154c267d5285901c71336a122cc46174eb25671ffc820808b4c6c5d301b1d |
| Figure_6.tif | 3402014 | 435edaff05e6b8c6b5018c52cba6e000dcf43f01374c051fd20e81d0b59cc9af |
| Figure_7.tif | 3331494 | 977117c59d63ae52817b0f1fe6b7a357be9266d5ebde6fb3d2778adcbc89696a |
| Additional_file_3.tif | 12719104 | d73c54917853952d88e686df8e70a8d59f2b68ac39c0aab2532bb4d19a6939c2 |
| Additional_file_4.tif | 12493914 | c45507819c3cbca760df677a9f180f1038d9bc6b33b3710045d14f6ebb33c0fe |
| Additional_file_5.tif | 1542250 | 9d1c13cc2361c41b7fc920bfbacb0023117cd128fb2738b0cbc7360aee006ddb |
| Additional_file_6.tif | 1545008 | 9aebfce407311f0c6ca97fdb5714e8e3a5d56195c05a043c9213b71f73919851 |
| Additional_file_7.tif | 11415892 | fac2b13237e22278767eaf961d8b1a56e227390d1066dfc54fae2c14cad723d8 |
| Additional_file_8.tif | 1536068 | 05ceda0c40c8a5280288659627174357ed049df4feb67a3b371fe2ec8e6a21b9 |

Supplementary figure legends

Supplementary Fig. S1. Loaded-side cortical compression under posterior vertical loading. (A–D) H4 with Trajectories A–D; (E–H) H6 with Trajectories A–D. H4 denotes two conventional plus two zygomatic implants; H6 denotes four conventional plus two zygomatic implants. Compression magnitude is max(0, −Pmin), where Pmin is minimum principal stress, on the fixed coordinate-space region of interest (ROI) using a common 0–20 MPa scale. Labels report fifth/first percentiles (P05/P01) and the percentage above 20 MPa; values above the ceiling use the uppermost color.

Supplementary Fig. S2. Loaded-side cortical compression under canine oblique loading. (A–D) H4 (two conventional plus two zygomatic implants); (E–H) H6 (four conventional plus two zygomatic implants). Layout, region of interest (ROI), camera, and the common 0–20 MPa scale match Supplementary Fig. S1.

Supplementary Fig. S3. Deterministic outcome profiles across categorical Trajectories A–D. (A–D) Vertical-protocol cortical maximum principal stress (Pmax) 95th percentile (P95), cortical compression fifth percentile (P05), loaded zygomatic-fixture von Mises P95, and maximum framework displacement (Umax); (E–H) corresponding oblique-protocol outcomes. Orange circles denote H4 (two conventional plus two zygomatic implants) and blue squares H6 (four conventional plus two zygomatic implants). For each protocol and outcome, H4 and H6 share a data-driven y-axis range; ranges differ between protocols. Curves may overlap where values are close. Points are deterministic solutions; no inferential statistics are shown.

Supplementary Fig. S4. Mesh sensitivity in (A) H4–Trajectory A vertical and (B) H6–Trajectory D oblique sentinels. H4 denotes two conventional plus two zygomatic implants; H6 denotes four conventional plus two zygomatic implants. Curves show absolute changes across adjacent spacings from 0.800 to 0.638 mm. Both panels use the same y-axis limit; the dashed line marks the 5% gate and the 0.658→0.638-mm confirmation pairs are highlighted. Coarser non-monotonic transitions are retained.

Supplementary Fig. S5. Coordinate-matched H6-minus-H4 cortical maximum principal stress (Pmax) fields. H4 denotes two conventional plus two zygomatic implants; H6 denotes four conventional plus two zygomatic implants. (A–D) Vertical and (E–H) oblique loading for Trajectories A–D. Signed coordinate-wise differences use common zero-centered symmetric limits; white denotes near-zero differences. These fields are not differences between separately summarized ROI P95 endpoints.

Supplementary Fig. S6. Anatomical model construction. A, cropped source CT bone segmentation before inferior modification; B, inferior truncation and addition of the ovoid ridge before mirroring; C, retained final H4–Trajectory A bone domain. The panels use a common orthographic camera and scale. Scale bars, 20 mm; arrows indicate projected superior and anterior model directions. No stress field is shown, and the solved geometry is unchanged.

# Cortical-shell sensitivity

The original H4–A cortical-thickness comparison is retained in Table S8. The expanded analysis repeats all 16 support–trajectory–load conditions with a 1.0-mm shell at 0.658 mm (Table S13), compares A–D contrasts at both thicknesses (Table S14), and checks H4–A vertical and H6–D oblique models at 0.638 mm (Table S15). Node coordinates, element connectivity, load and restraint sets, and implant–prosthesis material domains match the corresponding 2-mm models. Only cortical/trabecular labels and their assigned stiffness differ. The same fixed coordinate-space ROI rule was retained, but thinning changes which elements are cortical. These are deterministic material-partition sensitivity comparisons, not paired measurements in identical cortical tissue.

Supplementary Table S13. Primary outcomes with a 1.0-mm cortical shell

| Support | Trajectory | Load | Pmax P95 (MPa) | Compression P05 (MPa) | Fixture VM P95 (MPa) | Framework Umax (mm) |
| --- | --- | --- | --- | --- | --- | --- |
| H4 | A | Vertical | 11.555 | 17.130 | 80.528 | 0.6248 |
| H4 | B | Vertical | 11.108 | 16.707 | 79.651 | 0.6226 |
| H4 | C | Vertical | 11.546 | 16.524 | 77.917 | 0.6358 |
| H4 | D | Vertical | 9.048 | 18.244 | 85.796 | 0.7055 |
| H6 | A | Vertical | 11.705 | 17.139 | 80.427 | 0.6257 |
| H6 | B | Vertical | 11.153 | 16.658 | 79.517 | 0.6235 |
| H6 | C | Vertical | 11.606 | 16.541 | 77.815 | 0.6365 |
| H6 | D | Vertical | 9.120 | 18.230 | 85.438 | 0.7065 |
| H4 | A | Oblique | 8.144 | 8.328 | 30.265 | 0.4526 |
| H4 | B | Oblique | 8.153 | 7.905 | 29.731 | 0.4538 |
| H4 | C | Oblique | 8.731 | 7.773 | 30.756 | 0.4698 |
| H4 | D | Oblique | 10.061 | 8.022 | 39.593 | 0.5016 |
| H6 | A | Oblique | 8.085 | 8.257 | 30.606 | 0.4479 |
| H6 | B | Oblique | 8.106 | 7.840 | 30.051 | 0.4491 |
| H6 | C | Oblique | 8.682 | 7.724 | 30.908 | 0.4647 |
| H6 | D | Oblique | 10.046 | 7.996 | 39.492 | 0.4969 |

All models used the 0.658-mm grid and the original bonded interfaces and loading rules. The corresponding 2.0-mm results are reported in main Table 2. Compression is max(0, −Pmin P05). Full-precision values and paired percentage changes are supplied in Additional file 2.

Supplementary Table S14. Trajectory A-to-D contrasts at both cortical thicknesses

| Support | Load | Outcome | 2.0-mm shell | 1.0-mm shell |
| --- | --- | --- | --- | --- |
| H4 | Vertical | Cortical Pmax P95 | -24.26% | -21.69% |
| H4 | Vertical | Compression P05 magnitude | +9.68% | +6.50% |
| H4 | Vertical | Zygomatic VM P95 | +15.08% | +6.54% |
| H4 | Vertical | Framework Umax | +12.97% | +12.92% |
| H6 | Vertical | Cortical Pmax P95 | -24.56% | -22.08% |
| H6 | Vertical | Compression P05 magnitude | +10.51% | +6.37% |
| H6 | Vertical | Zygomatic VM P95 | +15.16% | +6.23% |
| H6 | Vertical | Framework Umax | +12.94% | +12.91% |
| H4 | Oblique | Cortical Pmax P95 | +20.97% | +23.55% |
| H4 | Oblique | Compression P05 magnitude | +0.39% | -3.67% |
| H4 | Oblique | Zygomatic VM P95 | +37.85% | +30.82% |
| H4 | Oblique | Framework Umax | +11.48% | +10.84% |
| H6 | Oblique | Cortical Pmax P95 | +21.68% | +24.26% |
| H6 | Oblique | Compression P05 magnitude | +1.34% | -3.16% |
| H6 | Oblique | Zygomatic VM P95 | +37.02% | +29.03% |
| H6 | Oblique | Framework Umax | +11.58% | +10.94% |

Change = 100×(D−A)/A. Interface area, endpoint and boundary separation covary. Across all four trajectories, exact ordering changed in 9 of 16 support–load–outcome combinations; the highest oblique compression shifted from D to A for both supports. Four of 32 H6–H4 primary contrasts changed sign. Full support contrasts are supplied in Additional file 2.

Supplementary Table S15. Grid sensitivity with a 1.0-mm cortical shell

| Model | Load | Outcome | 0.658 mm | 0.638 mm | Change |
| --- | --- | --- | --- | --- | --- |
| H4–A | Vertical | Cortical Pmax P95 | 11.555 | 11.585 | +0.261% |
| H4–A | Vertical | Compression P05 magnitude | 17.130 | 17.194 | +0.376% |
| H4–A | Vertical | Zygomatic VM P95 | 80.528 | 80.071 | -0.567% |
| H4–A | Vertical | Framework Umax | 0.6248 | 0.6310 | +0.992% |
| H6–D | Oblique | Cortical Pmax P95 | 10.046 | 9.711 | -3.341% |
| H6–D | Oblique | Compression P05 magnitude | 7.996 | 8.352 | +4.448% |
| H6–D | Oblique | Zygomatic VM P95 | 39.492 | 39.796 | +0.770% |
| H6–D | Oblique | Framework Umax | 0.4969 | 0.4934 | -0.701% |

Stress is reported in MPa and displacement in mm. All eight absolute changes were <5%; the largest was 4.4478%. These adjacent-grid tests include rerasterization effects and do not establish formal mesh independence.