Supplementary File S1
Structured Phantom-Assisted Abdominal Ultrasound Training Checklist

This checklist was used by faculty instructors to standardize the phantom-assisted simulation module. Training was delivered over 4 weeks, with 2 sessions per week and 90 minutes per session (total scheduled time, 12 hours), as part of the routine PGY2 curriculum.The checklist guided faculty demonstration, supervised hands-on practice, lesion-recognition exercises, real-time feedback, and confirmation of task completion.

Checklist instructions

- For each required task, the learner was expected to obtain a diagnostic-quality image, apply correct labeling, and save the minimum required number of images or clips.

- “Minimum dose” refers to the minimum amount of supervised deliberate practice required for that task.

- This supplementary file is intended to document the standardized instructional content and minimum practice requirements of the intervention. It is not a scoring sheet for the primary outcome.

A. Universal pre-scan and image-quality requirements

| Item | Requirement | Minimum dose | Documentation | Completed (Y/N) | Notes |
| --- | --- | --- | --- | --- | --- |
| A1 | Correct ergonomics, probe grip, screen alignment, and machine position. | Each session | Checked by instructor |  |  |
| A2 | Appropriate probe selection and basic optimization (depth, focus, overall gain/TGC). | Each session | Confirmed during scanning |  |  |
| A3 | Stable probe handling with controlled sweep, tilt, and rotation. | Throughout practice | Observed by instructor |  |  |
| A4 | Correct image labeling (organ, plane, and laterality when applicable). | Throughout practice | Shown on stored images |  |  |

- Required standard views for the five target organs/systems

These required views correspond to the predefined five target organs/systems emphasized during the intervention and were used to standardize faculty-led practice. To maintain consistency with the study-defined technical content, the view names below follow the prespecified standard views used in the phantom-based training module.

| Item | Organ/System | Required standard view | Minimum dose | Saved images | Completed (Y/N) | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| B1-1 | Left hepatic lobe and main portal vein | Longitudinal image of the left lateral hepatic lobe | ≥1 repetition | ≥1 stored image |  |  |
| B1-2 | Left hepatic lobe and main portal vein | Measurement image of the left lateral hepatic lobe | ≥1 repetition | ≥1 stored image |  |  |
| B1-3 | Left hepatic lobe and main portal vein | Oblique subxiphoid view of the liver | ≥1 repetition | ≥1 stored image |  |  |
| B1-4 | Left hepatic lobe and main portal vein | Left portal vein with its transverse and vertical portions | ≥1 repetition | ≥1 stored image |  |  |
| B1-5 | Left hepatic lobe and main portal vein | Second hepatic hilar section showing the portal vein confluence and inferior vena cava | ≥1 repetition | ≥1 stored image |  |  |
| B1-6 | Left hepatic lobe and main portal vein | Diameter measurement of the main portal vein | ≥1 repetition | ≥1 stored image |  |  |
| B2-1 | Right hepatic lobe and main portal vein | Second hepatic hilar section showing the portal vein confluence and inferior vena cava | ≥1 repetition | ≥1 stored image |  |  |
| B2-2 | Right hepatic lobe and main portal vein | Maximum oblique diameter measurement of the right hepatic lobe | ≥1 repetition | ≥1 stored image |  |  |
| B2-3 | Right hepatic lobe and main portal vein | Segmentation view of hepatic segments S4–S8 | ≥1 repetition | ≥1 stored image |  |  |
| B2-4 | Right hepatic lobe and main portal vein | Diameter measurement of the main portal vein | ≥1 repetition | ≥1 stored image |  |  |
| B2-5 | Right hepatic lobe and main portal vein | Intercostal oblique view of the right anterior hepatic lobe | ≥1 repetition | ≥1 stored image |  |  |
| B2-6 | Right hepatic lobe and main portal vein | Intercostal oblique view of the right posterior hepatic lobe | ≥1 repetition | ≥1 stored image |  |  |
| B3-1 | Biliary system | Long-axis view of the gallbladder | ≥1 repetition | ≥1 stored image |  |  |
| B3-2 | Biliary system | Short-axis view of the gallbladder | ≥1 repetition | ≥1 stored image |  |  |
| B3-3 | Biliary system | Oblique subxiphoid view of the left intrahepatic bile duct | ≥1 repetition | ≥1 stored image |  |  |
| B3-4 | Biliary system | Intercostal oblique view of the right intrahepatic bile duct | ≥1 repetition | ≥1 stored image |  |  |
| B3-5 | Biliary system | Long-axis view of the extrahepatic bile duct | ≥1 repetition | ≥1 stored image |  |  |
| B3-6 | Biliary system | Short-axis view of the distal common bile duct | ≥1 repetition | ≥1 stored image |  |  |
| B3-7 | Biliary system | Long-axis view of the distal common bile duct | ≥1 repetition | ≥1 stored image |  |  |
| B3-8 | Biliary system | Long- and short-diameter measurement of the gallbladder | ≥1 repetition | ≥1 stored image |  |  |
| B3-9 | Biliary system | Diameter measurement of the common bile duct | ≥1 repetition | ≥1 stored image |  |  |
| B4-1 | Pancreas | Long-axis pancreatic view including the head and body | ≥1 repetition | ≥1 stored image |  |  |
| B4-2 | Pancreas | Long-axis pancreatic view including the body and tail | ≥1 repetition | ≥1 stored image |  |  |
| B4-3 | Pancreas | Longitudinal view of the pancreatic neck and uncinate process | ≥1 repetition | ≥1 stored image |  |  |
| B4-4 | Pancreas | Longitudinal view of the pancreatic head and distal common bile duct | ≥1 repetition | ≥1 stored image |  |  |
| B4-5 | Pancreas | Left intercostal oblique view of the pancreatic tail via the splenic hilum | ≥1 repetition | ≥1 stored image |  |  |
| B4-6 | Pancreas | Anteroposterior diameter measurement of the pancreas | ≥1 repetition | ≥1 stored image |  |  |
| B4-7 | Pancreas | Diameter measurement of the pancreatic duct | ≥1 repetition | ≥1 stored image |  |  |
| B5-1 | Kidney | Coronal view of the right kidney | ≥1 repetition | ≥1 stored image |  |  |
| B5-2 | Kidney | Transverse view of the right kidney | ≥1 repetition | ≥1 stored image |  |  |
| B5-3 | Kidney | Long-axis measurement of the right kidney | ≥1 repetition | ≥1 stored image |  |  |
| B5-4 | Kidney | Transverse measurement of the right kidney | ≥1 repetition | ≥1 stored image |  |  |
| B5-5 | Kidney | Transverse hilar vascular view of the right kidney | ≥1 repetition | ≥1 stored image |  |  |

C. Lesion-recognition tasks on the abdominal phantom

For each lesion task, the learner was expected to localize the lesion, obtain two orthogonal views, optimize image quality, label the images, perform measurement when applicable, and provide a brief preliminary impression.

| Item | Domain | Required task | Minimum dose | Saved images | Completed (Y/N) | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| C1-1 | Liver | Identify and document a cystic hepatic lesion. | ≥1 lesion | 2 images (2 views) |  |  |
| C1-2 | Liver | Identify and document a solid hepatic lesion. | ≥1 lesion | 2 images (2 views) |  |  |
| C2-1 | Gallbladder / biliary system | Detect a gallbladder lesion. | ≥1 lesion | 2 images (2 views) |  |  |
| C2-2 | Gallbladder / biliary system | Recognize biliary dilatation or an abnormal duct pattern. | ≥1 lesion | 2 images (2 views) |  |  |
| C3-1 | Spleen | Identify and document a cystic splenic lesion. | ≥1 lesion | 2 images (2 views) |  |  |
| C3-2 | Spleen | Identify and document a solid splenic lesion. | ≥1 lesion | 2 images (2 views) |  |  |
| C4-1 | Pancreas | Identify and document a solid pancreatic lesion. | ≥1 lesion | 2 images (2 views) |  |  |
| C5-1 | Kidneys | Detect a renal cyst-like lesion. | ≥1 lesion | 2 images (2 views) |  |  |
| C5-2 | Kidneys | Identify renal calculi | ≥1 lesion | 2 images (2 views) |  |  |

D. End-of-session faculty documentation

| Item | Requirement | Timing | Record |
| --- | --- | --- | --- |
| D1 | Confirm that the minimum dose and image-saving requirements were met for all tasks practiced in that session. | End of each session | Yes / No |
| D2 | Archive representative images using the local naming convention. | End of each session | Verified |
| D3 | Provide 2–3 key feedback points and 1 improvement goal for the next session. | End of each session | Completed |