Table S1: Description of the methods and the reagents used on the assays performed in the study.

| Method | Basis of the method | Reagents (in home-made assays) or manufacture |  |  |
| --- | --- | --- | --- | --- |
|  |  | Reagent 1 |  | Reagent 2 |
| CUPRAC | The reduction of Cu2+ to Cu1+ caused by antioxidants in the sample | bathocuproinedisulfonic acid disodium salt in phosphate buffer |  | Cu(II) sulphate anhydrous in ultrapure water |
| FRAP/FRAS | The reduction of Fe3+ to Fe2+ caused by antioxidants in the sample | tripyridyltriazine, and ferric chloride hexahydrate in acetate buffer |  | none |
| TEAC | The reduction of 2,2′-azino- bis(3-ethylbenzthiazoline-6-sulfonic acid) (ABTS) radical to ABTS caused by the antioxidants present in the sample | ABTS, horseradish peroxidase (HRP), and H2O2 in phosphate buffer |  | none |
| Thiol | Reaction of thiols in the sample with ﻿5,5’-dithiobis-(2-nitrobenzoic acid) (DTNB) | Tris – EDTA Buffer |  | DTNB in phosphate buffer |
| PON-1 | Hydrolysis of phenylacetate into phenol | Tris and CaCl2 buffer |  | p-nytrophenyl acetate and methanol solution diluted with ultrapure water |
| GPx | Catalysis of the oxidation of glutathione by cumene hydroperoxide, and the reduction of the oxidise glutathione by presence of glutathione reductase | RANDOX |  |  |
| TOS | Estimation of the oxidation of ﻿Fe2+-o-dianisidine complexes to Fe3+ within the sample | Xylenol orange tetrasodium salt, NaCl and glycerol in H2SO4 solution |  | Ferrous ammonium sulfate and o-dianisidine dihydrochloride in H2SO4 solution |
| POX-Act | Measurement of total peroxides after a peroxide-peroxdiase reactiong using a chromogenic substrate; tetramethylbenzidine (TMB) | TMB and HRP in acetate buffer |  | none |
| d-ROMs | Based on Fenton’s reaction. Estimation of total peroxides by evaluating the N,N-dyethylparaphenyldiamine (DEPPD) radical cation concentration | Acetate buffer |  | DEPPD in ultrapure water |
| AOPP | Absorbance of oxidatively-modified albumin at 340 nm in the presence of potassium iodide (KI) in acidic conditions | KI in ultrapure water |  | Acetic acid |
| TBARS | Reaction of the sample to a trichloroacetic acid, thiobarbituric acid and ﻿N hydrochloric acid stock in heated conditions. | ﻿ trichloroacetic acid; thiobarbituric acid; hydrochloric acid | none |  |

All assays except TBARS, were conducted in the Olympus AU400, at a constant temperature of 37°C and in a reaction time between 5 and 8 minutes. Evaluation of TBARS was made manually using a microplate reader (Powerwave, XS, Biotek instruments).