Additional file 1 The final k13 dried blood spot (DBS) extraction protocol

|  | Home-made* Buffers | Commercial Buffer Substitutes |
| --- | --- | --- |
| Lysis: | 3M Guanidine thiocyanate 16.7% Isopropanol 2% Triton X100 10mM EDTA 5mM Trizma HCl pH 7.4 0.1% 6N HCl 0.5% 2-mercaptoethanol pH 6.0-6.5 | Qiagen RLT-plus 16.7% Isopropanol 0.5% 2-mercaptoethanol |
| Wash 1: | Same as lysis but no 2-mercaptoethanol | Same as lysis but no 2-mercaptoethanol |
| Wash 2: | 25% Ethanol 25% Isopropanol 100mM Sodium Chloride 10mM Trizma HCl pH 7.4 | 70% ethanol, 30% PBS |

NOTE: Lysis and Wash 1 contain guanidine thiocyanate which is TOXIC and INCOMPATIBLE with bleach. Lysis contains 2-mercaptoethanol which is TOXIC and should be used in a fume hood.

* Instructions on how to make large batches of these buffers (and proper contamination control measures), as well as sources of all reagents/materials have been previously published [17].

- Cut a 50µl DBS into small enough pieces to fit into a 96 well deep well plate

  - Take proper decontamination precautions to prevent cross-contamination from cutting: https://www.youtube.com/watch?v=K129KmLh_Ok

- Add 900µl of lysis buffer using a multichannel pipette

- Seal plate forcefully and centrifuge briefly to collect samples to the bottom

- Place in shaking incubator for 2 hours (60-65 °C, 250RPM)

- While waiting, place Nunc 961,2 well DNA plate on top of a 2ml 96 well deep well plate

- After incubation is finished, centrifuge briefly to collect samples to the bottom

- Transfer 750µl of the lysate to Nunc DNA plate

  - Use of plate covers will help prevent contamination from pipetting at this step

- Spin 3,700RPM for 1 minute

- Pipette 500µl of Wash 1

- Spin 3,700RPM for 1 minute

- Pipette 500µl of Wash 2

- Spin 3,700RPM for 2 minute

- Place DNA plate in 60-65 °C for 10 minutes to dry

- Place DNA plate on top of 0.5ml 96 well plate

- Pipette 50µl of TE buffer pH 8.0

- Spin 3,700RPM for 2 minute

1 Omega EZ 96-well DNA plates are also compatible, though display slightly reduced sensitivity for k13 amplification at parasitaemias below 5,000 parasites/mL.

2 If performing only a few extractions, individual DNA spin columns are also available from Omega Biotek (VWR 95043-210).