Table The PM10, SO2, NO2, CO, and O3 concentration averaging periods and their limits in the Regulation on conditions for monitoring and air quality requirements

| Air pollutant | Concentration | Averaging period |  | Permitted exceedances |
| --- | --- | --- | --- | --- |
| Particulate matter (PM10) | 50 µg/m3 |  | daily average | 35 |
| Particulate matter (PM10) | 40 µg/m3 |  | 1 year average |  |
| Sulphur dioxide (SO2) | 350 µg/m3 |  | 1 hour average | 24 |
| Sulphur dioxide (SO2) | 125 µg/m3 |  | daily average | 3 |
| Sulphur dioxide (SO2) | 50 µg/m3 |  | 1 year average |  |
| Nitrogen dioxide (NO2) | 150 µg/m3 |  | 1 hour average | 18 |
| Nitrogen dioxide (NO2) | 85 µg/m3 |  | daily average |  |
| Nitrogen dioxide (NO2) | 40 µg/m3 |  | 1 year average |  |
| Carbon monoxide (CO) | 10 mg/m3 |  | maximum daily 8hrs average |  |
| Carbon monoxide (CO) | 5 mg/m3 |  | daily average |  |
| Ozone (O3) | 120 µg/m3 |  | maximum daily 8hrs average | 25 days averaged over |

Suspended particles were in 2019 the dominant pollutant in the Republic of Serbia and exceedances of the daily limit values were at all measuring points. Exceedances of the yearly limit values were at 15 measuring points. Exceedances of the yearly limit values SO2 was only in Bor. Exceedances of the yearly limit values NO2 were only on two measuring points in Belgrade. Exceedances of the daily limit values CO were in Vranje and Zaječar. Exceedances of the target values O3 were at 16 measuring points, but exceedances of permitted number were only at two measuring points Kopaonik and Kamenički Vis-EMEP.