Instrument optimization

Tables S1 - S2 summarize the optimal operational parameters for the Inductively Coupled Plasma-Mass Spectrometry (ICP-MS) and Laser Ablation-ICP-MS systems used in the present work.

Table S1: Optimal operating parameters of wet plasma experimental systems

| ICP-MS |  |
| --- | --- |
|  | Perkin Elmer ELAN DRC-e |
| Plasma conditions |  |
| Incident RF power | 1150 Watts |
| Cooling gas flow rate | Ar 10.0 L min−1 |
| Auxiliary gas flow rate | Ar 0.95 L min−1 |
| Nebulizer gas flow rate | Ar 0.70 L min−1 |
| Detection mode | Dual |
| Auto lens | On |
| Scanning mode | Peak hopping |

Table S2: Optimized operating parameters of the dry plasma and laser ablation experimental systems

| ICP-MS |  |
| --- | --- |
|  | Perkin Elmer ELAN DRC-e |
| Plasma conditions |  |
| Incident RF power | 1150 Watts |
| Cooling gas flow rate | Ar 10.0 L min-1 |
| Auxiliary gas flow rate | Ar 0.95 L min-1 |
| Nebulizer gas flow rate | Ar 0.70 L min-1 |
| Detection mode | Dual |
| Auto lens | On |
| Scanning mode | Peak hopping |

| Laser Ablation |  |
| --- | --- |
| Laser ablation parameters |  |
| Laser | Nd:YAG |
| Wavelength | 213 nm |
| Repetition rate | 20 Hz |
| Energy | 750 Volt |
| Spot size | 25 m |
| Scan rate | 40 m sec-1 |
| Carrier gas flow | 0.4 L min-1 |