Metabolite extraction

Take out the sample at -80 ℃, add 500 ml of methanol acetonitrile aqueous solution(2:2:1, v/v)respectively, vortex for 60 sec, low temperature ultrasound for 30 min for twice time, place at -20 ℃ for 1hour to precipitate protein, 14000rpm, centrifuge at 4 ℃ for 20min, take the supernatant and freeze dry.

LC-MS analysis

HPLC condition

The sample was separated by Agilent 1290 Infinity LC ultra-high performance liquid chromatography system. Mobile phase: liquid A is 10 mM ammonium acetate aqueous solution, liquid B is acetonitrile. The sample is placed in a 4 ℃ automatic sampler, the column temperature is 45 ℃, and the flow rate is 300 μL/min, injection volume 2 μL. The relevant liquid phase gradient is as follows: 0-18 min, liquid B changes linearly from 90% to 40%; 18-18.1 min, liquid B changed linearly from 40% to 90%; 18.1-23 min, liquid B maintained at 90%. In the sample queue, one QC sample is set for each certain number of experimental samples at an interval to detect and evaluate the stability and repeatability of the system; The standard mixture of energy metabolism substances is set in the sample queue for correction of chromatographic retention time.

Mass spectrometric analysis

The 5500 QTRAP mass spectrometer (AB SCIEX) was used for mass spectrometry analysis in negative ion mode. 5500 QTRAP ESI source conditions are as follows: source temperature 450 ℃, ion Source Gas1 (Gas1): 45, Ion Source Gas2 (Gas2): 45, Current gas (CUR): 30, ionSapry Voltage Floating (ISVF) - 4500 V; Use MRM mode to detect ion pairs to be measured.

Data analysis

Multiquant software was used to extract the chromatographic peak area and retention time. The retention time was corrected with the standard of energy metabolism substances, and the metabolites were identified.