Multiplexed-IHC (OPAL™, AKOYA Biosciences, MA, USA) staining protocol, image acquisition and data analysis.

STAINING PROTOCOL

- Baked in the oven at 65⁰C for 1 hour.

- Dewaxed with xylene (3 x 10 min).

- Rehydrated through a series of ethanol solutions: (100% 1 x 10 min; 95% 1 x 10 min; and rinsed in 70%).

- Rinsed slides with distilled water.

- Fixed in 10% neutral buffered formalin for 20 min.

- Rinsed slides in distilled water and then in Antigen Retrieval (AR) buffer.

Step 1: Slide Preparation

- Placed the slides in appropriate AR buffer in Dako PT Link antigen retrieval system (Dako Denmark Aps, Denmark).

- Allowed slides to cool down at room temperature. Rinsed slides in distilled water followed by TBST to protect from drying out.

Step 2: Antigen Retrieval

- Covered the tissue section with blocking buffer and incubated in a humidified chamber for 10 min at room temperature.

Step 3: Blocking

- Drained off the blocking buffer and cover the tissue section with Primary Antibody Working Solution and incubated the slides at recommended time and temperature as per the antibody manufacturer's instructions.

- Rinsed the slides in TBST. Washed the slides 3 x 2 min in TBST at room temperature preferably with agitation.

Step 4: Primary Antibody Incubation

- Incubated slides with adequate volume (generally 100-300 μL per slide) of Polymer HRP Ms + Rb for 10 min at room temperature.

- Rinsed slides in TBST. Washed the slides 3 x 2 min in TBST at room temperature preferably with agitation.

Step 5: Introduction of Secondary-HRP (horseradish peroxidase)

- Drained off the excess wash buffer.

- Incubated each slide with 100-300 μL of Opal Fluorophore working solution at room temperature for 10 mins.

- Rinsed the slides in TBST. Washed the slides 3 x 2 min in TBST at room temperature preferably with agitation.

- Rinsed the slides with AR buffer.

Step 6: Opal Signal Generation

- THE STEP IS SAME AS STEP 2.

- STEP 3-7 repeated for the next antibody.

Step 7: Antibody Stripping

- Applied DAPI Working Solution for 5 min at room temperature in a humidity chamber.

- Washed the slides for 2 min in TBST buffer and then for 2 min in water.

- Covered the slides with Coverslip and with mounting medium.

Step 8: Counter stain and Mount

IMAGE ACQUISITION AND DATA ANALYSIS

Slide images were captured by the Quantitative Pathology Imaging System, Mantra™ (Perkin Elmer, MA) using Mantra 1.0.4 software. Color separation, Tissue and Cell Segmentation, and Cell Phenotyping were performed on inForm® Sofware v2.5.0 (Perkin Elmer, MA) to extract image data (.im3 files).