Effect of fixing scar T1 relative to the blood

A numerical simulation was performed to study the blood-to-scar contrast variations associated with deviation of the actual scar T1 value, T1_actscar from that of the fixed values used to estimate the imaging parameters (i.e, T1scar = 0.7 × T1blood). Similar to the simulation described in the main text, 10000 different combinations of tissues were generated by randomly selecting (using uniform probability distribution) the tissue T1 values from different continuous ranges for the blood (T1blood= 250-500 ms), myocardium (T1myo= 300-600 ms) and scar (T1scar= 250-500 ms). Only values satisfying T1r<T1myo-100 (with r = blood or scar) were used. Other simulation parameters included cardiac cycle duration= 1000 ms, flip angle = 25o, TR = 5 ms, number of RF pulses = 24 per cardiac cycle.

GB-LGE simulation: The optimal timing parameters; i.e., D1, D2, and D3, are estimated using Equation (2) for each of the 10000 tissue combinations described above assuming T2= 50, 200 and 55 ms, for the myocardium, blood and scar (assuming T1scar = 0.7 × T1blood), respectively. For simplicity the steady state magnetization was set to 1. The optimal parameters were then used to simulate the blood signal, Sb, and scar signal, Ss, using Equation (1). The scar signal was simulated at different actual scar T1 values, T1_actscar, with T1_actscar/T1blood= 0.5-0.85 with a step of 0.05. The (mean±standard deviation) of the ratio Sb/Ss was then calculated, over all tissue combinations, for each  T1_actscar/T1blood ratio.

Conventional LGE simulation: The effect of  T1_actscar/T1blood on Sb/Ss in conventional LGE was simulated for comparison with that of GB-LGE. First, the inversion time was calculated as TI=T1myo×log⁡(2) for each of the simulated 10000 tissue combinations. Then, the ratio Sb/Ss was calculated, where the scar signal was calculated at the different T1_actscar/T1blood ratios. The mean± standard deviation of the Sb/Ss ratio over the different tissue combinations was then calculated.

Figure S1 (below) shows a plot of Sb/Ss versus T1_actscar/T1blood. The Sb/Ss values in the figure were normalized by the anticipated signal ratio (i.e., at T1_actscar/T1blood=0.7).  It can be noticed that variable T1_actscar/T1blood can result in variability in Sb/Ss in GB-LGE (0.54-1.73) and in conventional LGE (0.68-1.45).

In conclusion, fixing the ratio T1scar/T1blood in the signal model of GB-LGE results in a deviation of the blood-to-scar contrast from the prescribed one. However, such contrast variability is inherent to LGE sequences due to the heavily T1-weighted contrast mechanism in these sequences.

Figure S1. A plot of the simulated blood-to-scar signal ratio (Sb/Ss) in gray blood (GB-) LGE and conventional LGE at different ratios of scar-to-blood T1 values (T1_actscar/T1blood). Data points and error bars respectively represent mean and standard deviation of Sb/Ss calculated over 10000 different tissue combinations.

Effect of fixing T2

A numerical simulation was performed to study the tissue signal variations associated with deviation of the actual tissue T2 from that of the fixed values used to estimate the imaging parameters. First, a set of T2 values is randomly (using uniform probability distribution)  selected for the blood, myocardium and scar from the continuous ranges 180-220 ms, 40-55 ms, and 40-60 ms, respectively. The optimal timing parameters; i.e., D1, D2, and D3, are estimated using Equation (2) for each of the 1000 tissue combinations described above assuming T2= 50, 200 and 55 ms, for the myocardium, blood and scar, respectively. Then, Equation (1) was used to simulate each tissue signal twice using: (1) the actual T2, and (2) the assumed T2 values. In both cases, a Rician model was used to generate a noisy tissue signal with signal-to-noise ratio of 5, defined as (signal mean) / (noise SD). The (mean±SD) of each tissue signal was calculated (Table S1). A two-tail paired t-test was used to compare T2-related signal variations. The results showed that the discrepancy among the assumed and the actual T2 values had no significant effect on the tissue contrast in both the BB-LGE and GB-LGE (Table S1).

Table S1. Signal intensity of each tissue type in GB-LGE and BB-LGE.

|  | scar |  |  | blood |  |  | Myocardium |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Actual T2 | Assumed T2 | P | Actual T2 | Assumed T2 | P | Actual T2 | Assumed T2 | P |
| GB-LGE | 0.8 ±0.3 | 0.8 ±0.3 | 0.36 | 0.25 ±0.05 | 0.25 ±0.04 | 0.40 | 0.04 ±0.03 | 0.02 ±0.01 | 0.53 |
| BB-LGE | 0.8 ±0.3 | 0.8 ±0.3 | 0.63 | 0.08 ±0.04 | 0.08 ±0.03 | 0.54 | 0.05 ±0.04 | 0.02 ±0.01 | 0.67 |

Note—Values represent mean±SD of the tissue signal (arbitrary units) calculated using the actual T2 and the assumed T2. P values are for the hypothesis test of the equality of the two signal means (estimated using actual T2 vs. assumed T2).