| Additional file 1: Table S1. CMR acquisition details |  |  |
| --- | --- | --- |
|  | 4D flow CMR |  |  |
| No. of slices (orientation) |  | 27 (coronal) |  |
| Field of view, mm |  | 350x271x56 |  |
| ECG-gating |  | Retrospective |  |
| No. of reconstructed cardiac phases |  | 24 |  |
| No. of signal averages |  | 1 |  |
| Respiratory compensation |  | navigator |  |
| Typical navigator efficiency |  | 40-60% |  |
| Acquired spatial resolution (mm) |  | 2.4x2.4x2.4 |  |
| Reconstructed spatial resolution (mm) |  | 2.1x2.1x2.1 |  |
| Acquired temporal resolution, ms (SD) |  | 32.0 |  |
| Flip angle (°) |  | 10 |  |
| TE (ms) |  | 4.5 |  |
| TR (ms) |  | 8.0 |  |
| VENC (cm/s) |  | 80 |  |
| Typical scan time (minutes) |  | 12-18 |  |
| Acceleration methods |  | SENSE factor 1.5, RL direction, EPI factor 5, Segmentation factor 1 |  |
| SD; standard deviation, ms; milliseconds, mm; millimeter, TE, echo time; TR, repetition time; VENC, velocity encoding; SENSE, sensitivity encoding; AP. anterior-posterior; EPI, echo planar imaging readout; ECG, electrocardiogram |  |  |  |

| Additional file 1: Table S2. Correlation analysis between cross-sectional area of the TCPC segments and 4D flow CMR energetics |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | n | CSA (mm2/m2) | KEnorm_flow+length |  | ELnorm_flow+length |  |
|  |  |  | Correlation coefficient | p | Correlation coefficient | p |
| Fontan tunnel | 35 | 106 (91-134) | -0.80 | <0.001 | -0.78 | <0.001 |
| SVC | 31 | 96 (76-109) | 0.23 | 0.21 | -0.02 | 0.93 |
| LPA | 46 | 92 (70-113) | 0.03 | 0.85 | -0.31 | 0.034 |
| RPA | 49 | 88 (81-101) | -0.10 | 0.48 | -0.10 | 0.50 |
| Correlation coefficients represent Pearson or Spearman rank analysis. SVC, superior vena cava; LPA/RPA, left/right pulmonary artery; CSA, cross-sectional area normalized for body surface area; TCPC, total cavopulmonary connection; KE, kinetic energy; EL, viscous energy loss rate |  |  |  |  |  |  |

| Additional file 1: Table S3. Correlation analysis between demographic and CMR parameters with exercise capacity and liver cT1 mapping |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | cT1 |  | Peak VO2 (ml/kg/min) |  | % Predicted peak VO2 |  |
| Demographics |  | Correlation coefficient | p | Correlation coefficient | p | Correlation coefficient | p |
| Age at CMR, years |  | 0.254 | 0.048 | -0.42 | 0.003 | -0.29 | 0.039 |
| Time since Fontan, years |  | 0.192 | 0.14 | -0.41 | 0.003 | -0.32 | 0.024 |
| LV/biv vs RV |  |  |  | - | 0.76 | - | 0.15 |
| HLHS/non-HLHS |  | - | 0.73 | - | 0.96 | - | 0.38 |
| MR parameters |  |  |  |  |  |  |  |
| Cardiac index, L/min/m2 |  | 0.079 | 0.55 | 0.25 | 0.08 | -0.07 | 0.61 |
| Ejection fraction, % |  | -0.05 | 0.70 | 0.07 | 0.64 | 0.20 | 0.17 |
| 4D flow CMR energetics total TCPC |  |  |  |  |  |  |  |
| KEnorm_flow |  | 0.50 | 0.006 | -0.61 | 0.003 | -0.44 | 0.04 |
| ELnorm_flow |  | 0.39 | 0.04 | -0.54 | 0.009 | -0.46 | 0.033 |
| CMR; cardiovascular magnetic resonance, LV/RV, left/right systemic ventricle, biv; biventricular; HLHS, hypoplastic left heart syndrome; 4D, four dimensional; TCPC, total cavopulmonary connection; KE, kinetic energy; EL, viscous energy loss rate |  |  |  |  |  |  |  |

Additional file 1: Figure S1.

A flow chart of the study methods and performed analyses.

.