Table S4. Time-based quality metrics of MinION sequencing run 4 (n = 33)

| Sequencing run time for each batch of 20,000 reads (total run time) | Mean read quality | Reads Q ≥ 10 | % of reads Q ≥ 10 | Mean read Q≥10 |
| --- | --- | --- | --- | --- |
| 30 (30) min | 11.3 | 16593 | 82.97 | 11.9 |
| 30 (60) min | 11.2 | 16274 | 81.37 | 11.8 |
| 45 (105) min | 11.0 | 15853 | 79.26 | 11.8 |
| 45 (150) min | 11.0 | 15714 | 78.57 | 11.7 |
| 40 (190) min | 10.8 | 15256 | 76.28 | 11.6 |

A Q score of 10 translates into 90% accuracy.

Table S5. Estimation of cost of reagents and sample processing time for MinION sequencing

| Steps | Time |  | Cost in USD |  |
| --- | --- | --- | --- | --- |
| Multiplexed samples | n = 6 | n = 33 | n = 6 | n = 33 |
| RNA extraction | 2 hrs | 8 hrs | 30 | 165 |
| One step RT-PCR | 2 hrs | 2 hrs | 48 | 264 |
| Amplicon purification | 20 min | 2 hrs | 18 | 99 |
| Barcode kit | NA | NA | 25 | 150 |
| Single library preparation | 4 hrs | 8 hrs | 99 | 99 |
| Flow cell | NA | NA | 500 | 500 |
| Flow cell per sequencing run (n) | NA | NA | (n = 5) 100 | (n = 2) 250 |
| Sequencing run | 32 min | 3 hrs 10 min | 320 | 1027 |
| aBasecalling | 1 hr | 1 hr 40 min | NA | NA |
| Post basecalling data processing and consensus assembly | 25 min | 40 min | NA | NA |
| Total time | b9-10 hrs | 26 hrs | NA | NA |
| Cost ($) per sample | NA | NA | $53 | $31 |

aBasecalling time varies based on average length and total number of reads. For amplicon sizes in this study, basecalling took approximately 60 minutes for 60,000 reads. Longer times will be required for longer amplicons.

bIncluding the sequencing run time