Supplementary informations

Fig. S1: Representative picture of shear-resistant firmly adhered Jurkat cells to hCMEC/D3 cells. TOP PANELS Phase contrast pictures of confluent hCMEC/D3 cells stimulated with combination of cytokine (TNF + IFN) at 1 ng/ml for 24 h (RIGHT PANEL) or left unstimulated (LEFT PANEL). BOTTOM PANELS Fluorescent pictures (ex= 495, em= 521) of firmly adhered Jurkat T cells to hCMEC/D3 cells. Field of view (640 x 480 m).

| Parameter | Equation | Definitions | Unit |
| --- | --- | --- | --- |
| Shear rate () | = 8v d |  = Shear rate v= Linear fluid velocity d= Inside diameter of the vessel | [1/s] |
| Dynamic viscosity () (mu) | F = A u y | The magnitude F of this force is found to be proportional to the speed u and the area A of each wall, and inversely proportional to their separation y. | cP(centipoise) 1cP= 1mPa s =[0.01dyn.s/cm2 = (eta) |
| Flow rate () |  = v | V = Velocity of the blood flowing A = Cross sectional vector of the vessel |  [ml/min] |
| Shear stress () | = | Newtonian fluids flowing upon a planar surface  = Shear stress = Shear rate = Dynamic viscosity | [dyn/cm2  |

Table S1:  Parameters to determine shear stress for vessel blood flow. Assuming that the vessel is inelastic, cylindrical and straight, and, the blood is a Newtonian fluid and flow is laminar, the Haagen-Poiseuille equation indicates that the shear stress is directly proportional to blood shear rate and inversely proportional to vessel diameter. Shear stress () depends on shear rate () and dynamic viscosity (), which are related to the properties of the fluid, and the geometry of the vessel. Blood and water`s dynamic viscosities are 1.2 cP and 1 cP, respectively.

| Pulled thought Ibidi channel u-slide IV 0.4 | Shear stress |  | Time (min) |
| --- | --- | --- | --- |
| Leukocyte in media | 0.5 dyn/cm2 | 0.28 ml/min | 5 |
| Media only | 1.5 dyn/cm2 | 0.85 ml/min | 0.5 |

Table S2: Parameters used for flow based adhesion in Ibidi channel u-slide IV 0.4. Data about the geometry of the chamber can be found at www.ibidi.com.