# Supplemental information

Table S1: Settings used for the experiments, if not otherwise mentioned

| Section | Property | Value |
| --- | --- | --- |
| Method | Resolution | 70,000 |
|  | AGC target | 1E06 |
|  | Maximum IT | Auto |
|  | Spectrum data type | Profile |
| Tune file | Sheath gas flow rate | 10 a. u. |
|  | Aux gas flow rate | 0 a. u. |
|  | Spray voltage | 2.5 kV |
|  | Capillary temperature | 320 °C |
|  | S-lens RF level | 50 |
| SESI | Intake line temperature | 100 °C |
|  | Ionizer core temperature | 130 °C |

Table S2: Lock masses used for the experiments

| MS Polarity | Exact Adduct Mass | Compound | Formula (excluding ionization) |
| --- | --- | --- | --- |
| Positive | 149.0233 | phthalic anhydrate | C8H4O3 |
| Positive | 279.1591 | diisobutyl phthalate | C16H22O4 |
| Positive | 445.1200 | polysiloxane (n6) | C24H36Si6O6 |
| Positive | 158.96403 | sodium trifluoroacetate | C2F3NaO2 |
| Positive | 107.07027 | polypropylene glycole | C3H6O |
| Positive | 214.08963 | n-butyl benzenesulfonamide (plasticizer) | C10H15NO2S |

Figure S1. Real-time detection of Tulipalin A from various tulip organs from a different flower. The samples were hold in front of the SESI-Orbitrap MS for 0.5 mins to establish the baseline. Then, the plant material was crushed with a garlic press to monitor tulipalin A signal intensity overtime (n=3). All data was normalized to the average of background signal measured within 0.5 mins.

Figure S2. Real-time detection of Tulipalin A release from flowers of Tulipa, Alstromeria and A. neipolitanum. The samples were hold in front of the SESI-Orbitrap MS for 0.5 mins to establish the baseline. Next, the plant material was crushed with a garlic press to monitor tulipalin A signal intensity overtime (n=3). All data was normalized to the average of background signal measured within 0.5 mins.