Additional file 4: Delivery quality assurance for patient # 3

Two DQA with different configurations were performed for each technique.

Helical Tomotherapy:

In the first DQA, the film was placed in a sagittal plane passing through the 7th level of the dose painting volume to check the capability of the machine to deliver accurately the gradients close to the highest prescription dose. Three ion chambers (IC) were placed in the transverse plane in homogeneous regions (IC 1: in the elective nodal PTV56Gy, IC 2&3: in the therapeutic nodal PTV70Gy).

In the second DQA, the film was placed in a sagittal plane through the elective nodal PTV56Gy while ICs were placed in a transverse plane passing through the dose painting volume (IC 1: centered on level 7 of the dose painting volume, IC 2&3: next to chamber 1 in the gradient of the PTV70Gy). The position of the first chamber was selected to quantify the accuracy in the painted volume using ion chamber, which is more reliable for local absolute dose measurements.

The gamma index was calculated for global normalization within a selected region-of-interest, namely a rectangle encompassing the area within 5 – 10 mm from the film edges.

RapidArc:

In the first DQA, the film was placed in a coronal plane passing through the 7th level of the dose painting volume. The IC was placed 2 cm posteriorly in a homogeneous PTV70Gy region.

In the second DQA, the film was placed in a sagittal plane passing through the 7th level of the dose painting volume. The IC was placed 3 cm on the right in a homogeneous PTV70Gy region.

The gamma index was calculated for global normalization with dose thresholds of 10% and 50%.

The results are summarized in the following table.

|  | HT |  | RA |  |
| --- | --- | --- | --- | --- |
|  | DQA 1 | DQA 2 | DQA 1 | DQA 2 |
| Film dosimetrya | 98.9% | 99.7% | 99.0% / 99.8% c | 96.2% / 98.3% c |
| Ionization chamber 1b | + 0.3% | + 0.7% | + 2.3% | + 2.5% |
| Ionization chamber 2 b | - 0.4% | - 1.4% | - | - |
| Ionization chamber 3 b | - 0.2% | + 1.0% | - | - |

a percentage of pixels passing the gamma-criteria of 3%/3 mm

b discrepancy between planned and measured dose

c 10% and 50% dose threshold respectively