Rolling-ball-based motion extraction (RBME) algorithm

We propose a novel algorithm termed rolling-ball-based motion extraction (RBME), which is specifically designed to extract the external boundary accurately and quantify irregular, noncyclical motion from the created boundary polygons in each plane of cine-MRI.

Notation

| Symbol | Definition |
| --- | --- |
| P | Set of 2D contour points for a single cine frame in a given imaging plane. |
| p_i | i-th point in P, represented by Cartesian coordinates (x_i, y_i). |
| N | Number of points in P. |
| c | Centroid of P, used as the reference origin after alignment. |
| s | Scale factor used for normalisation after centroid alignment. |
| dist(p, q) | Euclidean distance between two points p and q. |
| K | Spatial dimension of the point coordinates; K = 2 for 2D cine contours. |
| D(p) | Local point density at p, computed as the count of neighbours within radius r. |
| r | Radius parameter for local density estimation and outlier suppression. |
| I(·) | Indicator function that equals 1 when the condition is satisfied and 0 otherwise. |
| D | Density percentile threshold; points with rho above the D percentile are retained. |
| Pfiltered | Filtered point set after local density-based outlier suppression. |
| R | Rolling-circle radius used for boundary tracing. |

Preprocessing

All files in the dataset undergo a preprocessing step. We denote all the points in the dataset as . The centroid c is computed as follows:

All the points are then aligned to a coordinate system with c as the origin, followed by scale transformation and normalization. The normalized point set is defined as :

where s is the scale factor; this step eliminates the differences in translation, rotation, and scaling of the image.

K-nearest neighbors (KNN)-inspired local-local point density filtering (outlier suppression)

Next, a high-local point density point filtering method inspired by the KNN algorithm is employed to calculate the Euclidean distance between points.13

where p and q are any two points and where K represents the dimension of the space in which the points exist.

Boundary detection and intersection identification

This study defines the local point density of each point as follows:

where r is the preset radius and is the indicator function. We then sort according to the following:

High-local point density sites retain points above the 95th percentile of local density rankings are retained:

With this local point density filtering method, the influence of isolated points is considerably reduced, resulting in a filtered high-local point density point set . Boundary extraction is performed via a modified rolling-ball boundary tracing. A virtual circle with a radius R is gradually created around the point set to trace the external and interior boundaries. For the current point and the previously visited point , the algorithm selects the next boundary point  with the smallest angular displacement.

is the set of candidate points that satisfy the distance condition . The computation procedure is detailed in . The algorithm terminates when the boundary is closed ( when the closure threshold is reached.

A similar strategy is employed for interior boundary detection; however, the starting point is selected as the nearest interior point  to the left of the center of the GTV contour, which satisfies the following conditions:

Accurate extraction of internal boundaries is crucial for effectively capturing anatomical or structural changes.

Motion analysis

Finally, by computing the axis-specific boundary intercepts through the GTV centroid (i.e., the intercepts of the detected boundary curves along the centroid-crossing horizontal and vertical axes), the following equation can be derived:

For any adjacent boundary points that satisfy , there exists a horizontal intersection, computed as follows:

If the condition is satisfied, then a perpendicular intersection can be determined as follows:

Eventually, the set of intersection points can be defined as follows:

where (, ) and (, ) are neighboring points.

In the aforementioned method, the external boundary and the interior boundary of the set of points are obtained.

On the basis of the aforementioned method for identifying the intersection points, the upper, lower, left, and right boundary points of the initial tumor position are determined.

Consequently, displacement can be expressed as follows:

RBME algorithm validation in a dynamic phantom

To assess robustness to algorithm settings during phantom validation, we conducted a sensitivity analysis of centroid-aligned, one-sided boundary excursions with respect to three RBME hyperparameters, namely the density percentile threshold D, the spatial parameter r, and the search radius R. Nominal hyperparameters were set a priori to D of 0.95, r of 0.5 cm, and R of 10 pixels. These nominal values were selected as mid-range and commonly used defaults within the explored parameter sets rather than being tuned to minimise phantom-specific error, thereby reducing the risk of overfitting to a single phantom motion scenario. Each hyperparameter was varied one at a time while the remaining parameters were held at their nominal values, and the resulting boundary excursion estimates are summarised in Supplementary Table S1.

Table S1. Sensitivity of centroid-aligned, one-sided boundary excursions estimated by RBME to key hyperparameters

Coronal plane; Sagittal plane; Axial plane. D (density percentile); Cranial; Caudal; Left; Right; Cranial; Caudal; Anterior; Posterior; Anterior; Posterior; Left; Right. 0.93; 0.49; 0.36; 0.24; 0.32; 0.31; 0.31; 0.28; 0.26; 0.40; 0.45; 0.47; 0.51. 0.95; 0.49; 0.36; 0.24; 0.32; 0.31; 0.30; 0.28; 0.31; 0.40; 0.57; 0.47; 0.54. 0.99; 0.64; 0.44; 0.24; 0.32; 0.34; 0.32; 0.33; 0.33; 0.60; 0.57; 0.53; 0.64. r (cm). 0.3; 0.38; 0.33; 0.24; 0.29; 0.29; 0.30; 0.26; 0.30; 0.51; 0.47; 0.46; 0.55. 0.5; 0.49; 0.36; 0.24; 0.32; 0.31; 0.30; 0.28; 0.31; 0.40; 0.57; 0.47; 0.54. 0.7; 0.54; 0.32; 0.24; 0.33; 0.32; 0.35; 0.27; 0.50; 0.40; 0.62; 0.48; 0.56. R (pixel). 7; 0.53; 0.36; 0.24; 0.32; 0.26; 0.27; 0.29; 0.28; 0.45; 0.45; 0.48; 0.63. 10; 0.49; 0.36; 0.24; 0.32; 0.31; 0.30; 0.28; 0.31; 0.40; 0.57; 0.47; 0.54. 13; 0.53; 0.36; 0.24; 0.32; 0.34; 0.31; 0.29; 0.30; 0.40; 0.61; 0.50; 0.52.

Notes: Values represent one-sided excursions (peak-to-peak boundary range divided by 2) after centroid alignment. Coronal and sagittal 'up/low' map to cranial/caudal. Sagittal left/right correspond to anterior/posterior based on the image orientation used. Axial 'up/low' map to anterior/posterior.

Table S2. Prespecified pairwise contrasts with Holm-adjusted significance and standardised effect sizes.

| Estimate_mm | SE_mm | Z | p | p_holm_family | Effect_size | Effect_type | Sig (Holm, family) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| -0.272 | 0.266 | -1.02 | 0.3074 | 0.9222 | -0.186 | Cohen's dz | FALSE |
| 0.156 | 0.266 | 0.59 | 0.5584 | 1.0000 | 0.098 | Cohen's dz | FALSE |
| -0.116 | 0.266 | -0.44 | 0.6632 | 1.0000 | -0.082 | Cohen's dz | FALSE |
| 1.652 | 0.595 | 2.78 | 0.0055 | 0.0826 | 1.399 | Cohen's dz | FALSE |
| -1.275 | 0.595 | -2.14 | 0.0321 | 0.4500 | -1.009 | Cohen's dz | FALSE |
| -1.177 | 0.595 | -1.98 | 0.0479 | 0.6227 | -1.004 | Cohen's dz | FALSE |
| -1.093 | 0.595 | -1.84 | 0.0663 | 0.7956 | -0.535 | Cohen's dz | FALSE |
| 1.057 | 0.595 | 1.78 | 0.0759 | 0.8347 | 0.598 | Cohen's dz | FALSE |
| -0.981 | 0.595 | -1.65 | 0.0992 | 0.9924 | -0.681 | Cohen's dz | FALSE |
| 0.876 | 0.595 | 1.47 | 0.1411 | 1.0000 | 0.499 | Cohen's dz | FALSE |
| -0.829 | 0.595 | -1.39 | 0.1639 | 1.0000 | -3.338 | Cohen's dz | FALSE |
| 0.596 | 0.595 | 1.00 | 0.3169 | 1.0000 | 0.545 | Cohen's dz | FALSE |
| 0.447 | 0.595 | 0.75 | 0.4529 | 1.0000 | 0.355 | Cohen's dz | FALSE |
| -0.302 | 0.595 | -0.51 | 0.6124 | 1.0000 | -0.438 | Cohen's dz | FALSE |
| -0.120 | 0.595 | -0.20 | 0.8397 | 1.0000 | -0.075 | Cohen's dz | FALSE |
| 0.112 | 0.595 | 0.19 | 0.8510 | 1.0000 | 0.122 | Cohen's dz | FALSE |
| -0.074 | 0.595 | -0.12 | 0.9011 | 1.0000 | -0.070 | Cohen's dz | FALSE |
| -0.046 | 0.595 | -0.08 | 0.9378 | 1.0000 | -0.032 | Cohen's dz | FALSE |
| 1.303 | 0.409 | 3.18 | 0.0014 | 0.0043 | 1.119 | Hedges' g | TRUE |
| 0.555 | 0.409 | 1.36 | 0.1753 | 0.3506 | 0.550 | Hedges' g | FALSE |
| -0.288 | 0.409 | -0.70 | 0.4818 | 0.4818 | -0.243 | Hedges' g | FALSE |
| 2.625 | 0.633 | 4.15 | 0.0000 | 0.0010 | 1.938 | Hedges' g | TRUE |
| 2.079 | 0.627 | 3.32 | 0.0009 | 0.0265 | 1.552 | Hedges' g | TRUE |
| 1.914 | 0.633 | 3.02 | 0.0025 | 0.0699 | 1.349 | Hedges' g | FALSE |
| 1.805 | 0.632 | 2.86 | 0.0043 | 0.1152 | 2.960 | Hedges' g | FALSE |
| 1.445 | 0.615 | 2.35 | 0.0187 | 0.4864 | 1.356 | Hedges' g | FALSE |
| -1.454 | 0.627 | -2.32 | 0.0204 | 0.5097 | -1.218 | Hedges' g | FALSE |
| 1.259 | 0.615 | 2.05 | 0.0405 | 0.9712 | 2.127 | Hedges' g | FALSE |
| 1.134 | 0.627 | 1.81 | 0.0704 | 1.0000 | 0.994 | Hedges' g | FALSE |
| 1.097 | 0.632 | 1.74 | 0.0825 | 1.0000 | 1.036 | Hedges' g | FALSE |
| 1.094 | 0.632 | 1.73 | 0.0832 | 1.0000 | 1.477 | Hedges' g | FALSE |
| -0.953 | 0.632 | -1.51 | 0.1315 | 1.0000 | -0.857 | Hedges' g | FALSE |
| 0.913 | 0.632 | 1.45 | 0.1483 | 1.0000 | 0.704 | Hedges' g | FALSE |
| -0.841 | 0.627 | -1.34 | 0.1798 | 1.0000 | -0.550 | Hedges' g | FALSE |
| 0.820 | 0.632 | 1.30 | 0.1943 | 1.0000 | 0.597 | Hedges' g | FALSE |
| -0.711 | 0.633 | -1.12 | 0.2615 | 1.0000 | -0.997 | Hedges' g | FALSE |
| 0.699 | 0.627 | 1.11 | 0.2652 | 1.0000 | 0.675 | Hedges' g | FALSE |
| -0.613 | 0.633 | -0.97 | 0.3329 | 1.0000 | -0.422 | Hedges' g | FALSE |
| 0.604 | 0.632 | 0.96 | 0.3391 | 1.0000 | 0.449 | Hedges' g | FALSE |
| -0.554 | 0.627 | -0.88 | 0.3766 | 1.0000 | -0.505 | Hedges' g | FALSE |
| 0.546 | 0.627 | 0.87 | 0.3839 | 1.0000 | 0.939 | Hedges' g | FALSE |
| 0.515 | 0.627 | 0.82 | 0.4117 | 1.0000 | 0.396 | Hedges' g | FALSE |
| 0.398 | 0.615 | 0.65 | 0.5168 | 1.0000 | 0.299 | Hedges' g | FALSE |
| -0.320 | 0.633 | -0.51 | 0.6134 | 1.0000 | -0.316 | Hedges' g | FALSE |
| -0.311 | 0.632 | -0.49 | 0.6229 | 1.0000 | -0.365 | Hedges' g | FALSE |
| 0.293 | 0.633 | 0.46 | 0.6433 | 1.0000 | 0.208 | Hedges' g | FALSE |
| -0.184 | 0.633 | -0.29 | 0.7715 | 1.0000 | -0.209 | Hedges' g | FALSE |
| -0.165 | 0.627 | -0.26 | 0.7926 | 1.0000 | -0.257 | Hedges' g | FALSE |
| 0.144 | 0.633 | 0.23 | 0.8198 | 1.0000 | 0.253 | Hedges' g | FALSE |
| -0.040 | 0.633 | -0.06 | 0.9501 | 1.0000 | -0.042 | Hedges' g | FALSE |
| -0.009 | 0.632 | -0.01 | 0.9885 | 1.0000 | -0.008 | Hedges' g | FALSE |
| -0.481 | 0.451 | -1.07 | 0.2865 | 0.8595 | -0.192 | Cohen's dz | FALSE |
| -0.272 | 0.451 | -0.60 | 0.5475 | 1.0000 | -0.116 | Cohen's dz | FALSE |
| 0.210 | 0.451 | 0.46 | 0.6425 | 1.0000 | 0.076 | Cohen's dz | FALSE |
| 2.424 | 1.010 | 2.40 | 0.0163 | 0.2452 | 1.218 | Cohen's dz | FALSE |
| -2.256 | 1.010 | -2.24 | 0.0254 | 0.3558 | -1.058 | Cohen's dz | FALSE |
| -2.193 | 1.010 | -2.17 | 0.0298 | 0.3878 | -0.999 | Cohen's dz | FALSE |
| -1.959 | 1.010 | -1.94 | 0.0523 | 0.6275 | -0.525 | Cohen's dz | FALSE |
| 1.817 | 1.010 | 1.80 | 0.0718 | 0.7900 | 0.604 | Cohen's dz | FALSE |
| -1.785 | 1.010 | -1.77 | 0.0771 | 0.7900 | -0.678 | Cohen's dz | FALSE |
| 1.772 | 1.010 | 1.76 | 0.0793 | 0.7900 | 0.636 | Cohen's dz | FALSE |
| -1.679 | 1.010 | -1.66 | 0.0962 | 0.7900 | -2.299 | Cohen's dz | FALSE |
| 0.652 | 1.010 | 0.65 | 0.5183 | 1.0000 | 0.320 | Cohen's dz | FALSE |
| 0.514 | 1.010 | 0.51 | 0.6108 | 1.0000 | 0.237 | Cohen's dz | FALSE |
| -0.439 | 1.010 | -0.43 | 0.6637 | 1.0000 | -0.387 | Cohen's dz | FALSE |
| 0.175 | 1.010 | 0.17 | 0.8627 | 1.0000 | 0.103 | Cohen's dz | FALSE |
| 0.121 | 1.010 | 0.12 | 0.9047 | 1.0000 | 0.052 | Cohen's dz | FALSE |
| 0.097 | 1.010 | 0.10 | 0.9234 | 1.0000 | 0.058 | Cohen's dz | FALSE |
| 0.024 | 1.010 | 0.02 | 0.9812 | 1.0000 | 0.011 | Cohen's dz | FALSE |
| 2.274 | 0.698 | 3.26 | 0.0011 | 0.0034 | 1.153 | Hedges' g | TRUE |
| 0.962 | 0.698 | 1.38 | 0.1684 | 0.3368 | 0.557 | Hedges' g | FALSE |
| -0.631 | 0.698 | -0.90 | 0.3659 | 0.3659 | -0.310 | Hedges' g | FALSE |
| 4.377 | 1.082 | 4.04 | 0.0001 | 0.0016 | 1.997 | Hedges' g | TRUE |
| 3.384 | 1.062 | 3.19 | 0.0014 | 0.0417 | 1.542 | Hedges' g | TRUE |
| 3.436 | 1.080 | 3.18 | 0.0015 | 0.0417 | 3.443 | Hedges' g | TRUE |
| 3.055 | 1.082 | 2.82 | 0.0048 | 0.1285 | 1.288 | Hedges' g | FALSE |
| -2.760 | 1.062 | -2.60 | 0.0093 | 0.2430 | -1.301 | Hedges' g | FALSE |
| 2.520 | 1.038 | 2.43 | 0.0152 | 0.3788 | 1.367 | Hedges' g | FALSE |
| 2.443 | 1.038 | 2.35 | 0.0186 | 0.4457 | 2.341 | Hedges' g | FALSE |
| 2.114 | 1.080 | 1.96 | 0.0503 | 1.0000 | 1.565 | Hedges' g | FALSE |
| 1.787 | 1.062 | 1.68 | 0.0924 | 1.0000 | 0.920 | Hedges' g | FALSE |
| 1.636 | 1.080 | 1.51 | 0.1299 | 1.0000 | 1.009 | Hedges' g | FALSE |
| 1.554 | 1.080 | 1.44 | 0.1501 | 1.0000 | 0.756 | Hedges' g | FALSE |
| -1.375 | 1.062 | -1.29 | 0.1955 | 1.0000 | -0.536 | Hedges' g | FALSE |
| -1.385 | 1.082 | -1.28 | 0.2006 | 1.0000 | -0.558 | Hedges' g | FALSE |
| -1.362 | 1.080 | -1.26 | 0.2073 | 1.0000 | -0.761 | Hedges' g | FALSE |
| -1.322 | 1.082 | -1.22 | 0.2219 | 1.0000 | -1.009 | Hedges' g | FALSE |
| 1.146 | 1.080 | 1.06 | 0.2887 | 1.0000 | 0.509 | Hedges' g | FALSE |
| 1.098 | 1.062 | 1.03 | 0.3009 | 1.0000 | 0.550 | Hedges' g | FALSE |
| 1.017 | 1.062 | 0.96 | 0.3383 | 1.0000 | 0.425 | Hedges' g | FALSE |
| 0.993 | 1.062 | 0.94 | 0.3497 | 1.0000 | 0.941 | Hedges' g | FALSE |
| -0.973 | 1.082 | -0.90 | 0.3687 | 1.0000 | -0.549 | Hedges' g | FALSE |
| 0.941 | 1.080 | 0.87 | 0.3834 | 1.0000 | 0.427 | Hedges' g | FALSE |
| -0.824 | 1.062 | -0.78 | 0.4375 | 1.0000 | -0.379 | Hedges' g | FALSE |
| -0.733 | 1.080 | -0.68 | 0.4971 | 1.0000 | -0.515 | Hedges' g | FALSE |
| 0.537 | 1.038 | 0.52 | 0.6046 | 1.0000 | 0.242 | Hedges' g | FALSE |
| 0.412 | 1.082 | 0.38 | 0.7032 | 1.0000 | 0.176 | Hedges' g | FALSE |
| -0.329 | 1.062 | -0.31 | 0.7567 | 1.0000 | -0.273 | Hedges' g | FALSE |
| 0.274 | 1.082 | 0.25 | 0.8002 | 1.0000 | 0.246 | Hedges' g | FALSE |
| -0.240 | 1.080 | -0.22 | 0.8244 | 1.0000 | -0.148 | Hedges' g | FALSE |
| 0.192 | 1.082 | 0.18 | 0.8590 | 1.0000 | 0.114 | Hedges' g | FALSE |
| -0.082 | 1.082 | -0.08 | 0.9399 | 1.0000 | -0.054 | Hedges' g | FALSE |

Notes: The table reports prespecified pairwise contrasts. Estimate denotes the estimated difference in the endpoint and is expressed in millimetres. SE denotes the standard error of the estimate. Z denotes the Wald Z statistic. P denotes the nominal two-sided P value. Holm-adjusted P value denotes the P value adjusted for multiple testing within the prespecified family of contrasts using the Holm procedure. Significant after Holm adjustment indicates whether the Holm-adjusted P value meets the predefined significance threshold. Standardised effect sizes are reported as shown in the Effect size metric column. Cohen’s dz denotes the standardised mean difference for paired or within-subject comparisons, whereas Hedges’ g denotes the bias-corrected standardised mean difference.