Supplementary information

Table S1 – Main characteristics of reservoirs.

| Reservoir / parameters | Sayano-Shushenskoye Reservoir | Zeyskoye Reservoir | Bureiskoye reservoir | Kolymskoye Reservoir | Kuibyshevskoye reservoir | Volgogradskoye reservoir | Rybinskoe Reservoir | Chirkeyskoe reservoir |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upstream water level, m asl. | 536.1 | 318.1-317.8 | 254.8-255.0 | 447.5 – 448.5 | 51.7-52.0 | 14.3-14.5 | 99.9 | 349.4 |
| Number of stations for CH4 sampling (including downstream) | 7 | 19 | 19 | 21 | 10 | 22 | 14 | 9 |
| Thermocline depth (top-bottom) | 60-100 m | 15-20 m in the upper reaches, down to 60 m at the dam | 20 m in the upper reaches, down to 40 m at the dam | 30-45 m | Observed only in the Cheremshansky Bay, 6.5-10.0 m | Absent (full mixing) | Mostly absent (full mixing) | 9-12 m synoptic, 50-60 m seasonal |
| Presence of hypoxia layer | Not identified | Not identified | Not identified | Not identified | The hypoxia zone is only in the Cheremshansky Bay | Not identified | Rarely identified in the deepest water layers | Not identified |
| The period of filling up the reservoir | 1975-1990 | 1975-1985 | 2003-2008 | 1980-1994 | 1955-1957 | 1958-1960 | 1941—1947 | 1970-1974 |
| Water regulation type | Seasonal | Perennial | Seasonal | Perennial | Seasonal | Seasonal | Perennial | Perennial |
| Main river | Yenisei River | Zeya River | Bureya River | Kolyma River | Volga River | Volga River | Volga River | Sulak River |
| Location | 52°05′57″ N 92°13′58″ E | 54°25′ N 127°45′ E | 50°16′00″ N 130°19′00″ E | 62°02′43″ N 150°03′11″ E | 53°27′00″ N 49°10′00″ E | 49°22′22″ N 45°01′05″ E | 58°22′30″ N 38°25′04″ E | 42°58′00″ N 46°53′00″ E |
| Full reservoir water level, m asl. | 539.0 | 315.0 | 256.0 | 451.5 | 53.0 | 15.0 | 102.0 | 355.0 |
| Length, km | 312 | 290 | 236 | 148 | 510 | 540 | 250 | 35.5 |
| Average width (max), km | 6.3 (12) | 8.4 (24) | 1.0 (5.0) | 3 (10) | 11.6 (40) | 5.8 (17) | 18(56) | 1.2(4.0) |
| Reservoir surface area at full reservoir level, km 2 | 608 | 2420 | 740 | 441 | 6 150 | 3 117 | 4 550 | 42.4 |
| Volume at full reservoir level, km 3 | 30.7 | 68.4 | 20.9 | 15.1 | 57.3 | 31.5 | 25.4 | 2.78 |
| Maximum depth at full reservoir level, м | 220 | 100 | 125 | 121.5 | 41.0 | 41 | 30.4 | 140 |
| Average annual inflow, km3/yr | 47.3 | 24.7 | 27.49 | 14.7 | 264.5 | 260 | 31.4 | 5.55 |

Table S2. CH4 emission factors for reservoirs older than 20 years – Flooded Land Remaining Flooded Land [IPCC, 2019].

| Aggregated Climate Zone |  | CH4 Emission Factors EFCH4 age>20,j (kg CH4 ha-1 year-1) |  |  |
| --- | --- | --- | --- | --- |
|  | j | Average | Lower and upper 95% confidence interval of mean | N |
| Boreal | 1 | 13.6 | 7.3-19.9 | 96 |
| Cool Temperate | 2 | 54.0 | 48.3-59.5 | 1 879 |
| Warm temperate/dry | 3 | 150.9 | 133.3-168.1 | 578 |
| Warm temperate/moist | 4 | 80.3 | 74.0-86.0 | 1 946 |
| Tropical dry/montane | 5 | 283.7 | 261.9-305.8 | 710 |
| Tropical moist/wet | 6 | 141.1 | 131.1-152.7 | 805 |
| Note: the emission factors are derived from the G-Res model [Prairie et al., 2017] outputs from N reservoirs in each climate zone. N is the number of modelled reservoirs used to estimate EF values and their 95% confidence intervals. |  |  |  |  |

Table S3. Field measurement data of the average CH4 fluxes in reservoirs for 2021-2023, mg C-CH4 m-2 day-1.

| Reservoir / Zone | Sayano-Shushenskoye Reservoir | Zeyskoye Reservoir | Bureiskoye reservoir | Kolymskoye Reservoir | Kuibyshevskoye reservoir | Volgogradskoye reservoir | Rybinskoe Reservoir | Chirkeyskoe reservoir |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2021 |  |  |  |  |  |  |  |  |
| Deepwater | 1.8 | 2.8 | 2.0 | 0.1 | 5.8 | 3.0 | 19.1 | 2.3 |
| Shallow | 2.0 | 3.1 | 3.0 | 1.0 | 11.7 | 5.0 | 43.2 | - |
| River | 0.4 | 1.3 | 1.7 | 0.1 | 11.0 | 11.0 | 4.1 | 1.5 |
| 2022 |  |  |  |  |  |  |  |  |
| Deepwater | 0.7 | 1.8 | 3.7 | 0.6 | 7.5 | 3.9 | 16.2 | 1.5 |
| Shallow | 2.8 | 2.6 | 1.7 | 0.8 | 10.1 | 4.5 | 13.2 | 3.1 |
| River | 0.3 | 1.0 | 1.9 | 0.1 | 10.0 | 10.8 | 7.3 | 1.5 |
| 2023 |  |  |  |  |  |  |  |  |
| Deepwater | 2.3 | 2.7 | 2.2 | 1.3 | 3.4 | 5.0 | 40.5 | 3.1 |
| Shallow | 3.3 | 4.4 | 5.0 | 1.4 | 17.4 | 8.6 | 95.4 | 4.1 |
| River | 0.3 | 1.7 | 2.3 |  | 15.2 | 17.8 | 7.8 | 1.1 |

Table S4. The data for Tier 3 assessment (measurement and modeling data).

| Reservoir | FCH4,1 emission from the river surface (measurement data), CH4 kg/m2/year | Cwater,1 organic carbon concentration in river water (measurement data), Сorg kg/м3 | Fsed,1 sedimentation rate in river (modelling data), С kg/m2/year | FCH4,2 emission from the reservoir surface (modelling data), CH4 kg/m2/year | Cwater,2 organic carbon concentration in reservoir water (measurement data), Сorg kg/м3, | Fsed,2 sedimentation rate in reservoir (measurement data), С kg/m2/year | Cwater,2,allo allochthonous organic matter, dimensionless | Cwater,2,auto autochthonous organic matter, dimensionless |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rybinskoe Reservoir | 0.002(± 0.001) | 0.008(± 0.002) | 0.07(± 0.06) | 0.01(± 0.006) | 0.01(± 0.004) | 0.17(±0.07) | 0.60 | 0.40 |
| Kuibyshevskoye reservoir | 0.004(± 0.002) | 0.007(± 0.001) | 0.07(± 0.02) | 0.003(± 0.001) | 0.009(± 0.002) | 0.2(± 0.06) | 0.79 | 0.21 |
| Volgogradskoye reservoir | 0.004(± 0.003) | 0.007(± 0.001) | 0.07(± 0.02) | 0.002(± 0.0003) | 0.008(± 0.0002) | 0.14(±0.04) | 0.80 | 0.20 |
| Chirkeyskoe reservoir | 0.0003(± 0.001) | 0.003(± 0.001) | 0.03(± 0.008) | 0.001(± 0.003) | 0.003(± 0.005) | 12.6(± 0.5) | 0.88 | 0.12 |
| Zeyskoye Reservoir | 0.0004(± 0.0001) | 0.005(± 0.001) | 0.04(± 0.002) | 0.001(± 0.0002) | 0.02(± 0.003) | 0.7(± 0.2) | 0.31 | 0.69 |
| Bureiskoye reservoir | 0.0007(± 0.0001) | 0.005(± 0.001) | 0.16(± 0.01) | 0.001(± 0.0002) | 0.007(± 0.0004) | 3.2(± 0.1) | 0.61 | 0.39 |
| Kolymskoye Reservoir | 0.00003(± 0.000004) | 0.002(± 0.0002) | 0.3(± 0.08) | 0.00025(± 0.00005) | 0.004(± 0.0001) | 2.3(± 0.6) | 0.59 | 0.41 |
| Sayano-Shushenskoye Reservoir | 0.00006(± 0.00001) | 0.003(± 0.0001) | 0.25(± 0.1) | 0.001(± 0.0004) | 0.005(± 0.0002) | 1.1(± 0.3) | 0.72 | 0.28 |