| Supplementary Table 1A. LC-MS/MS settings for the analysis of the synaptic and lysosomal protein panel. |  |  |
| --- | --- | --- |
|  | Parameter | Setting |
| LC | Sample injection volume | 40 µL |
|  | Flow-rate | 0.3 mL/min |
|  | Gradient | Broken; 5–20%B (25 min), 20–45%B (6,5 min) |
|  | Total cycle time | 35 min |
|  | Mobile phase A | 0.1% formic acid in water (v/v) |
|  | Mobile phase B | 0.1% formic acid/84% acetonitrile in water (v/v) |
| Electrospray | Mode | Positive |
|  | Gas temperature | 220 °C |
|  | Gas flow | 15 L/min |
|  | Nebulizer pressure | 40 psi |
|  | Sheath gas temperature | 200 °C |
|  | Sheath gas flow | 11 L/min |
|  | Capillary voltage | 3500 V |
|  | Nozzle voltage | 500 V |
| iFunnel | Mode | Positive |
|  | High-pressure radio frequency | 200 V |
|  | Low-pressure radio frequency | 160 V |
| MRM method | Retention time window | 1 min |
|  | Collision energies | Individually optimized per transition |
|  | Cell accelerator voltage | Individually optimized per transition |

| Supplementary Table 1B. Concentrations of internal standard for each peptide as well as repeatability and intermediate precision for the study after plate correction. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Targets |  |  | QC1 |  |
|  |  | IS Conc [fmol/microL] | Repeatability | Intermediate precision |
| Protein | Peptide |  | %CV2 | %CV |
| 1433E | IISSIEQK | 0.26 | 6.46 | 13.06 |
| 1433Z | VVSSIEQK | 0.26 | 2.90 | 6.15 |
| AP2B1 | AVWLPAVK | 0.26 | 3.31 | 5.93 |
| AP2B1 | IQPGNPNYTLSLK | 0.82 | 7.59 | 10.85 |
| CMGA | EDSLEAGLPLQVR | 4.53 | 2.31 | 7.11 |
| CMGA | GLSAEPGWQAK | 5.9 | 3.58 | 5.96 |
| CPLX2 | AALEQPCEGSLTRPK | 0.26 | 5.57 | 10.63 |
| GDIA | QLICDPSYIPDR | 2.6 | 8.14 | 11.57 |
| NEUG | KGPGPGGPGGAGVAR | 0.26 | 6.37 | 8.43 |
| NPTX1 | CESQSTLDPGAGEAR | 2.6 | 4.50 | 7.87 |
| NPTX1 | ETVLQQK | 2.6 | 2.35 | 6.62 |
| NPTX1 | LTPGEVYNLATCSTK | 2.6 | 2.45 | 6.98 |
| NPTX2 | ETVVQQK | 2.6 | 6.59 | 12.86 |
| NPTX2 | VAELEDEK | 0.26 | 3.50 | 6.45 |
| NPTXR | LVEAFGGATK | 2.6 | 4.87 | 9.99 |
| NPTXR | NNYMYAR | 2.6 | 1.95 | 4.35 |
| PEBP1 | LYEQLSGK | 0.26 | 2.51 | 4.89 |
| PEBP1 | NRPTSISWDGLDSGK | 2.6 | 1.96 | 4.71 |
| SAP3 | EVAGLWIK | 9.93 | 5.02 | 7.08 |
| SAP3 | IESVLSSSGK | 6.28 | 2.44 | 4.18 |
| SCG2 | ALEYIENLR | 3.45 | 4.36 | 6.70 |
| SCG2 | VLEYLNQEK | 7.35 | 3.52 | 5.76 |
| STX1B | QHSAILAAPNPDEK | 0.26 | 10.13 | 13.55 |
| STX7 | EFGSLPTTPSEQR | 0.26 | 4.90 | 9.79 |
| SYUB | EGVVQGVASVAEK | 0.26 | 11.15 | 16.25 |
| SYUG | ENVVQSVTSVAEK | 0.26 | 8.33 | 12.89 |
| VGF | AYQGVAAPFPK | 3.71 | 4.27 | 6.84 |
| VGF | NSEPQDEGELFQGVDPR | 17.1 | 6.99 | 8.14 |
| Proteins/peptides marked as bold continued with for statistical analysis. 1Quality control; 2Coefficient of variation. |  |  |  |  |