Supplemental Figure 2 - Difficult Airway Translation

Difficult Airway
Version: 20220204

The VORTEX

Manipulation:

- Head / neck

- Larynx

- Device

Aids
Size / type
Suction / O2 flow
Muscle tone

Maximum of three attempts per airway management technique, then change device/approach.
The most experienced provider present should perform at least one attempt.
CICO status escalates with each unsuccessful attempt at securing the airway.

VORTEX: Optimization measures

- Head & neck:

  - sniffing position / Esmarch / adjust bed height

  - Insert denture / move tongue forward / remove denture

- Larynx:

  - larynx manipulation / BURP / no cricoid pressure

- Device:

  - 2 hands technique

  - Inflate cuff

  - Scissor grip

  - Rotate LMA 180°

  - Inflate cuff

  - Epiglottis downfold

  - Ice-pick method

  - Rotate tube

- Adjuncts:

  - Guedel

  - Wendl

  - Finger guidance around corner

  - Laryngoscope guidance

  - Bougie

  - S-Guide

  - Magill forceps

- Alternative sizes

  - Spatula / C-MAC

  - Tube / Bougie

- Alternative devices

- Suction

- O2 flush / increased flow

- Suction of foreign material

- Analgesia / anesthesia depth / muscle relaxation

CICO = Cannot Intubate Cannot Oxygenate

- Mobilize all available resources

- Clear leadership, critical self-assessment

- Oxygenate before intubation

- In line with the VORTEX approach

©RDZ