ECMO Assessment

To be documented in the notes during the ward round by ECMO fellow

| ECMO Mechanics: |  |  |
| --- | --- | --- |
| TMP (mmHg) |  |  |
| Resistance (mmHg/L/min) |  | TMP/ECBF |
| ECMO gas exchange |  |  |
|  | Value | Calculation |
| SGF (L/min): |  |  |
| ECBF (L/min): |  |  |
| Post Oxy PO2 (KPa): |  |  |
| VO2ML (ml/min) |  | (ctO2PostOxy-ctO2PreOxy)xECBFx10 |
| VCO2ML (ml/min) |  | (ctCO2(B)PreOxy- ctCO2(B)PostOxy)xECBFx25 |
| VCO2 NL (ml/min) |  | Volumetric Capnography |
| VCO2tot (ml/min) |  | VCO2ML + VCO2NL |
| % VCO2NL (%) |  | VCO2NL/ VCO2tot |
| VCO2NL/VE (mL/min/L) |  |  |
| P0.1 (cmH2O) |  |  |

Proceed to ECMO weaning test if:

- Disease resolution

- Spontaneous breathing

- Haemodynamically stable

- PaO2 on 100% test > 30 kPa

- %VCO2NL > 0.5

ECMO Weaning Test

ECMO nurse, Consultant or Fellow must be at the bedside throughout the test

| Step 1: ‘ECMO-Deoxy-Challenge-Test’ (EDCT) | Timing | Measures/Targets |
| --- | --- | --- |
|  |  | Measure P0.1 – ensure < 5 with sedation or SGF RASS -1/-2 |
| Maintain same SGF Place FiO2 to 0.6 | Wait 10 min | SpO2 > 88% |
| Decrease FdO2 from 1 to 0.6, then 0.3 to 0.21 | 5 min/step() | SpO2 > 88% |
| Stop if SpO2 < 88% > 5 min |  |  |
| If SpO2 > 88% - FiO2 =0.6 and FdO2 = 0.21 |  | Passed ECMO-Deoxy-Test |
| Step 2: ‘ECMO-CO2-Challenge-Test’ (ECCT) |  |  |
| Decrease SGF by 30% every 10 minutes |  | For each step measure P0.1; VCO2NL/VE; VCO2NL; ETCO2 |
| 10.0; 9.5; 9.0; 8.5; 8.0; 7.5; 7.0; 6.5; 6.0; 5.5; 5.0; 4.5; 4.0; 3.5; 3.0; 2.5; 2.0; 1.5; 1.0. 7; 7; 6; 6.0; 5.5; 5; 5; 4.5; 4; 4; 3.5; 3; 3; 2.5; 2.; 2; 1.5; 1; 0.7. 5; 5; 4.5; 4; 3.5; 3.5; 3.5; 3; 3; 2.5; 2.5; 2; 2.0; 1.5; 1.5; 1; 1.0; 0.5; 0.5. 3.5; 3; 3; 3; 2.5; 2.5; 2.5; 2; 2; 2; 1.5; 1.5; 1.5; 1; 1.0; 0.5; 0.5; 0; 0. 2.5; 2; 2; 2.0; 2; 1.5; 1.5; 1.5; 1.5; 1.1; 1; 1; 1.0; 0.5; 0.5; 0; 0. 1.5; 1.5; 1.5; 1.5; 1.5; 1; 1; 1; 1.0; 0.5; 0.5; 0.5; 0.5; 0; 0. 1; 1; 1; 1.0; 1; 0.5; 0.5; 0.5; 0.5; 0; 0; 0; 0. 0.5; 0.5; 0.5; 0.5; 0.5; 0; 0; 0; 0. 0; 0; 0; 0.; 0. |
| From the baseline SGF (light grey area), reduce the SGF following the relative column |  |  |
| Interrupt the test if these criteria are present: - P0.1 > 10 - RR > 35 - VCO2NL/VE decreases compared to baseline > 20% - Obvious signs of distress |  |  |
| Complete test | Return FdO2 to 1 Return SGF to pre-test value |  |

List of abbreviations

| ECBF | Extracorporeal Blood Flow |
| --- | --- |
| FdO2 | Device fraction of O2 on the ECMO |
| FiO2 | Inspiratory fraction of O2 on the ventilator |
| ML | Membrane Lung (i.e., ECMO) |
| NL | Native lung |
| P0.1 | Pressure in the first 100 ms of inspiration |
| RR | Respiratory rate |
| SGF | Sweep Gas Flow |
| VCO2NL/VE | Ventilatory efficiency |
| VCO2NL | Carbon dioxide elimination by the natural lung |
| VCO2ML | Carbon dioxide elimination by the membrane lung |

As ECMO may reduce or abolish the normal hypoxic vasoconstriction, oxygenation can improve over time once hypoxic vasoconstriction is restored. This may take 20-30 minutes.