Additional file 1

VOT variables calculation

StO2 [%]: Directly measured by the NIRS device

THC [µM/L]: Directly measured by the NIRS device

DeO2 [%/min]: The deoxygenation slope has been calculated by linearly fitting the curve StO2 [%] vs. Time [min] from the instant the occlusion started up to one minute after the occlusion started

ReO2 [%]: The reoxygenation slope has been calculated by linearly fitting the curve StO2 [%] vs. Time [min] from the instant the occlusion ended up to the instant the saturation reached the baseline value.

HAUC [%∙min]: The hyperemic response has been calculated as the area defined by the hyperemic peak and the recovery baseline StO2 level