Additional file 1 – Different methods for functional lung contouring

| METHOD | DESCRIPTION | NOTES |
| --- | --- | --- |
| Tidal impedance variation (TIV) | EIT image pixels with a TIV above a predefined cut-off value are considered ventilated and included in the functional ROI(1) | Cut-off value is typically a percentage of the maximum pixel TIV. Most commonly used and recommended method for functional ROI selection. |
| Standard deviation of pixel impedance | EIT image pixels with a standard deviation of TIV above a predefined cut-off value are considered ventilated and included in the functional ROI.(2) | Cut-off value is typically a percentage of the maximum pixel standard deviation of TIV; may include pixels with negative impedance changes in ROI. |
| Linear regression coefficient of pixel impedance | EIT image pixels with a regression coefficient above a predefined cut-off value are considered ventilated and included in the functional ROI. Regression coefficients are derived from linear regression equations between the individual pixel and average global impedance signals.(2) | Cut-off value is typically a percentage of the maximum pixel regression coefficient. |

Abbreviations: EIT, electrical impedance tomography; ROI, region of interest; TIV, tidal impedance variation.

References

- Becher T, Vogt B, Kott M, et al. Functional Regions of Interest in Electrical Impedance Tomography: A Secondary Analysis of Two Clinical Studies. PLoS One. 2016;11(3):e0152267.

- Pulletz S, van Genderingen HR, Schmitz G, et al. Comparison of different methods to define regions of interest for evaluation of regional lung ventilation by EIT. Physiol Meas. 2006;27(5):S115-27.