Legend: 1 - disposable resuscitator1, 2 - FlowCheck sensor2, 3 – connector for CO2 measuring tube2 ,4 – pressure measuring tube2, 5 – MEDUtrigger2, 6 - testing bag2 (as visualization of connected device), 7 – FlowCheck sensor connection line2, 8 – MEDUtrigger connection line2

1 Ambu Spur II, Ambu GmbH

2 WEINMANN Emergency, Medical Technology GmbH + Co. KG

Supplement 1 – Figure of the airway pressure measurement

Supplement 2 – Table of measured ventilation parameters and calculation methods

| Parameters | Description | Calculation |
| --- | --- | --- |
| ∆Vt (ml) | Parameter for visualizing effective ventilation | ∆Vt = Vtideal–Vte |
| Vtideal (ml) | Calculated tidal volume based on the ideal body weight (IBW) | Calculated by the mechanical ventilator: IBW female (kg) = 45 + 2.3 x (height in cm/2.54−60) IBW male (kg) = 50 + 2.3 x (height in cm/2.54−60) multiplicated with 6 ml |
| Vte (ml) | Expiratory tidal volume | Mean value over the last 5 expiratory volumes. Vte (single) calculates as integral over the expiration ( with Ti as begin and Te as end of the expiration). Measured by the mechanical ventilator. |
| Vti (ml) | Inspiratory tidal volume | Mean value over the last 5 inspiratory volumes. Vti (single) calculates as integral over all positive flows in one breath (with 0 as start of breath and Te as end). Measured by the mechanical ventilator. |
| Vleak (%) | Relative leakage volume | Integral of the flows over one minute (positive flows - negative flows) divided by the inspiratory minute volume. Measured by the mechanical ventilator. |
| MVe (l/min) | Expiratory volume per minute | Minute average of the expiratory tidal volume measured by the device. MVe=Vt×F, averaged over 5 breaths |
| Pmean (mbar) | Mean pressure | Average value of the pressure over one minute. All pressure values measured by the mechanical ventilator during a breath were totaled and divided by the number of time intervals in which the measurement was taken. |
| Ppeak (mbar) | Peak pressure | Mean value of the maximum airway pressure (peak pressure) per respiratory cycle over 1 minute. Measured by the mechanical ventilator. |
| Fmanual (x/min) | Manual ventilation frequency | Trend value measured by the device, averaged over 5 breaths. |