Purchases

Life Technologies

MessageAmp™ II aRNA Amplification Kit

Ambion AM1751

Life Technologies

ERCC RNA Spike-In Mix

Ambion 4456740

Life Technologies

SUPERase•In™ RNase Inhibitor (20 U/μL)

Ambion AM2694 or AM2696

Thermo Scientific

dNTP Mix, 25mM each

FERR1121

Stocks

5× Lysis Buffer

2.5% NP-40 (10%, Thermo Scientific 28324)

250 mM Tris-HCl, pH 8.4

5 mM EDTA

CEL-Seq Primers Stock

Plate of 96 primers each at a concentration of 25ng/ul in TE

C1 Mixes

Lysis Mix (1000 µL)

280	µL	5× Lysis Buffer

7	µL	20 U/μL SUPERase• In

5.5	µL	1:100 dilution of ERCC RNA Spike-In Mix

70	µL	C1™ Loading Reagent (Fluidigm 100-5170)

638	µL	H2O

9 µL Lysis Mix to Inlet #3

Diluted CEL-Seq Primers

Dispense 8 µL Lysis Mix to each of 96 wells

Add 2 µL CEL-Seq Primer Stock to each well

3 µL Diluted CEL-Seq Primer to each of 96 Outlets

RT Mix (10 µL)

3.5	µL	10× First Strand Buffer (from MessageAmp kit)

1.75 	µL	RNase Inhibitor (from MessageAmp kit)

1.75	µL	Array Script Reverse Transcriptase (from MessageAmp kit)

3	µL of the following mix:

1.4	µL	25 mM dNTPs

1	µL	C1™ Loading Reagent (Fluidigm 100-5170)

3.6	µL	H2O

9 µL RT Mix to Inlet #4

2nd Strand Mix (40.5 µL)

5	µL	10× Second Strand Buffer (from MessageAmp kit)

2	µL	dNTP Mix (from MessageAmp kit)

1	µL	DNA Polymerase (from MessageAmp kit)

0.5	µL	RNase H (from MessageAmp kit)

2	µL	C1™ Loading Reagent (Fluidigm 100-5170)

30	µL	H2O

24 µL 2nd Strand Mix to Inlet #7

IVT Mix (26.9 µL)

6	µL	T7 10× Reaction Buffer (from MessageAmp kit)

13.5	µL	rNTP (6ul of each, speedvaced down to 13.5ul)

6	µL	T7 Enzyme Mix (from MessageAmp kit)

1.36	µL	C1™ Loading Reagent (Fluidigm 100-5170)

24 µL IVT Mix to Inlet #8

C1 Steps

- 96 Diluted CEL-Seq Primers to chamber 1 from the 96 Outlet wells

- Lysis Mix to chambers 0+1+2 from Inlet #3

65°C, 5 min (300 sec)

10°C, 1 min (60 sec)

- RT Mix to chambers 0+1+2+3 from Inlet #4

42°C, 120 min (7200 sec)

10°C, 1 min (60 sec)

- 2nd Strand Mix to chambers 0+1+2+3+4 from Inlet #7

16°C, 120 min (7200 sec)

65°C, 20 min (1200 sec)

10°C, 1 min (60 sec)

- IVT Mix to chambers 0+1+2+3+4+5 from Inlet #8

37°C, 12 hr (12× 3600 sec)

10°C, 1 min (60 sec)

- Harvest

aRNA Clean up

- Pool 5µl of each of the harvested aRNA libraries (~480µl total)

- Add 1.8× volumes of RNAClean XP beads to the harvested aRNA. Mix well until the liquid appears homogeneous.  Incubate for 10 min at room temperature.

- Bind beads on a magnetic stand for 5 min.

- Discard the supernatant.

- Wash 3 times with 70% ethanol.

- Air dry for 10 min or until completely dry.

- Resuspend the beads with 16 µl RNase-free DDW. Incubate in room temperature for 2 min.

- Bind beads on a magnetic stand for 5 min.

- Transfer the eluted aRNA to a new Eppendorf, continue with ExoSAP treatment, as in regular CEL-Seq2 protocol.