Development of an HPLC-UV Method for Quantification of Posaconazole in Low-volume Plasma Samples: Design of Experiment and Machine Learning Models

Fereshteh Bayata, Ali Hashemi Baghib, Zahra Abbasiana, Simin Dadashzadeha, Reza Aboofazelia,c, Azadeh Haeria,c,*

a Department of Pharmaceutics and Pharmaceutical Nanotechnology, School of Pharmacy, Shahid Beheshti University of Medical Sciences, Tehran, Iran

b Department of Industrial Engineering and Management Systems, Amirkabir University of Technology, Tehran, Iran

c Protein Technology Research Center, Shahid Beheshti University of Medical Sciences, Tehran, Iran

Artificial neural network-genetic algorithm initiating

In this paper, the adjusted min-max method was used for the normalization of data as follows:

Xn=λ1+(λ2-λ1)(Xi-XminXmax‒Xmin)                                            Eq. S1

Where λ1 and λ2 are constant optional values based on statistical characteristics (In the present work λ1= 0.1 and λ2= 0.9), Xi is input variable, Xmin is the minimum value, Xmax is the maximum value, and finally Xn is normalized data.

Artificial neural network-genetic algorithm validation

The performance of neural networks could be evaluated by some statistical characteristics such as mean squared error (MSE), mean absolute error (MAE), and R-squared (R2). In this paper, all of them were adopted to evaluate the ANN accuracy. MSE, MAE, and R2 were calculated as follows:

MSE=1Ni=1N(yi-yi)2                                                      Eq. S2

MAE=1Ni=1N|yi-yi|                                                        Eq. S3

R2=1-Sum of squared regression SSRTotal sum of squares SST= 1-∑(yi-yi)2∑(yi-y)2             Eq. S4

Where the yi is actual output, yi is predicted output, y is mean, and N is the number of data.

Table S1 ANOVA statistical analysis of the model for RT of PCZ.

| Source | Sum of Squares | Df | Mean Square | F Value | p-value Prob > F |  |
| --- | --- | --- | --- | --- | --- | --- |
| Model | 657.1 | 8 | 82.1 | 147.7 | < 0.001 | Significant |
| A | 276.8 | 1 | 276.8 | 497.6 | < 0.001 |  |
| B | 66.0 | 1 | 66.0 | 118.6 | < 0.001 |  |
| C | 25.0 | 1 | 25.0 | 45.0 | < 0.001 |  |
| E | 239.3 | 1 | 239.3 | 430.3 | < 0.001 |  |
| AB | 20.8 | 1 | 20.8 | 37.4 | < 0.001 |  |
| AC | 9.4 | 1 | 9.4 | 16.9 | 0.002 |  |
| AE | 12.6 | 1 | 12.6 | 22.6 | < 0.001 |  |
| BC | 7.2 | 1 | 7.2 | 12.9 | 0.005 |  |
| Lack of Fit | 4.8 | 7 | 0.7 | 2.5 | 0.238 | not significant |

A: total organic phase; B: methanol; C: pH; E: flow rate; PCZ: posaconazole; RT: retention time.

Table S2 ANOVA statistical analysis of the model for RT of IS.

| Source | Sum of Squares | Df | Mean Square | F Value | p-value Prob > F |  |
| --- | --- | --- | --- | --- | --- | --- |
| Model | 244.0 | 8 | 30.5 | 220.5 | < 0.001 | Significant |
| A | 59.6 | 1 | 59.6 | 430.4 | < 0.001 |  |
| B | 10.9 | 1 | 10.9 | 79.1 | < 0.001 |  |
| D | 1.9 | 1 | 1.9 | 13.5 | 0.004 |  |
| E | 165.3 | 1 | 165.3 | 1195.0 | < 0.001 |  |
| AB | 2.6 | 1 | 2.6 | 19.1 | 0.001 |  |
| AC | 0.9 | 1 | 0.9 | 6.8 | 0.027 |  |
| AE | 1.6 | 1 | 1.6 | 11.4 | 0.007 |  |
| BE | 1.1 | 1 | 1.1 | 8.2 | 0.017 |  |
| Lack of Fit | 0.9 | 7 | 0.1 | 0.7 | 0.678 | Not significant |

A: total organic phase; B: methanol; C: pH; D: temperature; E: flow rate; IS: internal standard; RT: retention time.

Table S3 ANOVA statistical analysis of the model for resolution.

| Source | Sum of Squares | Df | Mean Square | F Value | p-value Prob > F |  |
| --- | --- | --- | --- | --- | --- | --- |
| Model | 163.9 | 11 | 14.9 | 481.2 | < 0.001 | Significant |
| A | 71.0 | 1 | 71.0 | 2292.5 | < 0.001 |  |
| B | 29.3 | 1 | 29.3 | 946.0 | < 0.001 |  |
| C | 26.5 | 1 | 26.5 | 855.7 | < 0.001 |  |
| D | 0.5 | 1 | 0.5 | 16.3 | 0.005 |  |
| AB | 15.1 | 1 | 15.1 | 489.0 | < 0.001 |  |
| AC | 14.5 | 1 | 14.5 | 467.8 | < 0.001 |  |
| AD | 1.0 | 1 | 1.0 | 31.7 | < 0.001 |  |
| BC | 3.0 | 1 | 3.0 | 97.4 | < 0.001 |  |
| BD | 0.3 | 1 | 0.3 | 9.3 | 0.019 |  |
| BE | 1.7 | 1 | 1.7 | 54.0 | < 0.001 |  |
| DE | 1.1 | 1 | 1.1 | 33.9 | < 0.001 |  |
| Lack of Fit | 0.04 | 4 | 8.6E-003 | 0.1 | 0.955 | Not significant |

A: total organic phase; B: methanol; C: pH; D: temperature; E: flow rate.

Table S4 Regression coefficients of the model for the RT of PCZ and IS as well as peak resolution.

|  | R2 | Adj R2 | Pred R2 |
| --- | --- | --- | --- |
| PCZ RT | 0.99 | 0.98 | 0.96 |
| IS RT | 0.99 | 0.99 | 0.98 |
| Resolution | 0.99 | 0.99 | 0.99 |

IS: internal standard; PCZ: posaconazole; R2: R-squared; RT: retention time.

Table S5 Ranges for independent and dependent variables required for the optimization of the chromatographic conditions and the corresponding statistical analysis of predicted and observed responses (mean ± SEM, n = 3).

|  | Factors | Unit | Limits |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Lower | Upper | Goal |
| Independent factors | Organic phase | % | 52 | 62 | In range |
|  | Methanol | % | 0 | 6 | In range |
|  | pH |  | 3 | 7 | In range |
|  | Temperature | C | 25 | 40 | In range |
|  | Flow rate | mL/min | 0.7 | 1.2 | In range |
| Responses | RT of PCZ | min | 8 | 15 | Minimize |
|  | RT of IS | min | 4 | 11 | Minimize |
|  | Resolution |  | 5 | 6.5 | In range |
|  | Response | Predicted | Observed |  | p-Value |
|  | PCZ RT (min) | 8.8 ± 0.6 | 8.2 ± 0.2 |  | 0.415 |
|  | IS RT (min) | 4.9 ± 0.4 | 5.6 ± 0.1 |  | 0.142 |
|  | Resolution | 5.5 ± 0.2 | 5.0 ± 0.1 |  | 0.064 |

IS: internal standard; PCZ: posaconazole; RT: retention time.

Table S6 Effect of extraction solvent on PCZ and IS recovery (mean ± SEM, n = 3).

| IS recovery (%) | PCZ recovery (%) | Extraction solvent |
| --- | --- | --- |
| 57.4 ± 5.5 | 90.4 ± 0.9 | TBME |
| 63.8 ± 11.8 | 5.7 ± 0.5 | n-hexane |
| 70.1 ± 30.2 | 26.4 ± 26.4 | Ethyl acetate |
| 49.1 ± 6.5 | 59.3 ± 15.9 | Chloroform |
| 48.9 ± 10.6 | 44.7 ± 1.6 | Dichloromethane |

IS: internal standard; PCZ: posaconazole; TBME: tertiary butyl methyl ether.

Table S7 ANOVA statistical analysis of the model for PCZ recovery.

| Source | Sum of Squares | Df | Mean Square | F Value | p-value Prob > F |  |
| --- | --- | --- | --- | --- | --- | --- |
| Model | 7794.9 | 10 | 779.5 | 56.7 | < 0.001 | Significant |
| A´ | 85.6 | 1 | 85.6 | 6.2 | 0.037 |  |
| B´ | 4750.2 | 1 | 4750.2 | 345.3 | < 0.001 |  |
| C´ | 597.2 | 1 | 597.2 | 43.4 | < 0.001 |  |
| A´B´ | 662.1 | 1 | 662.1 | 48.1 | < 0.001 |  |
| A´C´ | 325.7 | 1 | 325.7 | 23.7 | 0.001 |  |
| A´D´ | 235.0 | 1 | 235.0 | 17.1 | 0.003 |  |
| A´E´ | 302.4 | 1 | 302.4 | 22.0 | 0.002 |  |
| B´C´ | 529.7 | 1 | 529.7 | 38.5 | < 0.001 |  |
| C´D´ | 101.0 | 1 | 101.0 | 7.3 | 0.027 |  |
| D´E´ | 206.1 | 1 | 206.1 | 15.0 | 0.005 |  |
| Lack of Fit | 100.6 | 5 | 20.1 | 6.4 | 0.079 | Not significant |

A´: volume of extraction solvent; B´: pH; C´: NaCl; D´: extraction time; E´: centrifuge time; PCZ: posaconazole.

Table S8 ANOVA statistical analysis of the model for IS recovery.

| Source | Sum of Squares | Df | Mean Square | F Value | p-value Prob > F |  |
| --- | --- | --- | --- | --- | --- | --- |
| Model | 1073.1 | 8 | 134.1 | 15.7 | < 0.001 | Significant |
| A´ | 100.7 | 1 | 100.7 | 11.8 | 0.006 |  |
| B´ | 67.6 | 1 | 67.6 | 7.9 | 0.018 |  |
| C´ | 95.8 | 1 | 95.8 | 11.2 | 0.007 |  |
| D´ | 60.6 | 1 | 60.6 | 7.1 | 0.024 |  |
| A´C´ | 235.8 | 1 | 235.8 | 27.6 | < 0.001 |  |
| A´E´ | 122.4 | 1 | 122.4 | 14.3 | 0.004 |  |
| C´D´ | 345.2 | 1 | 345.2 | 40.4 | < 0.001 |  |
| D´E´ | 44.9 | 1 | 44.9 | 5.3 | 0.045 |  |
| Lack of Fit | 52.9 | 7 | 7.6 | 0.7 | 0.689 | Not significant |

A´: volume of extraction solvent; B´: pH; C´: NaCl; D´: extraction time; E´: centrifuge time; IS: internal standard.

Table S9 Regression coefficients of model for PCZ and IS recovery.

|  | R2 | Adj R2 | Pred R2 |
| --- | --- | --- | --- |
| PCZ recovery% | 0.99 | 0.97 | 0.90 |
| IS recovery% | 0.93 | 0.87 | 0.82 |

IS: internal standard; PCZ: posaconazole; R2: R-squared.

Table S10 Ranges for independent and dependent variables to optimize extraction condition and the corresponding statistical analysis of predicted and observed responses (mean ± SEM, n = 3).

|  | Factors | Unit | Limits |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Lower | Upper | Goal |
| Independent factors | Volume of extraction solvent | μL | 500 | 1000 | Minimize |
|  | pH |  | 3 | 11 | In range |
|  | NaCl | % w/v | 0 | 10 | In range |
|  | Extraction time | min | 1 | 10 | Minimize |
|  | Centrifuge time | min | 1 | 10 | Minimize |
| Responses | PCZ recovery | % | 80 | 100 | Maximize |
|  | IS recovery | % | 75 | 90 | Maximize |
| Drug | % Recovery predicted | % Recovery observed |  |  | P-value |
| PCZ 0.5 μg/mL | 98.7 ± 1.9 | 99.2 ± 6.6 |  |  | 0.95 |
| PCZ 1 μg/mL | 98.7 ± 1.9 | 103.2 ± 3.0 |  |  | 0.27 |
| PCZ 1.5 μg/mL | 98.7 ± 1.9 | 97.9 ± 0.4 |  |  | 0.70 |
| IS | 80.6 ± 2.3 | 79.1 ± 1.2 |  |  | 0.59 |

IS: internal standard; PCZ: posaconazole.

Table S11 The MSE, MAE, and R2 values of the ANN developed model for chromatographic conditions.

|  | PCZ RT | IS RT | Resolution |
| --- | --- | --- | --- |
| MSE | 0.0017 | 0.0013 | 0.0015 |
| MAE | 0.0674 | 0.0139 | 0.0299 |
| R2 | 0.999 | 0.998 | 0.999 |

IS: internal standard; MSE: mean squared error; MAE: mean absolute error; PCZ: posaconazole; R2: R-squared; RT: retention time.

Table S12 The MSE, MAE, and R2 values of the ANN developed model for extraction conditions.

|  | PCZ recovery | IS recovery |
| --- | --- | --- |
| MSE | 0.0018 | 0.0016 |
| MAE | 0.0711 | 0.0185 |
| R2 | 0.982 | 0.983 |

IS: internal standard; MSE: mean squared error; MAE: mean absolute error; PCZ: posaconazole; R2: R-squared.

Table S13 Intra-day (n = 5) and inter-day (n = 15) accuracy and precision (mean ± SEM).

| Coefficient of variation% |  |  | Relative error% |  | Concentration (μg/mL) |
| --- | --- | --- | --- | --- | --- |
| Intra-day | Inter-day |  | Intra-day | Inter-day |  |
| 10.4 | 14.0 |  | 2.4 | -6.5 | 0.05 |
| 8.7 | 5.3 |  | 7.1 | -6.0 | 0.1 |
| 2.0 | 4.1 |  | -4.0 | -2.0 | 0.75 |
| 1.5 | 1.5 |  | -8.9 | -4.9 | 1.5 |

Table S14 Robustness assessment of the optimized chromatographic conditions (organic phase 58%, methanol 6%, pH = 7, column temperature: 39 C, and flow rate of 1.2 mL/min).

| Independent Factors | Value | Responses |  |  |
| --- | --- | --- | --- | --- |
|  |  | PCZ RT (min) | IS RT (min) | Resolution |
| Organic phase (% v/v) | 57.5 | 9.3 | 5.1 | 6.0 |
|  | 58 | 8.8 | 4.9 | 5.5 |
|  | 58.5 | 8.3 | 4.7 | 4.9 |
|  | CV% | 5.7 | 4.1 | 10.1 |
| MeOH (%v/v) | 5.5 | 8.5 | 4.9 | 5.1 |
|  | 6 | 8.8 | 4.9 | 5.5 |
|  | 6.5 | 9.2 | 4.9 | 5.8 |
|  | CV% | 4.0 | 0.0 | 6.4 |
| pH | 6.8 | 8.7 | 4.9 | 5.3 |
|  | 7 | 8.8 | 4.9 | 5.5 |
|  | 7.2 | 9.0 | 4.9 | 5.6 |
|  | CV% | 1.7 | 0.0 | 2.8 |
| Temp. (℃) | 38.5 | 8.8 | 4.9 | 5.4 |
|  | 39 | 8.8 | 4.9 | 5.5 |
|  | 39.5 | 8.8 | 4.9 | 5.5 |
|  | CV% | 0.0 | 0.0 | 1.1 |

IS: internal standard; MeOH: methanol; PCZ: posaconazole; RT: retention time.