Supplementary Information

Table S1 Inventory for bioethanol production at the biorefinery per 1 kg ODW poplar processed a

|  | DA pretreatment | LHW pretreatment |
| --- | --- | --- |
| Key parameters |  |  |
| Pretreatment b | 190ºC, 1.1 min, 2.0% sulphuric acid | 200 ºC, 10 min, water |
| Saccharification b | Enzyme loading 15 FPU/g glucan 50 ºC, 72 hours | Enzyme loading 15 FPU/g glucan 50ºC, 72 hours |
| Conversion efficiency of glucan to glucose | 86.63% | 56.0% |
| Conversion efficiency of xylan to xylose | 71.78% | 95.83% |
| Fermentation c | Co-fermentation by recombinant Zymomonas mobilis, 32 ºC, 1.5 days Conversion of glucose and mannose to ethanol 95%, Conversion of xylose and arabinose to ethanol 85% |  |
| WWT c, d | Biogas composition (dry molar basis) CH4 51% CO2 49% Total COD removal 99.6% (86% converted to biogas) |  |
| CHP c | Boiler efficiency (feedstock heating value/steam heat) 80% |  |
| Flue gas treatment c | Desulphurisation by adding lime | None |
| Inputs |  |  |
| Poplar (OD kg) | 1.00E+00 | 1.00E+00 |
| Sulphuric acid (93%) (kg) | 2.01E-02 | 0.00E+00 |
| Ammonia (kg) | 7.87E-03 | 0.00E+00 |
| Enzyme Cellic Ctec 1 (kg) | 1.34E-01 | 1.41E-01 |
| Corn steep liquor (kg) | 1.44E-02 | 1.38E-02 |
| Diammonium phosphate (kg) | 1.91E-03 | 1.82E-03 |
| Sorbitol (kg) | 5.79E-05 | 5.47E-05 |
| Caustic (kg) | 6.72E-02 | 0.00E+00 |
| Boiler chemicals (kg) | 5.47E-06 | 4.48E-06 |
| Lime (kg) | 1.77E-03 | 0.00E+00 |
| Cooling tower chemicals (kg) | 6.11E-05 | 6.98E-05 |
| Makeup water e (kg) | 3.28E+00 | 3.47E+00 |
| Output |  |  |
| Ethanol production (kg) | 2.57E-01 | 2.01E-01 |
| Exported electricity (kWh) | 3.05E-01 | 4.18E-01 |
| Ethanol (kg) | 3.25E-05 | 1.97E-05 |
| CH4 (kg) | 1.77E-04 | 2.85E-05 |
| N2O(kg) | 5.52E-07 | 5.52E-07 |
| NH3(kg) | 7.20E-05 | 0.00E+00 |
| SO2(kg) | 1.33E-03 | 5.36E-04 |
| CO(kg) | 3.36E-08 | 3.36E-08 |
| HNO3(kg) | 1.14E-05 | 0.00E+00 |
| Landfill disposal of ash (kg) | 2.73E-02 | 2.43E-02 |

##### a. Derived from reference1

b. Based on results reported by Wyman et.al.2

c. Based on previous study carried out by National Renewable Energy Laboratory (NREL) 3.

d. WWT includes anaerobic digestion (AD) followed by aerobic treatment. During AD, organic compound (chemical oxygen demand (COD)) removal was assumed as 91% (86% converted to biogas, 5% to cell mass); during aerobic treatment, COD removal was assumed as 99.6% (74% converted to water and CO2 22% to cell mass).

e. Water assumed as natural origin.

Table S2 Inventory for transport involved in Imola-derived bioethanol supply chains d

| Transport | Distance | Mode |
| --- | --- | --- |
| On-site transport for VSRC plantation a | 5.5km | Tractor and trailer |
| On-site transport for SRC plantation b | 1km | Tractor and trailer |
| Poplar to bioethanol plant | 50km c | 32-tonne lorry |
| Bioethanol from bio-refinery plant to storage | 160 km d | 32-tonne lorry |
| Bioethanol from storage to forecourt | 160 km d | 32-tonne lorry |

- Tractor assumed to drive alongside the harvester to collect harvested chips; the transport distance was estimated for a field with row spacing of 3m as 5.5km; during transportation it was assumed a linear loading-weight increase from empty to full capacity

- The transport distance was assumed as 1 km from field to gate; loaded with a full capacity

- Default value for transport from field to bioethanol plant derived from farmed wood was given by the Department for Transport 4

- Derived from reference5

Table S3 Characterized LCIA profiles of Imola Poplar biomass at farm gate per kg ODW Imola Poplar biomass (method: CML 2 baseline 2000)

| Impact category | SRC | VSRC |
| --- | --- | --- |
| Abiotic depletion (kg Sb eq) | 4.57E-01 | 5.36E-01 |
| Acidification(kg SO2 eq) | 5.00E-01 | 5.91E-01 |
| Eutrophication(kg PO43- eq) | 1.59E-01 | 1.86E-01 |
| GWP100 (kg CO2 eq) | -5.77E+02 | -8.58E+02 |
| ODP (kg CFC-11 eq) | 1.18E-05 | 2.76E-05 |
| Human toxicity(kg 1,4-DB eq) | 2.08E+01 | 3.13E+01 |
| Fresh water aquatic eco-toxicity (kg 1,4-DB eq) | 7.85E+00 | 1.04E+01 |
| Marine aquatic eco-toxicity (kg 1,4-DB eq) | 1.99E+04 | 2.55E+04 |
| Terrestrial eco-toxicity (kg 1,4-DB eq) | 2.43E-01 | 3.11E-01 |
| POCP (kg C2H4) | 4.77E-03 | 9.00E-03 |

Table S4 Characterized LCIA profiles of SRC Imola-derived bioethanol (DA) at bio-refinery gate per kg bioethanol (method: CML 2 baseline 2000)

| Impact category | Total | Plantation | Pre-treatment | Saccharification | Fermentation | Combustion | Make up water | Flue gas | Surplus electricity | Transport | C sequestration in poplar feedstock | Waste water treatment | C sequestration in soil |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abiotic depletion (kg Sb eq) | 1.73E-02 | 1.78E-03 | 6.58E-04 | 1.73E-02 | 5.12E-04 | 1.10E-04 | 0 | 0.00E+00 | -5.34E-03 | 1.54E-04 | 0.00E+00 | 2.08E-03 | 0.00E+00 |
| Acidification (kg SO2 eq) | 2.24E-02 | 1.95E-03 | 1.44E-03 | 1.38E-02 | 4.75E-04 | 6.88E-05 | 0 | 6.68E-03 | -3.49E-03 | 6.51E-05 | 0.00E+00 | 1.38E-03 | 0.00E+00 |
| Eutrophication (kg PO43- eq) | 3.80E-03 | 6.18E-04 | 5.89E-05 | 1.29E-03 | 4.80E-04 | 9.19E-04 | 0 | 1.02E-04 | -6.63E-04 | 1.77E-05 | 0.00E+00 | 9.68E-04 | 0.00E+00 |
| GWP100 (kg CO2 eq) | -2.10E+00 | 4.90E-01 | 7.41E-02 | 2.31E+00 | 6.54E-02 | 4.13E-02 | 0 | 1.44E-02 | -7.58E-01 | 2.08E-02 | -1.91E+00 | 2.95E-01 | -2.74E+00 |
| ODP (kg CFC-11 eq) | 1.38E-07 | 4.58E-08 | 1.04E-08 | 1.12E-07 | 6.38E-09 | 2.18E-09 | 0 | 0.00E+00 | -5.98E-08 | 3.41E-09 | 0.00E+00 | 1.77E-08 | 0.00E+00 |
| Human toxicity (kg 1,4-DB eq) | 1.64E+00 | 8.09E-02 | 5.45E-02 | 7.10E-01 | 2.26E-02 | 6.97E-01 | 0 | 5.26E-04 | -1.50E-01 | 5.41E-03 | 0.00E+00 | 2.25E-01 | 0.00E+00 |
| Fresh water eco-toxicity (kg 1,4-DB eq) | 4.66E-01 | 3.06E-02 | 1.21E-02 | 1.36E-01 | 8.85E-03 | 1.92E-01 | 0 | 0.00E+00 | -8.29E-02 | 2.06E-03 | 0.00E+00 | 1.68E-01 | 0.00E+00 |
| Marine aquatic eco-toxicity (kg 1,4-DB eq) | 9.25E+02 | 7.73E+01 | 3.81E+01 | 4.64E+02 | 2.25E+01 | 1.69E+02 | 0 | 0.00E+00 | -2.08E+02 | 4.30E+00 | 0.00E+00 | 3.57E+02 | 0.00E+00 |
| Terrestrial eco-toxicity (kg 1,4-DB eq) | 8.22E-03 | 9.45E-04 | 8.06E-04 | 1.79E-03 | 1.59E-04 | 4.54E-04 | 0 | 0.00E+00 | -1.73E-03 | 4.71E-05 | 0.00E+00 | 5.75E-03 | 0.00E+00 |
| POCP (kg C2H4) | 9.83E-04 | 1.85E-05 | 5.97E-05 | 6.61E-04 | 2.05E-05 | 3.33E-06 | 0 | 3.04E-04 | -1.43E-04 | 2.72E-06 | 0.00E+00 | 5.61E-05 | 0.00E+00 |

Table S5 Characterized LCIA profiles of SRC Imola-derived bioethanol (LHW) at bio-refinery gate per kg bioethanol (method: CML 2 baseline 2000)

| Impact category | Total | Plantation | Pretreatment | Saccharification | Fermentation | Combustion | Make up water | Flue gas | Surplus electricity | Transport | C sequestration in poplar feedstock | Waste water treatment | C sequestration in soil |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abiotic depletion (kg Sb eq) | 1.71E-02 | 2.28E-03 | 0 | 2.33E-02 | 6.26E-04 | 1.13E-04 | 0 | 0.00E+00 | -9.38E-03 | 1.97E-04 | 0.00E+00 | 0 | 0.00E+00 |
| Acidification (kg SO2 eq) | 1.89E-02 | 2.49E-03 | 0 | 1.86E-02 | 5.81E-04 | 7.43E-05 | 0 | 3.20E-03 | -6.12E-03 | 8.34E-05 | 0.00E+00 | 0 | 0.00E+00 |
| Eutrophication (kg PO43- eq) | 3.03E-03 | 7.92E-04 | 0 | 1.74E-03 | 5.87E-04 | 1.05E-03 | 0 | 0.00E+00 | -1.16E-03 | 2.26E-05 | 0.00E+00 | 0 | 0.00E+00 |
| GWP100 (kg CO2 eq) | -2.87E+00 | 6.28E-01 | 0 | 3.10E+00 | 8.01E-02 | 4.12E-02 | 0 | 3.65E-03 | -1.33E+00 | 2.67E-02 | -1.91E+00 | 0 | -3.50E+00 |
| ODP (kg CFC-11 eq) | 1.19E-07 | 5.87E-08 | 0 | 1.51E-07 | 7.80E-09 | 2.10E-09 | 0 | 0.00E+00 | -1.05E-07 | 4.37E-09 | 0.00E+00 | 0 | 0.00E+00 |
| Human toxicity (kg 1,4-DB eq) | 1.62E+00 | 1.04E-01 | 0 | 9.53E-01 | 2.77E-02 | 7.96E-01 | 0 | 2.56E-04 | -2.64E-01 | 6.92E-03 | 0.00E+00 | 0 | 0.00E+00 |
| Fresh water eco-toxicity (kg 1,4-DB eq) | 3.08E-01 | 3.91E-02 | 0 | 1.82E-01 | 1.08E-02 | 2.19E-01 | 0 | 0.00E+00 | -1.46E-01 | 2.63E-03 | 0.00E+00 | 0 | 0.00E+00 |
| Marine aquatic eco-toxicity (kg 1,4-DB eq) | 5.83E+02 | 9.90E+01 | 0 | 6.23E+02 | 2.75E+01 | 1.93E+02 | 0 | 0.00E+00 | -3.64E+02 | 5.51E+00 | 0.00E+00 | 0 | 0.00E+00 |
| Terrestrial eco-toxicity (kg 1,4-DB eq) | 1.34E-03 | 1.21E-03 | 0 | 2.40E-03 | 1.94E-04 | 5.17E-04 | 0 | 0.00E+00 | -3.04E-03 | 6.03E-05 | 0.00E+00 | 0 | 0.00E+00 |
| POCP (kg C2H4) | 8.60E-04 | 2.38E-05 | 0 | 8.88E-04 | 2.51E-05 | 2.83E-06 | 0 | 1.68E-04 | -2.50E-04 | 3.49E-06 | 0.00E+00 | 0 | 0.00E+00 |

Table S6 Characterized LCIA profiles of VSRC Imola-derived bioethanol (DA) at bio-refinery gate per kg bioethanol (method: CML 2 baseline 2000)

| Impact category | Total | Plantation | Pretreatment | Saccharification | Fermentation | Combustion | Make up water | Flue gas | Surplus electricity | Transport | C sequestration in poplar feedstock | Waste water treatment | C sequestration in soil |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abiotic depletion (kg Sb eq) | 1.76E-02 | 2.08E-03 | 6.58E-04 | 1.73E-02 | 5.12E-04 | 1.10E-04 | 0 | 0.00E+00 | -5.34E-03 | 1.54E-04 | 0.00E+00 | 2.08E-03 | 0.00E+00 |
| Acidification (kg SO2 eq) | 2.27E-02 | 2.30E-03 | 1.44E-03 | 1.38E-02 | 4.75E-04 | 6.88E-05 | 0 | 6.68E-03 | -3.49E-03 | 6.51E-05 | 0.00E+00 | 1.38E-03 | 0.00E+00 |
| Eutrophication (kg PO43- eq) | 3.90E-03 | 7.23E-04 | 5.89E-05 | 1.29E-03 | 4.80E-04 | 9.19E-04 | 0 | 1.02E-04 | -6.63E-04 | 1.77E-05 | 0.00E+00 | 9.68E-04 | 0.00E+00 |
| GWP100 (kg CO2 eq) | -3.19E+00 | 5.54E-01 | 7.41E-02 | 2.31E+00 | 6.54E-02 | 4.13E-02 | 0 | 1.44E-02 | -7.58E-01 | 2.08E-02 | -1.91E+00 | 2.95E-01 | -3.89E+00 |
| ODP (kg CFC-11 eq) | 2.00E-07 | 1.07E-07 | 1.04E-08 | 1.12E-07 | 6.38E-09 | 2.18E-09 | 0 | 0.00E+00 | -5.98E-08 | 3.41E-09 | 0.00E+00 | 1.77E-08 | 0.00E+00 |
| Human toxicity (kg 1,4-DB eq) | 1.69E+00 | 1.22E-01 | 5.45E-02 | 7.10E-01 | 2.26E-02 | 6.97E-01 | 0 | 5.26E-04 | -1.50E-01 | 5.41E-03 | 0.00E+00 | 2.25E-01 | 0.00E+00 |
| Fresh water eco-toxicity (kg 1,4-DB eq) | 4.76E-01 | 4.04E-02 | 1.21E-02 | 1.36E-01 | 8.85E-03 | 1.92E-01 | 0 | 0.00E+00 | -8.29E-02 | 2.06E-03 | 0.00E+00 | 1.68E-01 | 0.00E+00 |
| Marine aquatic eco-toxicity (kg 1,4-DB eq) | 9.47E+02 | 9.93E+01 | 3.81E+01 | 4.64E+02 | 2.25E+01 | 1.69E+02 | 0 | 0.00E+00 | -2.08E+02 | 4.30E+00 | 0.00E+00 | 3.57E+02 | 0.00E+00 |
| Terrestrial eco-toxicity (kg 1,4-DB eq) | 8.48E-03 | 1.21E-03 | 8.06E-04 | 1.79E-03 | 1.59E-04 | 4.54E-04 | 0 | 0.00E+00 | -1.73E-03 | 4.71E-05 | 0.00E+00 | 5.75E-03 | 0.00E+00 |
| POCP (kg C2H4) | 9.99E-04 | 3.50E-05 | 5.97E-05 | 6.61E-04 | 2.05E-05 | 3.33E-06 | 0 | 3.04E-04 | -1.43E-04 | 2.72E-06 | 0.00E+00 | 5.61E-05 | 0.00E+00 |

Table S7 Characterized LCIA profiles of VSRC Imola-derived bioethanol (LHW) at bio-refinery gate per kg bioethanol (method: CML 2 baseline 2000)

| Impact category | Total | Plantation | Pretreatment | Saccharification | Fermentation | Combustion | Make up water | Flue gas | Surplus electricity | Transport | C sequestration in poplar feedstock | Waste water treatment | C sequestration in soil |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abiotic depletion (kg Sb eq) | 1.75E-02 | 2.67E-03 | 0 | 2.33E-02 | 6.26E-04 | 1.13E-04 | 0 | 0.00E+00 | -9.38E-03 | 1.97E-04 | 0.00E+00 | 0 | 0.00E+00 |
| Acidification (kg SO2 eq) | 1.93E-02 | 2.94E-03 | 0 | 1.86E-02 | 5.81E-04 | 7.43E-05 | 0 | 3.20E-03 | -6.12E-03 | 8.34E-05 | 0.00E+00 | 0 | 0.00E+00 |
| Eutrophication (kg PO43- eq) | 3.16E-03 | 9.27E-04 | 0 | 1.74E-03 | 5.87E-04 | 1.05E-03 | 0 | 0.00E+00 | -1.16E-03 | 2.26E-05 | 0.00E+00 | 0 | 0.00E+00 |
| GWP100 (kg CO2 eq) | -4.27E+00 | 7.09E-01 | 0 | 3.10E+00 | 8.01E-02 | 4.12E-02 | 0 | 3.65E-03 | -1.33E+00 | 2.67E-02 | -1.91E+00 | 0 | -4.99E+00 |
| ODP (kg CFC-11 eq) | 1.98E-07 | 1.38E-07 | 0 | 1.51E-07 | 7.80E-09 | 2.10E-09 | 0 | 0.00E+00 | -1.05E-07 | 4.37E-09 | 0.00E+00 | 0 | 0.00E+00 |
| Human toxicity (kg 1,4-DB eq) | 1.68E+00 | 1.56E-01 | 0 | 9.53E-01 | 2.77E-02 | 7.96E-01 | 0 | 2.56E-04 | -2.64E-01 | 6.92E-03 | 0.00E+00 | 0 | 0.00E+00 |
| Fresh water eco-toxicity (kg 1,4-DB eq) | 3.21E-01 | 5.18E-02 | 0 | 1.82E-01 | 1.08E-02 | 2.19E-01 | 0 | 0.00E+00 | -1.46E-01 | 2.63E-03 | 0.00E+00 | 0 | 0.00E+00 |
| Marine aquatic eco-toxicity (kg 1,4-DB eq) | 6.12E+02 | 1.27E+02 | 0 | 6.23E+02 | 2.75E+01 | 1.93E+02 | 0 | 0.00E+00 | -3.64E+02 | 5.51E+00 | 0.00E+00 | 0 | 0.00E+00 |
| Terrestrial eco-toxicity (kg 1,4-DB eq) | 1.68E-03 | 1.55E-03 | 0 | 2.40E-03 | 1.94E-04 | 5.17E-04 | 0 | 0.00E+00 | -3.04E-03 | 6.03E-05 | 0.00E+00 | 0 | 0.00E+00 |
| POCP (kg C2H4) | 8.82E-04 | 4.49E-05 | 0 | 8.88E-04 | 2.51E-05 | 2.83E-06 | 0 | 1.68E-04 | -2.50E-04 | 3.49E-06 | 0.00E+00 | 0 | 0.00E+00 |

Table S8 Characterized LCIA comparison of SRC Imola-derived bioethanol (DA) over life cycle per functional unit, 100km driven in a FFV (method: CML 2 baseline 2000)

| Impact category | Total | Plantation | Bioethanol conversion process | Surplus electricity | Transport, poplar to biorefinery | C sequestration in poplar feedstock | C sequestration in soil | Transport, bioethanol | Use phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abiotic depletion (kg Sb eq) | 1.74E-01 | 1.76E-02 | 2.05E-01 | -5.29E-02 | 1.52E-03 | 0.00E+00 | 0.00E+00 | 2.52E-03 | 0.00E+00 |
| Acidification (kg SO2 eq) | 2.23E-01 | 1.93E-02 | 2.36E-01 | -3.45E-02 | 6.44E-04 | 0.00E+00 | 0.00E+00 | 1.07E-03 | 0.00E+00 |
| Eutrophication (kg PO43- eq) | 3.79E-02 | 6.12E-03 | 3.79E-02 | -6.57E-03 | 1.75E-04 | 0.00E+00 | 0.00E+00 | 2.89E-04 | 0.00E+00 |
| GWP100 (kg CO2 eq) | -1.39E+00 | 4.85E+00 | 2.77E+01 | -7.50E+00 | 2.06E-01 | -1.89E+01 | -2.71E+01 | 3.41E-01 | 1.91E+01 |
| ODP (kg CFC-11 eq) | 1.42E-06 | 4.54E-07 | 1.47E-06 | -5.92E-07 | 3.38E-08 | 0.00E+00 | 0.00E+00 | 5.58E-08 | 0.00E+00 |
| Human toxicity (kg 1,4-DB eq) | 1.64E+01 | 8.01E-01 | 1.69E+01 | -1.49E+00 | 5.35E-02 | 0.00E+00 | 0.00E+00 | 8.85E-02 | 0.00E+00 |
| Fresh water aquatic eco-toxicity (kg 1,4-DB eq) | 4.65E+00 | 3.03E-01 | 5.11E+00 | -8.21E-01 | 2.03E-02 | 0.00E+00 | 0.00E+00 | 3.36E-02 | 0.00E+00 |
| Marine aquatic eco-toxicity (kg 1,4-DB eq) | 9.22E+03 | 7.65E+02 | 1.04E+04 | -2.06E+03 | 4.26E+01 | 0.00E+00 | 0.00E+00 | 7.04E+01 | 0.00E+00 |
| Terrestrial eco-toxicity (kg 1,4-DB eq) | 8.21E-02 | 9.36E-03 | 8.87E-02 | -1.71E-02 | 4.66E-04 | 0.00E+00 | 0.00E+00 | 7.70E-04 | 0.00E+00 |
| POCP (kg C2H4) | 9.80E-03 | 1.84E-04 | 1.09E-02 | -1.41E-03 | 2.70E-05 | 0.00E+00 | 0.00E+00 | 4.46E-05 | 2.87E-05 |

Table S9 Characterized LCIA comparison of SRC Imola-derived bioethanol (LHW) over life cycle per functional unit, 100km driven in a FFV (method: CML 2 baseline 2000)

| Impact category | Total | Plantation | Bioethanol conversion process | Surplus electricity | Transport, poplar to bio-refinery | C sequestration in poplar feedstock | C sequestration in soil | Transport, bioethanol | Use phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abiotic depletion (kg Sb eq) | 1.72E-01 | 2.25E-02 | 2.38E-01 | -9.29E-02 | 1.95E-03 | 0.00E+00 | 0.00E+00 | 2.52E-03 | 0.00E+00 |
| Acidification (kg SO2 eq) | 1.88E-01 | 2.47E-02 | 2.22E-01 | -6.06E-02 | 8.25E-04 | 0.00E+00 | 0.00E+00 | 1.07E-03 | 0.00E+00 |
| Eutrophication (kg PO43- eq) | 3.03E-02 | 7.84E-03 | 3.34E-02 | -1.15E-02 | 2.24E-04 | 0.00E+00 | 0.00E+00 | 2.89E-04 | 0.00E+00 |
| GWP100 (kg CO2 eq) | -9.03E+00 | 6.22E+00 | 3.19E+01 | -1.32E+01 | 2.64E-01 | -1.89E+01 | -3.47E+01 | 3.41E-01 | 1.91E+01 |
| ODP (kg CFC-11 eq) | 1.23E-06 | 5.81E-07 | 1.59E-06 | -1.04E-06 | 4.33E-08 | 0.00E+00 | 0.00E+00 | 5.58E-08 | 0.00E+00 |
| Human toxicity (kg 1,4-DB eq) | 1.62E+01 | 1.03E+00 | 1.76E+01 | -2.61E+00 | 6.86E-02 | 0.00E+00 | 0.00E+00 | 8.85E-02 | 0.00E+00 |
| Fresh water eco-toxicity (kg 1,4-DB eq) | 3.08E+00 | 3.88E-01 | 4.08E+00 | -1.44E+00 | 2.61E-02 | 0.00E+00 | 0.00E+00 | 3.36E-02 | 0.00E+00 |
| Marine aquatic eco-toxicity (kg 1,4-DB eq) | 5.85E+03 | 9.80E+02 | 8.35E+03 | -3.61E+03 | 5.46E+01 | 0.00E+00 | 0.00E+00 | 7.04E+01 | 0.00E+00 |
| Terrestrial eco-toxicity (kg 1,4-DB eq) | 1.41E-02 | 1.20E-02 | 3.08E-02 | -3.01E-02 | 5.97E-04 | 0.00E+00 | 0.00E+00 | 7.70E-04 | 0.00E+00 |
| POCP (kg C2H4) | 8.59E-03 | 2.35E-04 | 1.07E-02 | -2.48E-03 | 3.46E-05 | 0.00E+00 | 0.00E+00 | 4.46E-05 | 2.87E-05 |

Table S10 Characterized LCIA comparison of VSRC Imola-derived bioethanol (DA) over life cycle per functional unit, 100km driven in a FFV (method: CML 2 baseline 2000)

| Impact category | Total | Plantation | Bioethanol conversion process | Surplus electricity | Transport, poplar to bio-refinery | C sequestration in poplar feedstock | C sequestration in soil | Transport, bioethanol | Use phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abiotic depletion (kg Sb eq) | 1.77E-01 | 2.06E-02 | 2.05E-01 | -5.29E-02 | 1.52E-03 | 0.00E+00 | 0.00E+00 | 2.52E-03 | 0.00E+00 |
| Acidification (kg SO2 eq) | 2.26E-01 | 2.28E-02 | 2.36E-01 | -3.45E-02 | 6.44E-04 | 0.00E+00 | 0.00E+00 | 1.07E-03 | 0.00E+00 |
| Eutrophication (kg PO43- eq) | 3.89E-02 | 7.16E-03 | 3.79E-02 | -6.57E-03 | 1.75E-04 | 0.00E+00 | 0.00E+00 | 2.89E-04 | 0.00E+00 |
| GWP100 (kg CO2 eq) | -1.22E+01 | 5.48E+00 | 2.77E+01 | -7.50E+00 | 2.06E-01 | -1.89E+01 | -3.85E+01 | 3.41E-01 | 1.91E+01 |
| ODP (kg CFC-11 eq) | 2.03E-06 | 1.06E-06 | 1.47E-06 | -5.92E-07 | 3.38E-08 | 0.00E+00 | 0.00E+00 | 5.58E-08 | 0.00E+00 |
| Human toxicity (kg 1,4-DB eq) | 1.68E+01 | 1.21E+00 | 1.69E+01 | -1.49E+00 | 5.35E-02 | 0.00E+00 | 0.00E+00 | 8.85E-02 | 0.00E+00 |
| Fresh water aquatic eco-toxicity (kg 1,4-DB eq) | 4.75E+00 | 4.00E-01 | 5.11E+00 | -8.21E-01 | 2.03E-02 | 0.00E+00 | 0.00E+00 | 3.36E-02 | 0.00E+00 |
| Marine aquatic eco-toxicity (kg 1,4-DB eq) | 9.44E+03 | 9.83E+02 | 1.04E+04 | -2.06E+03 | 4.26E+01 | 0.00E+00 | 0.00E+00 | 7.04E+01 | 0.00E+00 |
| Terrestrial eco-toxicity (kg 1,4-DB eq) | 8.47E-02 | 1.20E-02 | 8.87E-02 | -1.71E-02 | 4.66E-04 | 0.00E+00 | 0.00E+00 | 7.70E-04 | 0.00E+00 |
| POCP (kg C2H4) | 9.97E-03 | 3.47E-04 | 1.09E-02 | -1.41E-03 | 2.70E-05 | 0.00E+00 | 0.00E+00 | 4.46E-05 | 2.87E-05 |

Table S11 Characterized LCIA comparison of VSRC Imola-derived bioethanol (LHW) over life cycle per functional unit, 100km driven in a FFV (method: CML 2 baseline 2000)

| Impact category | Total | Plantation | Bioethanol conversion process | Surplus electricity | Transport, poplar to bio-refinery | C sequestration in poplar feedstock | C sequestration in soil | Transport, bioethanol | Use phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abiotic depletion (kg Sb eq) | 1.76E-01 | 2.64E-02 | 2.38E-01 | -9.29E-02 | 1.95E-03 | 0.00E+00 | 0.00E+00 | 2.52E-03 | 0.00E+00 |
| Acidification (kg SO2 eq) | 1.92E-01 | 2.92E-02 | 2.22E-01 | -6.06E-02 | 8.25E-04 | 0.00E+00 | 0.00E+00 | 1.07E-03 | 0.00E+00 |
| Eutrophication (kg PO43- eq) | 3.16E-02 | 9.18E-03 | 3.34E-02 | -1.15E-02 | 2.24E-04 | 0.00E+00 | 0.00E+00 | 2.89E-04 | 0.00E+00 |
| GWP100 (kg CO2 eq) | -2.29E+01 | 7.02E+00 | 3.19E+01 | -1.32E+01 | 2.64E-01 | -1.89E+01 | -4.94E+01 | 3.41E-01 | 1.91E+01 |
| ODP (kg CFC-11 eq) | 2.01E-06 | 1.36E-06 | 1.59E-06 | -1.04E-06 | 4.33E-08 | 0.00E+00 | 0.00E+00 | 5.58E-08 | 0.00E+00 |
| Human toxicity (kg 1,4-DB eq) | 1.67E+01 | 1.55E+00 | 1.76E+01 | -2.61E+00 | 6.86E-02 | 0.00E+00 | 0.00E+00 | 8.85E-02 | 0.00E+00 |
| Fresh water aquatic eco-toxicity (kg 1,4-DB eq) | 3.21E+00 | 5.13E-01 | 4.08E+00 | -1.44E+00 | 2.61E-02 | 0.00E+00 | 0.00E+00 | 3.36E-02 | 0.00E+00 |
| Marine aquatic eco-toxicity (kg 1,4-DB eq) | 6.13E+03 | 1.26E+03 | 8.35E+03 | -3.61E+03 | 5.46E+01 | 0.00E+00 | 0.00E+00 | 7.04E+01 | 0.00E+00 |
| Terrestrial eco-toxicity (kg 1,4-DB eq) | 1.74E-02 | 1.53E-02 | 3.08E-02 | -3.01E-02 | 5.97E-04 | 0.00E+00 | 0.00E+00 | 7.70E-04 | 0.00E+00 |
| POCP (kg C2H4) | 8.80E-03 | 4.44E-04 | 1.07E-02 | -2.48E-03 | 3.46E-05 | 0.00E+00 | 0.00E+00 | 4.46E-05 | 2.87E-05 |

Table S12 Ccharacterized LCIA comparison of E100 bioethanol (soil carbon sequestration included) vs. petrol per functional unit, 100km driven in a FFV (method: CML 2 baseline 2000)

| Impact category | SRC, DA | SRC, LHW | VSRC, DA | VSRC, LHW | Petrol |
| --- | --- | --- | --- | --- | --- |
| Abiotic depletion (kg Sb eq) | 1.74E-01 | 1.72E-01 | 1.77E-01 | 1.76E-01 | 1.64E-01 |
| Acidification (kg SO2 eq) | 2.23E-01 | 1.88E-01 | 2.26E-01 | 1.92E-01 | 1.02E-01 |
| Eutrophication (kg PO43- eq) | 3.79E-02 | 3.03E-02 | 3.89E-02 | 3.16E-02 | 1.91E-02 |
| GWP100 (kg CO2 eq) | -1.39E+00 | -9.03E+00 | -1.22E+01 | -2.29E+01 | 2.63E+01 |
| ODP (kg CFC-11 eq) | 1.42E-06 | 1.23E-06 | 2.03E-06 | 2.01E-06 | 3.10E-06 |
| Human toxicity (kg 1,4-DB eq) | 1.64E+01 | 1.62E+01 | 1.68E+01 | 1.67E+01 | 2.87E+00 |
| Fresh water aquatic eco-toxicity (kg 1,4-DB eq) | 4.65E+00 | 3.08E+00 | 4.75E+00 | 3.21E+00 | 6.08E-01 |
| Marine aquatic eco-toxicity (kg 1,4-DB eq) | 9.22E+03 | 5.85E+03 | 9.44E+03 | 6.13E+03 | 2.59E+03 |
| Terrestrial eco-toxicity (kg 1,4-DB eq) | 8.21E-02 | 1.41E-02 | 8.47E-02 | 1.74E-02 | 1.72E-02 |
| POCP (kg C2H4) | 9.80E-03 | 8.59E-03 | 9.97E-03 | 8.80E-03 | 2.99E-02 |

Table S13 Ccharacterized LCIA comparison ofE100 bioethanol (soil carbon sequestration excluded) vs. petrol per functional unit, 100km driven in a FFV (method: CML 2 baseline 2000)

| Impact category | SRC, DA | SRC, LHW | VSRC, DA | VSRC, LHW | Petrol |
| --- | --- | --- | --- | --- | --- |
| Abiotic depletion (kg Sb eq) | 1.74E-01 | 1.72E-01 | 1.77E-01 | 1.76E-01 | 1.64E-01 |
| Acidification (kg SO2 eq) | 2.23E-01 | 1.88E-01 | 2.26E-01 | 1.92E-01 | 1.02E-01 |
| Eutrophication (kg PO43- eq) | 3.79E-02 | 3.03E-02 | 3.89E-02 | 3.16E-02 | 1.91E-02 |
| GWP100 (kg CO2 eq) | 2.57E+01 | 2.57E+01 | 2.63E+01 | 2.65E+01 | 2.63E+01 |
| ODP (kg CFC-11 eq) | 1.42E-06 | 1.23E-06 | 2.03E-06 | 2.01E-06 | 3.10E-06 |
| Human toxicity (kg 1,4-DB eq) | 1.64E+01 | 1.62E+01 | 1.68E+01 | 1.67E+01 | 2.87E+00 |
| Fresh water aquatic eco-toxicity (kg 1,4-DB eq) | 4.65E+00 | 3.08E+00 | 4.75E+00 | 3.21E+00 | 6.08E-01 |
| Marine aquatic eco-toxicity (kg 1,4-DB eq) | 9.22E+03 | 5.85E+03 | 9.44E+03 | 6.13E+03 | 2.59E+03 |
| Terrestrial eco-toxicity (kg 1,4-DB eq) | 8.21E-02 | 1.41E-02 | 8.47E-02 | 1.74E-02 | 1.72E-02 |
| POCP (kg C2H4) | 9.80E-03 | 8.59E-03 | 9.97E-03 | 8.80E-03 | 2.99E-02 |

Table S14 Characterized cradle-to-grave LCIA comparison of E100 bioethanol from SRC and VSRC Imola Poplar per functional unit, 100km driven in a FFV (method: CML 2 baseline 2000)

| Impact category | SRC, DA | SRC, DA | SRC, DA | SRC, LHW | SRC, LHW | SRC, LHW | VSRC, DA | VSRC, DA | VSRC, DA | VSRC, LHW | VSRC, LHW | VSRC, LHW | Petrol |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | grid electricity | green electricity | flood irrigation | grid electricity | green electricity | flood irrigation | grid electricity | green electricity | flood irrigation | grid electricity | green electricity | flood irrigation |  |
| Abiotic depletion (kg Sb eq) | 1.74E-01 | 1.65E-01 | 1.64E-01 | 1.72E-01 | 1.61E-01 | 1.60E-01 | 1.77E-01 | 1.66E-01 | 1.65E-01 | 1.76E-01 | 1.62E-01 | 1.61E-01 | 1.64E-01 |
| Acidification (kg SO2 eq) | 2.23E-01 | 2.17E-01 | 2.16E-01 | 1.88E-01 | 1.80E-01 | 1.80E-01 | 2.26E-01 | 2.19E-01 | 2.18E-01 | 1.92E-01 | 1.83E-01 | 1.82E-01 | 1.02E-01 |
| Eutrophication(kg PO43- eq) | 3.79E-02 | 3.66E-02 | 3.65E-02 | 3.03E-02 | 2.87E-02 | 2.85E-02 | 3.89E-02 | 3.74E-02 | 3.72E-02 | 3.16E-02 | 2.97E-02 | 2.94E-02 | 1.91E-02 |
| GWP100 (kg CO2 eq) | -1.39E+00 | -1.50E+00 | -2.73E+00 | -9.03E+00 | -9.17E+00 | -1.07E+01 | -1.22E+01 | -1.23E+01 | -1.38E+01 | -2.29E+01 | -2.31E+01 | -2.50E+01 | 2.63E+01 |
| ODP (kg CFC-11 eq) | 1.42E-06 | 1.33E-06 | 1.32E-06 | 1.23E-06 | 1.11E-06 | 1.10E-06 | 2.03E-06 | 1.92E-06 | 1.91E-06 | 2.01E-06 | 1.86E-06 | 1.85E-06 | 3.10E-06 |
| Human toxicity (kg 1,4-DB eq) | 1.64E+01 | 1.61E+01 | 1.60E+01 | 1.62E+01 | 1.58E+01 | 1.57E+01 | 1.68E+01 | 1.65E+01 | 1.64E+01 | 1.67E+01 | 1.63E+01 | 1.62E+01 | 2.87E+00 |
| Fresh water eco-toxicity (kg 1,4-DB eq) | 4.65E+00 | 4.47E+00 | 4.46E+00 | 3.08E+00 | 2.85E+00 | 2.84E+00 | 4.75E+00 | 4.53E+00 | 4.51E+00 | 3.21E+00 | 2.93E+00 | 2.91E+00 | 6.08E-01 |
| Marine aquatic eco-toxicity (kg 1,4-DB eq) | 9.22E+03 | 8.79E+03 | 8.76E+03 | 5.85E+03 | 5.29E+03 | 5.25E+03 | 9.44E+03 | 8.91E+03 | 8.88E+03 | 6.13E+03 | 5.44E+03 | 5.40E+03 | 2.59E+03 |
| Terrestrial eco-toxicity (kg 1,4-DB eq) | 8.21E-02 | 7.68E-02 | 7.63E-02 | 1.41E-02 | 7.26E-03 | 6.58E-03 | 8.47E-02 | 7.82E-02 | 7.76E-02 | 1.74E-02 | 9.11E-03 | 8.28E-03 | 1.72E-02 |
| POCP (kg C2H4) | 9.80E-03 | 9.56E-03 | 9.54E-03 | 8.59E-03 | 8.29E-03 | 8.25E-03 | 9.97E-03 | 9.67E-03 | 9.64E-03 | 8.80E-03 | 8.43E-03 | 8.39E-03 | 2.99E-02 |

Table S15 Sensitivity analysis on allocation approach per functional unit, 100km driven in a FFV (method: CML 2 baseline 2000)

| Impact category | SRC, DA, substitution | SRC, DA, energy | SRC, LHW, substitution | SRC, LHW, energy | VSRC, DA, substitution | VSRC, DA, energy | VSRC, LHW, substitution | VSRC, LHW, energy | Petrol |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abiotic depletion (kg Sb eq) | 1.74E-01 | 1.96E-01 | 1.72E-01 | 2.07E-01 | 1.77E-01 | 1.98E-01 | 1.76E-01 | 2.11E-01 | 1.64E-01 |
| Acidification (kg SO2 eq) | 2.23E-01 | 2.22E-01 | 1.88E-01 | 1.94E-01 | 2.26E-01 | 2.25E-01 | 1.92E-01 | 1.98E-01 | 1.02E-01 |
| Eutrophication (kg PO43- eq) | 3.79E-02 | 3.83E-02 | 3.03E-02 | 3.27E-02 | 3.89E-02 | 3.92E-02 | 3.16E-02 | 3.37E-02 | 1.91E-02 |
| GWP100 (kg CO2 eq) | -1.39E+00 | 7.95E+00 | -9.03E+00 | 7.48E+00 | -1.22E+01 | -1.38E+00 | -2.29E+01 | -3.35E+00 | 2.63E+01 |
| ODP (kg CFC-11 eq) | 1.42E-06 | 1.75E-06 | 1.23E-06 | 1.79E-06 | 2.03E-06 | 2.27E-06 | 2.01E-06 | 2.40E-06 | 3.10E-06 |
| Human toxicity (kg 1,4-DB eq) | 1.64E+01 | 1.54E+01 | 1.62E+01 | 1.47E+01 | 1.68E+01 | 1.58E+01 | 1.67E+01 | 1.51E+01 | 2.87E+00 |
| Fresh water aquatic eco-toxicity (kg 1,4-DB eq) | 4.65E+00 | 4.72E+00 | 3.08E+00 | 3.54E+00 | 4.75E+00 | 4.80E+00 | 3.21E+00 | 3.64E+00 | 6.08E-01 |
| Marine aquatic eco-toxicity (kg 1,4-DB eq) | 9.22E+03 | 9.73E+03 | 5.85E+03 | 7.40E+03 | 9.44E+03 | 9.92E+03 | 6.13E+03 | 7.62E+03 | 2.59E+03 |
| Terrestrial eco-toxicity (kg 1,4-DB eq) | 8.21E-02 | 8.57E-02 | 1.41E-02 | 3.46E-02 | 8.47E-02 | 8.79E-02 | 1.74E-02 | 3.73E-02 | 1.72E-02 |
| POCP (kg C2H4) | 9.80E-03 | 9.68E-03 | 8.59E-03 | 8.66E-03 | 9.97E-03 | 9.82E-03 | 8.80E-03 | 8.83E-03 | 2.99E-02 |

Table S16 Sensitivity analysis on characterization model per functional unit, 100km driven in a FFV (method: Eco-indicator 99 H)

| Impact category | Unit | SRC, DA | SRC, LHW | VSRC, DA | VSRC, LHW | Petrol |
| --- | --- | --- | --- | --- | --- | --- |
| Carcinogens | DALY | 4.85E-06 | 3.67E-06 | 4.94E-06 | 3.79E-06 | 4.34E-07 |
| Resp. organics | DALY | 1.55E-08 | 1.68E-08 | 1.58E-08 | 1.71E-08 | 1.36E-07 |
| Resp. inorganics | DALY | 1.35E-05 | 1.03E-05 | 1.38E-05 | 1.07E-05 | 1.24E-05 |
| Climate change | DALY | -2.46E-07 | -1.83E-06 | -2.51E-06 | -4.74E-06 | 5.52E-06 |
| Radiation | DALY | 6.36E-08 | 2.34E-08 | 6.58E-08 | 2.63E-08 | 1.05E-08 |
| Ozone layer | DALY | 1.50E-09 | 1.30E-09 | 2.14E-09 | 2.12E-09 | 3.26E-09 |
| Eco-toxicity | PAF*m2yr | 3.62E+00 | 2.08E+00 | 3.74E+00 | 2.23E+00 | 9.90E-01 |
| Acidification/ Eutrophication | PDF*m2yr | 4.93E-01 | 4.16E-01 | 5.06E-01 | 4.32E-01 | 6.80E-01 |
| Minerals | MJ surplus | 3.05E-01 | 1.24E-01 | 3.20E-01 | 1.44E-01 | 3.60E-02 |
| Fossil fuels | MJ surplus | 4.59E+01 | 5.10E+01 | 4.65E+01 | 5.18E+01 | 4.94E+01 |

Note: DALY=Disability Adjusted Life Years; PAF=Potentially Affected Fraction; PDF= Potentially Disappeared Fraction.

References

1.	J. Littlewood, M. Guo, W. Boerjan and R. J. Murphy, Biotechnology for Biofuels 2014.

2.	C. E. Wyman, B. E. Dale, R. T. Elander, M. Holtzapple, M. R. Ladisch, Y. Y. Lee, C. Mitchinson and J. N. Saddler, Biotechnol. Prog., 2009, 25, 333-339.

3.	D. Humbird, R. Davis, L. Tao, C. Kinchin, D. Hsu, A. Aden, P. Schoen, J. Lukas, B. Olthof, M. Worley, D. Sexton and D. Dudgeon, National Renewable Energy Laboratory, 2011.

4.	DepartmentforTranport, ed. D. f. transport, London, Colorado, USA, 2012.

5.	M. Guo, J. Littlewood, J. Joyce and R. Murphy, Green Chemistry, 2014, 16, 4680-4695.