| M1B |  |  |  |
| --- | --- | --- | --- |
| - Characteristics of the biogas |  |  |  |
| Total Biogas production: L/working volume of the bioreactor/d |  | 36.3 ± 3.3 |  |
| Composition of biogas [%]: |  |  |  |
| methane |  | 71.9 ± 0.4 |  |
| carbon dioxide |  | 28.0 ± 1.1 |  |
| hydrogen |  | 0.01 ± 0.01 |  |
| hydrogen sulphide |  | 0.01 ± 0.01 |  |
| Methane production: |  |  |  |
| L-CH4/working volume of the bioreactor/d |  | 25.0 ± 2.0 |  |
| L-CH4/g COD |  | 3.4 ± 0.3 |  |
| - Characteristics of the substrate and effluent after the methanogenic process |  |  |  |
|  | Substrate |  | Effluent |
| COD (g O2/L) | 12.5 ±1.8 |  | 0.6 ± 0.3 |
| concentration of sulphide (mg/L) | <0.05 * |  | 0.01 ± 0.0 |
| pH | 4.91 ± 0.12 |  | 7.40 ± 0.1 |
| Redox potential in the UASB bioreactor (mV) ** |  |  | (-310)–(-320) |

Additional file 8. Performance of the M1B methane-yielding microbial community processing a lactate-rich artificial medium between 44th and 50th week of cultivation.

* – the limit of quantification

** – according to hydrogen electrode