Additional file 1

Evolutionary engineering of Lactobacillus bulgaricus reduces enzyme usage and enhances conversion of lignocellulosics to D-lactic acid by simultaneous saccharification and fermentation

Vishnu Prasad J1#, Tridweep K. Sahoo1#, Naveen S., Guhan Jayaraman1*

1Bioprocess and Metabolic Engineering Laboratory,

Department of Biotechnology, Bhupat and Jyoti Mehta School of Biosciences,

Indian Institute of Technology-Madras, Chennai, Tamil Nadu, India, 600036

# These authors have contributed equally to the work.

*Corresponding author: Guhan Jayaraman

email id: guhanj@iitm.ac.in

Table S1. Comparison of titer and yield of D-LA among previously reported literature

Figure S1. Growth profile of L. bulgaricus WT on 30 g/L glucose at 37 °C in MRS media of a) 6 pH b) 5.5 pH . Values are mean ± SD (n=3).

Figure S2. Growth profile of L. bulgaricus WT in LCB hydrolysate (SHF) at 37 °C and pH 5.5. Values are mean ± SD (n=3).

Figure S3. Effect of temperature on enzymatic cellulose hydrolysis with 10% (w/v) solid loading using 5 FPU/gLCB at pH 5.5. Values are mean ± SD (n=3).

Figure S4.  Tukey Simultaneous 95% Confidence Intervals –of one way ANOVA comparing the effect of temperature on enzymatic cellulosic hydrolysis with 10% (w/v) solid loading using at pH 5.5 at enzyme loadings of a) 5 FPU/gLCB and b) 15 FPU/gLCB. Values are mean ± SD (n=3).

Figure S5. Effect of Cellobiose on cellulose hydrolysis with 10% (w/v) solid loading at pH 5.5 and 15 FPU/gLCB. Time course profile of (a) net cellobiose accumulation (b) percentage cellulose conversion. Values are mean ± SD (n=3).

Figure S6. Effect of glucose removal on enzymatic cellulose hydrolysis with different enzyme loadings. Values are mean ± SD (n=3).

Figure S7.  Growth profile of L. bulgaricus WT on 30 g/L glucose at in MRS media of pH 5.5 at a) 40 °C and b) 42 °C . Values are mean ± SD (n=3).

Figure S8. Growth profile of L. bulgaricus ET45 at 45 °C and pH 5.5 in a) 30 g/L glucose b) lignocellulosic hydrolysate containing 30 g/L glucose (SHF). Values are mean ± SD (n=3).

Figure S9. Production D-LA in batch SSF at pH 5.5 with 5 % (w/v) solid loading by a) L. bulgaricus ET45 at 5 FPU/gLCB and 45 °C (b) L. bulgaricus ET45 at 3 FPU/gLCB and 45 °C (c) L. bulgaricus WT at 5 FPU/gLCB and 37 °C. Values are mean ± SD (n=3).

Table S1: Comparison of reported titer and yield of D-LA produced from LCB in literature and current work

| Strain | Method | Substrate | Yield (g/g) | Titer (g/L) | Conditions## | PER (g/FPU) | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Lactobacillus coryniformis subsp.torquens ATCC 25600 | Batch | Filter paper | 0.89 | ~25 | 39 °C/ pH 5.4 33.33 g/L 28 FPU | 0.027 | Yanez et al. 2003 [45] |
| Lactobacillus coryniformis ssp torquens ATCC 25600 | Batch | Alkali treated waste cardboard | 0.68 | 13.9 | 39 °C/ pH 5.4 33.33 g/L 28 FPU | 0.015 | Yanez et al. 2005 [7] |
|  | Fed Batch |  | 0.57 | 23.4 | 39 °C/ pH 5.4 66.66 g/L 14 FPU | 0.025 |  |
| Lactobacillus lactis mutant RM2-24 | Batch | Alpha cellulose (from sugarcane bagasse) | 0.73 | 73 | 42 °C/ pH 6.5 100 g/L 10 FPU | 0.073 | Singhvi et al. 2010 [8] |
| L. delbrueckii ATCC 9649 | Batch | Pulp | 0.48# | 19.2 | 40 °C/ pH 5.5 40 g/L 8 FPU | 0.06 | Zhang and Vadlani 2013 [9] |
|  |  | Alkali pretreated corn Stover | 0.50# | 20.1 |  | 0.063 |  |
| L. plantarum NCIMB 8826 ΔldhL1::PxylAB-Δxpk1::tkt-Δxpk2::PxylAB | Batch | Delignified hardwood pulp | 0.88* | 122.8 | 37 °C/ pH 6 150 g/L 20 FPU | 0.041 | Hama et al. 2015 [46] |
| Pediococcus acidilactici zp26 | Batch | Dilute acid pretreated corn stover | 0.58 | 76.8 | 45 °C/ pH 5.5 25% (w/w) 15 FPU | 0.015 | Yi et al. 2016 [10] |
| L. plantarum NCIMB 8826 ΔldhL1-pLEM415-xylAB | Batch | Alkali pretreated corn Stover | 0.73*# | 29.4 | 37 °C/ pH 5.5 40 g/L 8 FPU | 0.092 | Zhang et al 2016 [11] |
|  | Fed-batch |  | 0.77*# | 61.4 | 45 °C 80 g/L 5.6 FPU | 0.137 |  |
| L. bulgaricus | Batch | Organosolv pretreated beech wood | 0.69# | 62 | 44 °C/pH 5.5 90 g/L 9.6 FPU | 0.072 | Karnaouriet al 2020 [30] |
| L. bulgaricus ET45 | Fed- batch | Dilute acid pretreated rice straw | 0.67 | 108.6 | 45 °C/pH 5.5 350 g/L 5 FPU | 0.062 | This study |

*D-LA is formed from both glucose and xylose
Yield is calculated by the amount of D-LA divided by the amount of cellulose present in the biomass pretreated LCB
#product overall yield was calculated by the amount of D-lactic acid produced divided by the total amount of LCB used
##Indicates the experimental conditions in the order of Temperature, pH, Biomass loading and Enzyme loading in FPU per gram of LCB; PER is the amount of D-LA produced in grams per FPU of enzyme.

Figure S1. Growth profile of L. bulgaricus WT on 30 g/L glucose at 37 °C in MRS media of a) 6 pH b) 5.5 pH. Values are mean ± SD (n=3).

Figure S2. Growth profile of L. bulgaricus WT in LCB hydrolysate (SHF) at 37 °C and pH 5.5. Values are mean ± SD (n=3).

Figure S3: Effect of temperature on enzymatic cellulose hydrolysis with 10% (w/v) solid loading using 5 FPU/gLCB at pH 5.5. FPU-37 denotes the corresponding enzyme loading (5 FPU/gLCB) at 37 °C, FPU- 45 denotes the corresponding enzyme loading (5 FPU/gLCB) at 45 °C, FPU-50 denotes the corresponding enzyme loading (5 FPU/gLCB) at 50 °C. Values are mean ± SD (n=3).

Figure S4: Tukey Simultaneous 95% Confidence Intervals –of one way ANOVA comparing the effect of temperature on enzymatic cellulosic hydrolysis with 10% (w/v) solid loading using at pH 5.5 at enzyme loadings of a) 5 FPU/gLCB and b) 15 FPU/gLCB. The plot compares the means between the three operating temperatures (If an interval does not contain zero, the corresponding means are significantly different.).

Figure S5: Effect of Cellobiose on cellulose hydrolysis with 10% (w/v) solid loading at pH 5.5 and 15 FPU/gLCB. Time course profile of (a) net cellobiose accumulation (b) percentage cellulose conversion. Values are mean ± SD (n=3).

Figure S6: Effect of glucose removal on enzymatic cellulose hydrolysis with different enzyme loadings (5, 15, 30 FPU/gLCB) using 10% (w/v) solid loading at pH 5.5 and temperature 45 °C. FPU-GR denotes the corresponding enzyme loading (FPU/gLCB) with glucose removal, FPU-C denotes the corresponding enzyme loading (FPU/gLCB) without glucose removal (control). Values are mean ± SD (n=3).

Figure S7.  Growth profile of L. bulgaricus WT on 30 g/L glucose at in MRS media of pH 5.5 at a) 40 °C and b) 42 °C. Values are mean ± SD (n=3).

Figure S8. Growth profile of L. bulgaricus ET45 at 45 °C and pH 5.5 in a) 30 g/L glucose b) lignocellulosic hydrolysate containing 30 g/L glucose (SHF). Values are mean ± SD (n=3).

Figure S9: Production D-LA in batch SSF at pH 5.5 with 5 % (w/v) solid loading a) L. bulgaricus ET45 at 5 FPU/gLCB and 45 °C (b) L. bulgaricus ET45 at 3 FPU/gLCB and 45 °C (c) L. bulgaricus WT at 5 FPU/gLCB and 37 °C. Values are mean ± SD (n=3).