Supplementary information

Fig S1. Design matrix with coded levels and real values (in parenthesis) for a three-level full factorial design

| Run | Coded value |  | Real values |  |
| --- | --- | --- | --- | --- |
|  | A | B | A: Time (h) | B: Glucose concentration (g L-1) |
| 1 | +1 | - 1 | 8.0 | 5.0 |
| 2 | 0 | - 1 | 8.0 | 2.5 |
| 3 | - 1 | - 1 | 8.0 | 1.0 |
| 4 | 0 | +1 | 53.0 | 2.5 |
| 5 | - 1 | +1 | 53.0 | 1.0 |
| 6 | +1 | 0 | 30.0 | 5.0 |
| 7 | 0 | +1 | 53.0 | 2.5 |
| 8 | - 1 | - 1 | 8.0 | 1.0 |
| 9 | +1 | +1 | 53.0 | 5.0 |
| 10 | +1 | 0 | 30.0 | 5.0 |
| 11 | 0 | 0 | 30.0 | 2.5 |
| 12 | - 1 | 0 | 30.0 | 1.0 |
| 13 | +1 | - 1 | 8.0 | 5.0 |
| 14 | - 1 | +1 | 53.0 | 1.0 |
| 15 | - 1 | 0 | 30.0 | 1.0 |
| 16 | 0 | 0 | 30.0 | 2.5 |
| 17 | 0 | - 1 | 8.0 | 2.5 |
| 18 | +1 | +1 | 53.0 | 5.0 |

Fig S2. Calibration curve for tested parameters in HPLC using laboratory grade reagents

Fig S3. Calibration curve for ethanol quantification in GC using laboratory grade reagents

Table S1. High-Performance Liquid Chromatographer instruments and chromatographic conditions

| Instrument | Perkin Elmer® Flexar™ LC |
| --- | --- |
| Column | RestekTM Ultra Amino (150 mm column length , 4.6 mm inside diameter, 3.0 μm particle size, 100 A° pore size) |
| Injector | Rheodyne (20 μL loop) |
| Injection volume (μL) | 20 |
| Mobile phase | Acetonitrile (70%): Water (30%) |
| Carrier flow (mL min-1) | 0.8 |
| Detector temperature (°C) | 35 |
| Detector | Refractive Index (RI) Water 410 |

Table S2. Gas Chromatographer instruments and chromatographic conditions

| Instrument | Shimadzu® GC 2010plus |
| --- | --- |
| Column | GC Zebron® ZB-WAXplus™ (30 m length, 0.26 mm internal diameter, 0.25 μm film thickness) |
| Injector | Shimadzu® AOC 20i (1.0 μL Loop) |
| Injection volume (μL) | 1.0 |
| Mobile phase | Ultrapure Helium (He) ≥ 99.0% |
| Carrier pressure (psi) | 21.2 |
| Carrier flow (mL min-1) | 2.0 |
| Column oven temperature (°C) | 250 |
| Rate of temperature increase (Ramp) (°C min-1) | 50 |
| Detector temperature (°C) | 50 |
| Detector | Barrier discharge ionization detector (BID 2010plus) |