Additional file 15

Assuming the ideal gas law, hydrogen production of the polymer electrolyte membrane water electrolyser was calculated based on the second Faraday law as follows:

Q=I×t  A×s                                                                                                                                          (S15-1)

Q=n×z×F A×s                                                                                                                                  (S15-2)

Where Q is the electric charge ((A×s) or C), I is the current (A), t is the time (s), n is the amount of substance (in this case H2 (mole)), z is the number of electrons required for separating a molecule of substance (z(H2) =2) and F is Faraday constant (96485 (A×s)/mol).

The amount of H2 produced in 1 (s) and by applying 1 (A) ( φmole) is calculated as follows:

φmole=nI×t=1z×F=12×96485 (A×S) mol=5.1812×10-6 (molA×s)                                             (S15-3)

Considering the molar volume of gas at standard conditions is 22.414 (L), φvol or the volume of H2 produced in 1 (min) by applying 1 (A) is calculated as follows:

φvol= φmole×22.414NLmole×60 (smin) = 6.696×10-3NLA×min                         (S15-4)