Table S1. Operating conditions of other authors

| Treatment | Electrodes | Conditions | Salts | EPS | Methane yield | Solubilization degree | Mineralization | Energy analysis | Removal % | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EC-DA (35 days) | Fe Effective area 16 cm2 | 25 mA/cm² 120 min Electrode spacing 2 cm | 1.8 g/L H2O2 | - | 78% (102.7 mL of CH4) | 28% | 37.5 mA/cm² there is mineralization | - | COD removal 30.2% VS removal 28.3% | (Feki et al., 2015) |
| EC-DA (40 days) | BDD Effective area 42 cm2 | 6.6 mA/cm² 60 min | Na2SO4 NaOH | pH 10 favors breakage | 18% | - | - | - | VS removal 22.2% | (Arenas et al., 2021) |
| EC-DA (16 days) | BDD Effective area 70 cm2 | 28.6 mA/cm² 30 min Electrode spacing 1 mm | - | - | 76% (305 LCH4/kgVS) | 2.8% | - | 1.69 kW-h/kgVS | COD removal 50.8% VS removal 39.8% | (Barrios et al., 2021) |
| EC-DA (40 days) | Ti/RuO2 Effective area 40 cm2 | 125 mA/cm² 40 min Electrode spacing 5 cm | ClO- 0.6% | - | 61.1% | 11.8% | - | $23.53/ton solids | COD removal 39.7% VS removal 38.6% | (Ye et al., 2016) |
| EC-DA (40 days) | Ti/RuO2 | 25 A 45 min | NaClO 0.4% | Releases EPS | 47% (81.4 m3/m3 WAS) | 6.65% | - | 3196 kW-h/kgVS | VS removal 18.08% | (Yuan et al., 2016) |
| EC-DA (60 days) | Ti/RuO2 Effective area 43.65 cm2 | 6 mA/cm² 35 min Electrode spacing 2 cm | H2SO4 NaCl | Releases EPS | 30.8-46.9% | - | - | - | COD removal 61.5% VS removal 47.2% | (Heng et al., 2021) |