Macros for immunofluorescence analysis

Below is the text to be used in the ImageJ macro system (IJ1 macro language) for the automated analysis of all area and markers described in the manuscript. Before running macros, regions of interest must manually be defined, saved as an ROI set, and the ROI set must be open. Image files must also be organized in the image folder as described in each macro. Minimum and maximum thresholds for each channel must also be determined manually then inserted into the macro where indicated.

Macro #1: total cortical and hippocampal GFAP, X-34, and CD31 area; vessel-association GFAP in the cortex and hippocampus; X-34 plaque-associated GFAP in the cortex and hippocampus

//images are organized in the following order for each brain section: GFAP, CD31, merged, X-34

input="(insert mapping to image folder here)";

images = getFileList(input);

images = Array.sort(images);

GFAPmin = (insert threshold min for GFAP here)

GFAPmax = (insert threshold max for GFAP here)

X34min = (insert threshold min for X-34 here)

X34max = (insert threshold max for X-34 here)

//i=CD31 image

//j=GFAP image

//k=ROI hippocampus, ROIset already open and organized (list order cortex then hippocampus for each sections)

//l=ROI cortex, ROIset already open and organized (list order cortex then hippocampus for each sections)

//m=X-34 image

for (i=3; i<images.length; i+=4) {

j=i-3;

m=i-1;

if (j==0){

l=0;

k=1;

} else {

l=j/2;

k=l+1;

}

//make mask of CD31

open(input+images[i]);

run("8-bit");

run("Auto Local Threshold", "method=Bernsen radius=15 parameter_1=0 parameter_2=0 white");

run("Create Mask");

close(images[i]);

//measure GFAP within CD31 mask

//convert GFAP into mask

open(input+images[j]);

print("opening image "+images[j]);

run("8-bit");

setThreshold(GFAPmin, GFAPmax);

run("Convert to Mask");

imageCalculator("AND create", images[j],"mask");

//measure GFAP within CD31 mask, cortex then hippocampus

selectWindow("Result of "+images[j]);

roiManager("Select", l);

run("Analyze Particles...", "clear summarize");

roiManager("Select", k);

run("Analyze Particles...", "clear summarize");

//measure CD31 mask area within ROI, cortex then hippocampus

selectWindow("mask");

roiManager("Select", l);

run("Analyze Particles...", "clear summarize");

roiManager("Select", k);

run("Analyze Particles...", "clear summarize");

//measure GFAP area within cortical or hippocampal ROI, cortex then hippocampus

selectWindow(images[j]);

roiManager("Select", l);

run("Analyze Particles...", "clear summarize");

roiManager("Select", k);

run("Analyze Particles...", "clear summarize");

close(images[j]);

close("Result of "+images[j]);

//measure X-34 area within ROI

open(input+images[m]);

print("opening image "+images[m]);

run("8-bit");

setThreshold(X34min, X34max);

selectWindow(images[m]);

roiManager("Select", l);

run("Analyze Particles...", "clear summarize");

roiManager("Select", k);

run("Analyze Particles...", "clear summarize");

//measure GFAP around plaques

//make mask of X-34

selectWindow(images[m]);

run("Create Mask");

//convert GFAP into X-34 mask

open(input+images[j]);

print("opening image "+images[j]);

run("8-bit");

setThreshold(GFAPmin, GFAPmax);

run("Convert to Mask");

imageCalculator("AND create", images[j], "mask");

//measure GFAP in X-34 mask, cortex then hippocampus

roiManager("Select", l);

run("Analyze Particles...", "clear summarize");

roiManager("Select", k);

run("Analyze Particles...", "clear summarize");

run("Close All");

}

Macro #2: cerebral amyloid angiopathy (CAA) area and GFAP-associated CAA

//images are organized in the following order for each brain sections: GFAP, CD31, merged, X-34

input="(insert mapping to image folder here)";

images = getFileList(input);

images = Array.sort(images);

GFAPmin = (insert threshold min for GFAP here)

GFAPmax = (insert threshold max for GFAP here)

X34min = (insert threshold min for X-34 here)

X34max = (insert threshold max for X-34 here)

for (i=3; i<images.length; i++) {

print(images[i]);

}

CAAinput="(insert mapping to CAA manual mask folder here)";

CAAmask = getFileList(CAAinput);

CAAmask = Array.sort(CAAmask);

for (k=0; k<CAAmask.length; k++) {

print(images[k]);

}

//i=CD31 image

//j=GFAP image

//m=X-34 image

//k=CAA mask

//l=ROIcortex

for (i=3; i<images.length; i+=4) {

j=i-3;

m=i-1;

if (j==0) {

k=0;

l=0;

} else {

k=(j/4);

l=j/4;

}

//open mask

open(CAAinput+CAAmask[k]);

//convert X-34 into CAA manual mask

open(input+images[m]);

print("opening image "+images[m]);

run("8-bit");

setThreshold(X34min, X34max);

run("Convert to Mask");

imageCalculator("AND create", images[m],CAAmask[k]);

setThreshold(1, 255);

run("Create Mask");

//measure X-34 in CAA manual mask

roiManager("Select", l);

run("Analyze Particles...", "clear summarize");

close(images[m]);

close("Result of "+images[m]);

close(CAAmask[k]);

//create GFAP in CD31 mask

//open CD31

open(input+images[i]);

run("8-bit");

run("Auto Local Threshold", "method=Bernsen radius=15 parameter_1=0 parameter_2=0 white");

//convert GFAP into CD31

open(input+images[j]);

print("opening image "+images[j]);

run("8-bit");

setThreshold(GFAPmin, GFAPmax);

run("Convert to Mask");

imageCalculator("AND create", images[j], images[i]);

//convert CAA into GFAP vascular mask

run("Convert to Mask");

imageCalculator("AND create", "mask", "Result of "+images[j]);

//measure vascular GFAP and CAA co-localization

roiManager("Select", l);

run("Analyze Particles...", "clear summarize");

run("Close All");

}

Macro #3: total GFAP area in the hypothalamus

//images are organized in the following order for each brain sections: GFAP, merged, CD31

input="(insert mapping to image folder here)";

images = getFileList(input);

images = Array.sort(images);

GFAPmin = (insert threshold min for GFAP here)

GFAPmax = (insert threshold max for GFAP here)

//j=GFAP image

//k=ROI hypothalamus, ROIset already open and organized

for (i=2; i<images.length; i+=3) {

j=i-2;

if (j==0){

k=0;

} else {

k=j/3;

}

//measure GFAP area within cortical or hippocampal ROI, cortex then hippocampus

open(input+images[j]);

run("8-bit");

setThreshold(GFAPmin, GFAPmax);

run("Convert to Mask");

roiManager("Select", k);

run("Analyze Particles...", "clear summarize");

run("Close All");

}