Additional File 1:  MRI settings per scanner type

| MRI sequence | Settings |  |  |
| --- | --- | --- | --- |
|  | Philips 3T | GE 3T | Siemens 3T |
| 3DT1 | Sagittal 3D turbo field echo sequence (1.00 mm isotropic voxels, TR = 7.9 ms, TE = 4.5 ms, FA = 8 degrees) | Sagittal 3D FSPGR (1.00 mm isotropic voxels, TR = 8.2 ms, TE = 3.2 ms / TI = 450 ms, FA = 12 degrees) | 3D MPRAGE (1.00 mm x 1.00 mm x 1.00 mm voxels, TR = 2300 ms, TE = 3 ms) |
| 3D Fluid-attenuated inversion recovery | 3D sagittal VISTA sequence (1.12 mm isotropic voxels, TR = 4800 ms, TE = 279 ms, inversion time = 1650 ms) | 3D sagittal CUBE sequence (1.2 mm isotropic voxels, TR = 8000 ms, TE = 130 ms, inversion time = 2340 ms) | 3D sagittal SPACE sequence (1.2 mm isptropic voxels, TR = 5000 ms, TE = 387 ms ms, inversion time = 2340 ms) |
| 2D PD-T2: | Oblique axial orientation, 45-48 slices of 3mm thickness (TR = 3925 ms, TE = 19 and 100 ms) | Oblique axial orientation, 45-48 slices of 3mm thickness (TR = 8600 ms, TE = 20 and 112 ms) | Oblique axial orientation, 45-48 slices of 3mm thickness (TR = 4730 ms, TE = 20 ms) |

TR: repetition time; TE: echo time; FA: flip angle.