Supplementary Online Content

eMethods. Detailed Methods

eTable 1. Characteristics of macular retinal sublayer thickness using ETDRS grid in CN, MCI, and ADD.

eTable 2. Repeatability results of full-layer retinal OCT-ISCF parameters.

eTable 3. Comparison of retinal OCT-ISCF among three groups.

eFigure 1. Changes of OCT-ISCF before and after cataract surgery. (A-E) stands for CON, ASM, COR, HOM and DIS, respectively (Pre: Preoperative; Post: postoperative).

eFigure 2. OCT-ISCF parameter change pattern of speckle noise signal in 5 normal subjects. (A-E) The results of CON, ASM, COR, HOM and DIS, respectively (CG: control group; TG: test group).

eReferences

This supplementary material has been provided by the authors to give readers additional information about their work.

eMethods.

Neurological examinations

Cognitive function of all participants was assessed by experienced professionals using neuropsychological batteries, such as Montreal Cognitive Assessment (MoCA) and Clinical Dementia Rating (CDR). MoCA is a widely used and rapid assessment method of cognitive function, but it often neglects to assess a patient's daily executive function.1 In contrast, CDR integrates patients' cognitive and daily executive function, and not just based on the patient's psychological performance, including six dimensions of memory, orientation, judgment and ability to solve problems, social affairs, family life and hobbies, and personal care.2 Therefore, it is reliable to use CDR score to classify different stages of AD, including MCI, while MoCA is suitable as a tool to quickly evaluate the impairment degree of cognitive function. All patients underwent magnetic resonance imaging (MRI) and Aβ-PET for differential diagnosis with other neurodegeneration diseases. The brain Aβ plaque burden was quantified by the standardised uptake value ratio (SUVr), which is the region of interest (ROI) of standardised uptake value normalised by the average uptake in the reference areas.3 In Aβ-PET, cerebellar gray matter, whole cerebellum, pons, and white matter are commonly used as reference areas.4,5 In this study, PET images of all subjects were processed using the AIBL (The Australian Imaging, Biomarker and Lifestyle) PET Computational Analysis (CapAIBL) platform. We chose cerebellum as the reference area, while different cerebral cortex regions and hippocampus were chosen as the ROI. In general, the cutoff value of SUVr in ROI sets 1.43 to distinguish negative and positive results.6

Routine ophthalmologic examinations

Two eyes of all participants underwent routine ophthalmic examinations by an experienced performer, including best-corrected visual acuity (BCVA) measurement, slit lamp microscopy, intraocular pressure (IOP) measurement, and fundus photography. Firstly, autorefraction (RM-800, Topcon Inc., Tokyo, Japan) was used to obtain the primary refractive condition, and then BCVA and spherical equivalent (SE) were obtained by subjective refraction using standard logarithmic visual acuity chart (GB11533, wenzhou xingkang medical tec. Co., Ltd, Wenzhou, China). Secondly, IOP was averaged over three measurements using a non-contact tonometer (TX-20, Canon Inc., Tokyo, Japan). Next, the ocular anterior segment was examined using the slit lamp microscopy (SLM-8E, ChongQingKangHuaRuiMing S&T Co., Ltd, Chongqing, China). Finally, fundus color images were taken using a retinal digital camera (CR-2AF, Canon Inc., Tokyo, Japan). Ocular conditions met the inclusion criteria to be included in the study.

Retinal OCT imaging and analysis

This study adopted 200 KHz sweep source OCT device (VG 200S, SVision Imaging Inc., Henan, China) to acquire retinal OCT images. The device has a light source wavelength of 1050nm with scan speed of 200,000 A-scans/second. The device is equipped with an eye tracking utility based on an integrated confocal scanning laser ophthalmoscope to eliminate eye movement artifacts. The axial resolution of the device is 5 μm, and the scan depth is 3 mm. We used a radial scan protocol centered on the macular fovea with scan width of 12 mm, scan lines of 18 lines, and repeat number of 32. After retinal OCT imaging for two eyes, OCT images from the right eye were prioritised for inclusion in subsequent image analysis. If the image quality of the right eye is poor (image quality index less than 7 or image discontinuity), the left eye will be included for subsequent image analysis.

Analysis of retinal OCT-ISCF

Suppose the image size is N×M, grey scale level sets k, and the spatial coordinate position of two points are (x, y) and (x+a, y+b) in the image. These two points form a pair whose grey scale association condition can be expressed as (g1, g2). If the point (x, y) moves across the whole image, grey scale association condition of the pair will vary (Figure 1-). Thus, the frequency of grey scale association condition (g1, g2) was calculated as f (g1, g2), and then was normalized into the occurrence probability P (g1, g2). P (g1, g2) is so called the gray level co-occurrence matrix (GLCM), and satisfies the following formula:

Pg1,g2=f(g1,g2)R, R=NM-1, θ=0N-1M, θ=90N-1M-1, θ=45 or 135         （1）

where θ indicates the movement direction, 0 and 90 indicate horizontal and vertical movement, respectively, and 45 and 135 degrees indicate oblique scanning.

There are several eigenvalues used to characterize the GLCM, which is called ISCF. The ISCF contain angular second order moment (ASM), contrast (CON), correlation (COR), homogeneity (HOM) and dissimilarity (DIS).7 ASM reflects the uniformity of image gray distribution and texture coarseness, which can be represented by the following formula:

ASM=i=1kj=1k(Pi,j)2                   （2）

CON reflects the sharpness of the image and the depth of the texture furrows, which meets the following formula:

CON=n=0k-1n2i-j=nP(i,j)                （3）

COR reflects the consistency of the texture and satisfies the following formula:

COR=i=1kj=1kijPi,j-uiujSiSj                  （4）

ui=i=1kj=1kiP(i,j)                     （5）

uj=i=1kj=1kjP(i,j)                     （6）

Si2=i=1kj=1kP(i,j)(i-ui)2                  （7）

Sj2=i=1kj=1kP(i,j)(j-uj)2                  （8）

HOM reflects the local gray level uniformity of the image and satisfies the following formula:

HOM=i=1kj=1kP(i,j)1+i-j                     （9）

DIS has a strong positive linear relationship with COR, and meets the following formula:

DIS=i=1kj=1kP(i,j)i-j                    （10）

Using the retinal sublayers segmentation algorithm, we can obtain the ISCF of each retinal sublayers. Then, we averaged the ISCF of all 18 retinal OCT images of each retinal sublayers from different scan angles for each eye.

eTable 1. Characteristics of macular retinal sublayer thickness using ETDRS grid in CN, MCI, and ADD.

| ETDRS quadrant | CN | MCI | ADD | P Value |
| --- | --- | --- | --- | --- |
| RNFL thickness, µm |  |  |  |  |
| Central 0-1mm | 8.09 (2.57) | 8.05 (3.49) | 8.09 (3.16) | 0.999 |
| Inner superior | 30.27 (2.67) | 30.48 (1.87) | 29.71 (2.33) | 0.413 |
| Inner nasal | 25.43 (3.12) | 25.97 (2.03) | 25.64 (2.86) | 0.815 |
| Inner inferior | 31.01 (2.77) | 31.46 (2.01) c | 29.66 (2.73) b | 0.022* |
| Inner temporal | 25.17 (2.16) | 25.12 (2.27) | 25.09 (2.10) | 0.993 |
| Outer superior | 39.67 (3.72) | 40.04 (3.18) | 38.94 (4.24) | 0.534 |
| Outer nasal | 47.99 (6.39) | 49.72 (6.06) | 47.08 (5.01) | 0.219 |
| Outer inferior | 41.63 (4.83) c | 41.49 (3.75) c | 39.14 (3.93) a, b | 0.034* |
| Outer temporal | 20.78 (1.88) | 20.75 (1.82) | 21.01 (1.62) | 0.822 |
| GCIPL thickness, µm |  |  |  |  |
| Central 0-1mm | 13.80 (3.59) | 15.89 (6.10) | 16.97 (7.86) | 0.221 |
| Inner superior | 77.77 (7.74) | 76.52 (5.72) | 77.82 (7.31) | 0.766 |
| Inner nasal | 76.71 (8.75) | 75.76 (6.22) | 76.07 (9.01) | 0.931 |
| Inner inferior | 77.71 (7.96) | 75.88 (4.42) | 76.38 (7.24) | 0.665 |
| Inner temporal | 72.75 (7.80) | 70.74 (3.89) | 71.00 (6.42) | 0.514 |
| Outer superior | 59.12 (4.45) | 57.90 (4.55) | 59.57 (5.68) | 0.470 |
| Outer nasal | 66.45 (5.48) | 64.21 (4.06) | 66.52 (5.83) | 0.232 |
| Outer inferior | 56.40 (5.07) | 55.31 (3.71) | 57.55 (5.66) | 0.248 |
| Outer temporal | 62.01 (6.35) | 61.49 (3.64) | 62.03 (5.95) | 0.927 |
| INL thickness, µm |  |  |  |  |
| Central 0-1mm | 23.21 (2.91) | 24.68 (3.13) | 25.09 (3.20) | 0.092 |
| Inner superior | 45.43 (3.09) | 46.26 (2.99) | 45.35 (2.99) | 0.499 |
| Inner nasal | 44.48 (4.15) | 45.94 (3.67) | 45.4 (3.94) | 0.481 |
| Inner inferior | 45.27 (3.33) | 46.15 (2.80) | 45.15 (2.67) | 0.408 |
| Inner temporal | 43.27 (3.08) | 44.04 (3.30) | 43.49 (3.37) | 0.727 |
| Outer superior | 40.34 (3.37) | 40.43 (3.50) | 40.93 (3.18) | 0.759 |
| Outer nasal | 44.39 (2.67) | 44.44 (2.93) | 44.51 (2.74) | 0.986 |
| Outer inferior | 38.7 (3.55) | 38.93 (3.35) | 39.14 (3.26) | 0.890 |
| Outer temporal | 40.08 (3.53) | 41.00 (2.52) | 40.78 (3.26) | 0.613 |
| OPL thickness, µm |  |  |  |  |
| Central 0-1mm | 15.45 (2.66) | 16.17 (3.07) | 15.64 (3.23) | 0.717 |
| Inner superior | 23.85 (2.79) | 23.62 (2.21) | 22.86 (2.67) | 0.305 |
| Inner nasal | 24.50 (4.42) | 24.91 (3.84) | 25.71 (4.54) | 0.556 |
| Inner inferior | 23.90 (2.72) | 24.00 (2.45) | 24.19 (3.15) | 0.925 |
| Inner temporal | 22.96 (2.47) | 22.28 (2.17) | 22.13 (1.90) | 0.356 |
| Outer superior | 24.87 (1.80) | 25.40 (1.52) c | 24.09 (1.55) b | 0.009* |
| Outer nasal | 24.77 (2.77) | 25.76 (2.99) | 26.22 (3.50) | 0.262 |
| Outer inferior | 25.14 (1.79) | 25.69 (1.61) | 25.33 (1.83) | 0.582 |
| Outer temporal | 25.14 (2.09) | 25.88 (1.45) c | 24.72 (1.51) b | 0.035* |
| ETDRS quadrant | CN | MCI | ADD | P Value |
| ONL thickness, µm |  |  |  |  |
| Central 0-1mm | 99.41 (8.97) | 99.32 (8.92) | 99.84 (9.97) | 0.974 |
| Inner superior | 79.10 (7.29) | 78.62 (5.51) | 79.22 (6.79) | 0.943 |
| Inner nasal | 81.19 (7.47) | 79.73 (6.53) | 79.20 (10.58) | 0.720 |
| Inner inferior | 76.16 (8.56) | 76.10 (5.28) | 76.66 (8.02) | 0.949 |
| Inner temporal | 80.58 (7.47) | 81.25 (5.51) | 81.98 (6.21) | 0.718 |
| Outer superior | 60.91 (6.05) | 59.13 (5.32) | 60.49 (5.57) | 0.547 |
| Outer temporal | 57.97 (7.02) | 57.63 (4.95) | 59.11 (5.60) | 0.582 |
| Outer inferior | 52.43 (6.84) | 51.91 (4.16) | 53.18 (5.10) | 0.659 |
| Outer nasal | 59.44 (6.93) | 56.53 (5.94) | 57.42 (7.24) | 0.371 |
| MEZ thickness, µm |  |  |  |  |
| Central 0-1mm | 23.81 (1.53) | 23.16 (1.45) | 23.10 (1.61) | 0.229 |
| Inner superior | 22.73 (2.09) | 21.95 (0.97) | 21.78 (1.19) | 0.051 |
| Inner nasal | 22.84 (2.05) | 22.55 (1.05) | 22.29 (1.57) | 0.451 |
| Inner inferior | 22.41 (1.44) | 21.86 (0.95) | 21.77 (0.99) | 0.108 |
| Inner temporal | 22.85 (1.96) c | 22.22 (1.63) | 21.69 (1.29) a | 0.029* |
| Outer superior | 20.95 (1.58) | 20.43 (0.96) | 20.55 (1.09) | 0.330 |
| Outer nasal | 21.50 (1.29) | 20.81 (1.03) | 20.94 (1.31) | 0.158 |
| Outer inferior | 21.18 (0.96) c | 20.6 (1.10) | 20.52 (1.03) a | 0.048* |
| Outer temporal | 21.27 (1.40) | 20.67 (1.25) | 20.60 (1.05) | 0.116 |
| OS thickness, µm |  |  |  |  |
| Central 0-1mm | 44.08 (4.33) | 42.80 (3.61) | 43.01 (4.38) | 0.558 |
| Inner superior | 37.06 (3.47) | 37.65 (3.26) | 37.45 (2.97) | 0.827 |
| Inner nasal | 37.90 (3.61) | 38.30 (2.81) | 38.31 (3.28) | 0.889 |
| Inner inferior | 37.37 (3.36) | 37.57 (2.56) | 37.22 (2.81) | 0.900 |
| Inner temporal | 36.82 (3.76) | 37.15 (2.45) | 36.86 (2.78) | 0.921 |
| Outer superior | 36.53 (3.06) | 36.21 (2.90) | 36.06 (2.98) | 0.844 |
| Outer nasal | 35.69 (3.23) | 35.04 (2.88) | 35.12 (3.04) | 0.741 |
| Outer inferior | 35.78 (2.62) | 34.94 (2.63) | 35.28 (2.66) | 0.590 |
| Outer temporal | 36.41 (2.61) | 35.66 (2.65) | 36.38 (2.43) | 0.511 |
| RPE thickness, µm |  |  |  |  |
| Central 0-1mm | 23.55 (1.41) | 22.99 (1.93) | 23.32 (1.30) | 0.480 |
| Inner superior | 21.99 (1.97) | 21.03 (1.77) | 21.20 (1.36) | 0.129 |
| Inner nasal | 22.16 (1.72) | 21.58 (1.14) | 21.71 (1.51) | 0.415 |
| Inner inferior | 20.39 (1.99) | 20.05 (1.25) | 20.50 (1.24) | 0.499 |
| Inner temporal | 21.87 (2.18) | 21.38 (2.17) | 21.78 (1.64) | 0.444# |
| Outer superior | 21.65 (2.82) | 21.23 (2.87) | 20.76 (1.70) | 0.376 |
| Outer nasal | 21.21 (2.07) | 20.68 (1.64) | 20.62 (1.50) | 0.432 |
| Outer inferior | 20.56 (1.81) | 20.27 (2.16) | 19.87 (1.43) | 0.338 |
| Outer temporal | 21.45 (2.58) | 21.30 (2.65) | 20.86 (1.59) | 0.549 |

Abbreviations: ADD, Alzheimer's disease-derived dementia; CN, cognitively normal; ETDRS, Early Treatment Diabetic Retinopathy Study; GCIPL, ganglion cells and inner plexiform layer, INL, inner nuclear layer; MCI, mild cognitive impairment; MEZ, myoid and ellipsoid zone; ONL, outer nuclear layer; OPL, outer plexiform layer; OS, outer segment of photoreceptor; RNFL, retinal nerve fiber layer; RPE, retinal pigment epithelium; a, Statistical difference with CN group; b, Statistical difference with MCI group; c, Statistical difference with ADD group; #, Kruskal-Wallis non-parametric test; *, P＜0.05.

eTable 2. Repeatability results of full-layer retinal OCT-ISCF parameters.

| Parameters | First time | Second time | ICC |
| --- | --- | --- | --- |
| CON | 711.57 (55.56) | 708.45 (53.83) | 0.819 |
| ASM | 1.8e-4 (9.0e-6) | 1.8e-4 (1.3e-5) | 0.866 |
| COR | 7586.49 (472.44) | 7582.89 (568.88) | 0.935 |
| HOM | 9.0e-2 (4.0e-3) | 9.0e-2 (5.0e-3) | 0.878 |
| DIS | 14.32 (0.56) | 14.33 (0.67) | 0.871 |

Abbreviations: ASM, angular second moment; CON, contrast; COR, correlation; DIS, dissimilarity; HOM, homogeneity; ICC, intraclass correlation efficient.

eTable 3. Comparison of retinal OCT-ISCF among three groups.

| Retinal sublayers | OCT-ISCF | CN | MCI | ADD | P Value |
| --- | --- | --- | --- | --- | --- |
| RNFL | CON | 5067.55 (685.91) | 4847.33 (957.99) | 4951.13 (853.70) | 0.710 |
|  | ASM | 1.7e-3 (1.2e-4) | 1.8e-3 (2.2e-4) | 1.8e-3 (2.1e-4) | 0.125 |
|  | COR | 9569.87 (690.92) | 9235.14 (1034.07) | 9132.28 (1040.18) | 0.272 |
|  | HOM | 0.45 (0.03) | 0.47 (0.03) | 0.48 (0.04) | 0.071 |
|  | DIS | 61.49 (4.13) | 60.00 (5.35) | 60.04 (5.49) | 0.561 |
| GCIPL | CON | 3284.16 (570.23) | 3164.06 (520.6) | 3155.59 (568.69) | 0.691 |
|  | ASM | 2.5e-3 (2.9e-4) c | 2.6e-3 (3.2e-4) | 2.7e-3 (3.3e-4) a | 0.023* |
|  | COR | 7077.51 (805.9) c | 6746.76 (925.08) | 6447.67 (785.60) a | 0.026* |
|  | HOM | 0.52 (0.04) | 0.54 (0.04) | 0.55 (0.04) | 0.064 |
|  | DIS | 48.89 (4.22) | 47.78 (3.58) | 47.09 (4.45) | 0.305 |
| INL | CON | 3809.06 (639.97) | 3528.43 (639.62) | 3628.08 (644.98) | 0.374 |
|  | ASM | 2.6e-3 (2.6e-4) c | 2.8e-3 (3.1e-4) | 2.9e-3 (3.2e-4) a | 0.048* |
|  | COR | 6478.65 (652.98) | 6176.07 (666.47) | 6078.76 (665.39) | 0.101 |
|  | HOM | 0.52 (0.04) | 0.54 (0.04) | 0.54 (0.04) | 0.080 |
|  | DIS | 55.11 (4.71) | 52.78 (4.55) | 52.97 (4.87) | 0.210 |
| OPL | CON | 5274.05 (868.41) | 4891.76 (843.98) | 5130.30 (868.22) | 0.353 |
|  | ASM | 2.7e-3 (1.8e-4) c | 2.9e-3 (3.1e-4) | 2.9e-3 (3.4e-4) a | 0.006* |
|  | COR | 5847.10 (415.79) c | 5490.84 (571.87) | 5386.75 (619.24) a | 0.017* |
|  | HOM | 0.48 (0.04) c | 0.50 (0.04) | 0.51 (0.04) a | 0.048* |
|  | DIS | 68.58 (5.83) | 65.74 (5.52) | 66.60 (5.99) | 0.287 |
| ONL | CON | 1636.38 (351.9) | 1575.52 (280.63) | 1588.85 (328.55) | 0.816 |
|  | ASM | 3.4e-3 (5.2e-4) | 3.7e-3 (6.9e-4) | 3.7e-3 (5.2e-4) | 0.166 |
|  | COR | 5496.87 (764.5) | 5183.97 (821.34) | 5122.4 (683.19) | 0.191 |
| Retinal sublayers | OCT-ISCF | CN | MCI | ADD | P Value |
| ONL | HOM | 0.62 (0.06) | 0.65 (0.06) | 0.65 (0.06) | 0.210 |
|  | DIS | 36.82 (4.07) | 35.84 (3.40) | 35.63 (3.59) | 0.498 |
| MEZ | CON | 10962.42 (1219.25) | 10487.98 (1570.33) | 10886.81 (1600.8) | 0.528 |
|  | ASM | 1.9e-3 (1.9 -4) | 2.1e-3 (2.8e-4) | 2.1e-3 (2.6e-4) | 0.086 |
|  | COR | 8744.21 (833.52) | 8148.7 (986.25) | 8115.09 (927.08) | 0.053 |
|  | HOM | 0.35 (0.03) | 0.36 (0.03) | 0.36 (0.02) | 0.324 |
|  | DIS | 105.83 (5.68) | 104.09 (7.38) | 105.55 (6.97) | 0.656 |
| OS | CON | 8997.68 (1315.42) | 8806.33 (1091.21) | 8852.48 (1039.7) | 0.850 |
|  | ASM | 1.2e-3 (1.0e-4) b, c | 1.3e-3 (1.8e-4) a | 1.3e-3 (1.6e-4) a | 0.003* |
|  | COR | 13302.27 (1211.54) b, c | 11991.38 (1476.05) a | 11880.40 (1313.21) a | 0.001* |
|  | HOM | 0.36 (0.02) c | 0.37 (0.03) | 0.38 (0.03) a | 0.037* |
|  | DIS | 82.73 (6.19) | 81.00 (5.65) | 80.34 (5.65) | 0.336 |
| RPE | CON | 15095.05 (1437.88) | 15087.07 (1544.06) | 15413.08 (1454.78) | 0.619 |
|  | ASM | 2.2e-3 (2.0e-4) c | 2.3e-3 (2.1e-4) | 2.4e-3 (2.2e-4) a | 0.006* |
|  | COR | 7049.06 (707.24) c | 6718.35 (613.91) | 6473.98 (606.02) a | 0.006* |
|  | HOM | 0.40 (0.02) c | 0.41 (0.02) | 0.42 (0.02) a | 0.096 |
|  | DIS | 107.8 (5.69) | 107.79 (5.88) | 108.16 (5.57) | 0.960 |

Abbreviations: ASM, angular second moment; CON, contrast; COR, correlation; DIS, dissimilarity; GCIPL, ganglion cells and inner plexiform layer; HOM, homogeneity; INL, inner nuclear layer; MEZ, myoid and ellipsoid zone; OCT-ISCF, optical coherence tomography intensity spatial correlation features; ONL, outer nuclear layer; OPL, outer plexiform layer; OS, outer segment of photoreceptor; RNFL, retinal nerve fiber layer; RPE, retinal pigment epithelium; a, Statistical difference with CN group; b, Statistical difference with MCI group; c, Statistical difference with ADD group; *, P＜0.05.

eFigure 1. Changes of OCT-ISCF before and after cataract surgery. (A-E) stands for CON, ASM, COR, HOM and DIS, respectively. (Pre, Preoperative; Post, postoperative).

Abbreviations: ASM, angular second moment; CON, contrast; COR, correlation; DIS, dissimilarity; GCIPL, ganglion cells and inner plexiform layer; HOM, homogeneity; INL, inner nuclear layer; MEZ, myoid and ellipsoid zone; ONL, outer nuclear layer; OPL, outer plexiform layer; OS, outer segment of photoreceptor; RNFL, retinal nerve fiber layer; RPE, retinal pigment epithelium.

eFigure 2. OCT-ISCF parameters change pattern of speckle noise signal in 5 normal subjects. (A-E) The results of CON, ASM, COR, HOM and DIS, respectively. (CG: control group; TG: test group)

Abbreviations: ASM, angular second moment; CON, contrast; COR, correlation; DIS, dissimilarity; HOM, homogeneity.

eReferences

1.	Nasreddine ZS, Phillips NA, Bédirian V, et al. The Montreal Cognitive Assessment, MoCA: a brief screening tool for mild cognitive impairment. J Am Geriatr Soc. 2005;53(4):695-699. https://doi.org/10.1111/j.1532-5415.2005.53221.x

2.	Morris JC. The Clinical Dementia Rating (CDR): current version and scoring rules. Neurology. 1993;43(11):2412-2414. https://doi.org/10.1212/wnl.43.11.2412-a

3.	Landau SM, Ward TJ, Murphy A, et al. Quantification of amyloid beta and tau PET without a structural MRI. Alzheimers Dement. 2023;19(2):444-455. https://doi.org/10.1002/alz.12668

4.	Landau SM, Fero A, Baker SL, et al. Measurement of longitudinal β-amyloid change with 18F-florbetapir PET and standardized uptake value ratios. J Nucl Med. 2015;56(4):567-574. https://doi.org/10.2967/jnumed.114.148981

5.	Iulita MF, Bejanin A, Vilaplana E, et al. Association of biological sex with clinical outcomes and biomarkers of Alzheimer's disease in adults with Down syndrome. Brain Commun. 2023;5(2):fcad074. https://doi.org/10.1093/braincomms/fcad074

6.	Bullich S, Seibyl J, Catafau AM, et al. Optimized classification of (18)F-Florbetaben PET scans as positive and negative using an SUVR quantitative approach and comparison to visual assessment. Neuroimage Clin. 2017;15:325-332. https://doi.org/10.1016/j.nicl.2017.04.025

7.	Kobayashi T, Sundaram D, Nakata K, Tsurui H. Gray-level co-occurrence matrix analysis of several cell types in mouse brain using resolution-enhanced photothermal microscopy. J Biomed Opt. 2017;22(3):36011. https://doi.org/10.1117/1.Jbo.22.3.036011