SUPPLEMENTARY MATERIALS

Supplementary Methods	2

1. Protocols of Aβ PET imaging acquisition	2

2. Protocols of plasma collection for the K-ROAD study	4

Supplementary Tables	5

Supplementary Table 1. Cost-based assessment of pTau217 cutoffs for therapy stratification including Aβ-positive and Aβ-negative participants	5

Supplementary Table 2. Cost-based assessment of pTau217 cutoffs for therapy stratification in Aβ-positive cognitively impaired participants	7

Supplementary Figure	9

Supplementary Figure 1. Study flow chart of participant inclusion and exclusion	9

Supplementary Methods 1. Protocols of Aβ PET imaging acquisition

1.1 K-ROAD

In the K-ROAD study, amyloid PET imaging was conducted using one of two tracers: [18F]florbetaben (FBB), or [18F]flutemetamol (FMM). All scans were acquired in 3D mode using Discovery STe PET/CT scanners across participating sites.

For PiB PET, an average dose of 420 MBq was administered, followed by CT-based attenuation correction and a 30-minute static emission scan capturing 35 slices at 4.25 mm thickness. For FBB and FMM, participants received 311.5 MBq or 197.7 MBq, respectively, and underwent 20-minute dynamic scans beginning 90 minutes after tracer injection. These scans encompassed 47 axial slices with a thickness of 3.3 mm.

All PET images were reconstructed using an ordered-subsets expectation maximization (OSEM) algorithm with 4 iterations and 20 subsets. The reconstructed image matrix was 128 × 128 × 48 voxels, with a voxel size of 2 × 2 × 3.27 mm³.

Preprocessing of FBB and FMM data were processed using SPM8 within Matlab 2014b. PET volumes were co-registered to individual structural MRIs and spatially normalized to MNI-152 standard space. Brain regions were segmented using the automated anatomical labeling (AAL) atlas to enable regional analyses.

1.2 NA-ADNI Cohort

Amyloid PET imaging in the NA-ADNI cohort was performed using [18F]florbetapir. Participants received an intravenous bolus of approximately 370 MBq (10 mCi) of the tracer, followed by PET acquisition beginning 50 to 70 minutes post-injection. Scanning consisted of four consecutive 5-minute frames in 3D dynamic mode. The acquired frames were realigned, averaged, and resampled to a standardized voxel dimension of 1.5 × 1.5 × 1.5 mm³, then smoothed to achieve an 8 mm³ full width at half-maximum (FWHM) resolution.

Each participant’s structural T1-weighted MRI (MPRAGE) was used for anatomical co-registration and cortical surface segmentation using FreeSurfer. Cortical and reference regions were defined in native space. Further technical details regarding PET acquisition protocols are publicly available via the ADNI resource (http://www.loni.ucla.edu/ADNI).

Supplementary Methods 2. Protocols of plasma collection for the K-ROAD study

A total of 8 mL of blood was drawn from each participant and laced in tubes containing 0.5 M ethylenediaminetetraacetic acid. The samples were agitated for 5 min and then centrifuged at 1300×g for 10 min, separating the plasma into 5–10 vials, each containing 0.3 mL. The plasma samples were then stored at -75 ºC until they were analyzed. The procedures were conducted in accordance with the guidelines by the National Biobank of the Republic of Korea for human resource collection and registration.

Supplementary Table 1 Cost-based assessment of pTau217 cutoffs for therapy stratification including Aβ-positive and Aβ-negative participants.

| Cohort | Assay (n) | Strategy | FN (n) | FP (n) | Intermediate (n) | Total Cost (1,000 USD) |
| --- | --- | --- | --- | --- | --- | --- |
| K-ROAD | UGOT-Simoa (n=120) | Single | 8 | 11 | – | 394 |
|  |  | Double | 5 | 6 | 24 | 316 |
|  | Lilly-MSD (n=120) | Single | 16 | 4 | – | 248 |
|  |  | Double | 5 | 6 | 23 | 312 |
| NA-ADNI | Quanterix-Simoa (n=280) | Single | 8 | 63 | – | 1,954 |
|  |  | Double | 6 | 21 | 109 | 1,114 |
|  | C2N ratio (n=171) | Single | 17 | 26 | – | 916 |
|  |  | Double | 9 | 11 | 84 | 738 |
|  | Fujirebio-Lumipulse (n=269) | Single | 6 | 64 | – | 1,968 |
|  |  | Double | 6 | 25 | 56 | 1,022 |
|  | Janssen-Simoa (n=27) | Single | 14 | 31 | – | 1,042 |
|  |  | Double | 6 | 26 | 93 | 1,200 |

Abbreviations: pTau217, plasma phosphorylated tau 217; Aβ+, β-amyloid positive; K-ROAD, Korea–Registries to Overcome Alzheimer’s Disease and Accelerate Dementia; NA-ADNI, North American Alzheimer’s Disease Neuroimaging Initiative; UGOT-Simoa, ALZpath antibody on Simoa platform (University of Gothenburg); Lilly-MSD, customized pTau217 assay on MSD (Lilly Research Labs); Quanterix-Simoa, ALZpath antibody on Simoa platform (Quanterix); C2N ratio, LC–MS/MS–based pTau217/non-pTau217 ratio (C2N Diagnostics); Fujirebio-Lumipulse, pTau217 assay on Lumipulse G platform (Fujirebio); Janssen-Simoa, Janssen-developed pTau217 assay on Simoa platform (Janssen); FN, false negative; FP, false positive; USD, United States dollar.

Supplementary Table 2 Cost-based assessment of pTau217 cutoffs for therapy stratification in Aβ-positive cognitively impaired participants.

| Cohort | Assay (n) | Strategy | FN (n) | FP (n) | Intermediate (n) | Total Cost (1,000 USD) |
| --- | --- | --- | --- | --- | --- | --- |
| K-ROAD | UGOT-Simoa (n=86) | Single | 7 | 3 | – | 146 |
|  |  | Double | 4 | 1 | 20 | 182 |
|  | Lilly-MSD (n=86) | Single | 14 | 2 | – | 172 |
|  |  | Double | 4 | 3 | 15 | 182 |
| NA-ADNI | Quanterix-Simoa (n=63) | Single | 2 | 20 | – | 616 |
|  |  | Double | 2 | 8 | 32 | 384 |
|  | C2N ratio (n=58) | Single | 7 | 11 | – | 386 |
|  |  | Double | 3 | 4 | 28 | 256 |
|  | Fujirebio-Lumipulse (n=59) | Single | 1 | 21 | – | 638 |
|  |  | Double | 1 | 10 | 17 | 376 |
|  | Janssen-Simoa (n=60) | Single | 6 | 11 | – | 378 |
|  |  | Double | 2 | 9 | 24 | 382 |

Abbreviations: pTau217, plasma phosphorylated tau 217; Aβ+, β-amyloid positive; K-ROAD, Korea–Registries to Overcome Alzheimer’s Disease and Accelerate Dementia; NA-ADNI, North American Alzheimer’s Disease Neuroimaging Initiative; UGOT-Simoa, ALZpath antibody on Simoa platform (University of Gothenburg); Lilly-MSD, customized pTau217 assay on MSD (Lilly Research Labs); Quanterix-Simoa, ALZpath antibody on Simoa platform (Quanterix); C2N ratio, LC–MS/MS–based pTau217/non-pTau217 ratio (C2N Diagnostics); Fujirebio-Lumipulse, pTau217 assay on Lumipulse G platform (Fujirebio); Janssen-Simoa, Janssen-developed pTau217 assay on Simoa platform (Janssen); FN, false negative; FP, false positive; USD, United States dollar.

(A) K-ROAD cohort

(B) NA-ADNI cohort

Eligible ADCI participants with

tau PET scans (n = 456)

Eligible ADCI participants with

tau PET scans (n=190)

Exclusion criteria met (n=176)

• Plasma p-tau217 not available
• Outliers excluded (mean±3 SD)

Exclusion criteria met (n=70)

• Plasma p-tau217 not available
• Outliers excluded (mean±3 SD)

Final ADNI participants (n=280)

CU/MCI/DAT (n= 172/81/27)

Final Korean cohort (n=120)

CU/MCI/DAT (n= 26/41/53)

Assay-specific quality control

LOD-imputed values excluded

Assay-specific quality control

LOD-imputed values excluded

- Quanterix-Simoa (n = 280)

- C2N ratio (n = 171)

- Fujirebio Lumipulse (n = 269)

- Janssen-Simoa (n = 277)

- UGOT-Simoa (n = 120)

- Lilly-MSD (n = 120)

Amyloid-β- positive participants

Amyloid-β- positive participants

- Quanterix-Simoa (n = 136)

- C2N ratio (n = 114)

- Fujirebio Lumipulse (n = 132)

- Janssen-Simoa (n = 133)

- UGOT-Simoa (n = 100)

- Lilly-MSD (n = 100)

Supplementary Figure 1. Study flow chart of participant inclusion and exclusion.

Abbreviations: p-tau217, plasma phosphorylated tau at threonine 217; K-ROAD, Korea–Registries to Overcome Dementia and Accelerate Dementia; NA-ADNI, North American Alzheimer’s Disease Neuroimaging Initiative; ADCI, Alzheimer’s disease–related cognitive impairment; CU, cognitively unimpaired; MCI, mild cognitive impairment; DAT, dementia of Alzheimer’s type; tau PET, tau positron emission tomography; LOD, limit of detection; UGOT-Simoa, ALZpath antibody–based p-tau217 assay on the Simoa platform (University of Gothenburg); Lilly-MSD, customized p-tau217 assay on the Meso Scale Discovery platform (Lilly Research Laboratories); Quanterix-Simoa, ALZpath antibody–based p-tau217 assay on the Simoa platform (Quanterix); C2N ratio, LC–MS/MS–based p-tau217/non-p-tau217 ratio (C2N Diagnostics); Fujirebio-Lumipulse, p-tau217 assay on the Lumipulse G platform (Fujirebio); Janssen-Simoa, Janssen-developed p-tau217 assay on the Simoa platform (Janssen).