Supplementary methods

7-AAD-flow cytometry analysis

For 7-AAD viability analyses, cells were grown in growth medium and treated for 24 h with DMSO or Nilotinib. Then, myoblasts were washed 3 times with PBS 1X prior to the addition of cell dissociation buffer (0.025% Trypsin-EDTA (0.5 X) (Life technologies, NY, USA) plus (0.5% BSA, 1 mM EDTA, 1 mM EGTA, pH 7.4). Cells were incubated for 15 minutes until they detached from the plate. To evaluate apoptosis 7-AAD was added (1:500) to the detached-cells (1x106 cells) and incubated for 15 minutes in the dark, prior to analysis by flow cytometry. The analysis was performed on a BD Influx flow cytometer (Becton Dickinson, USA) using 488-excitation laser. For each sample, 20,000 events were collected and analyzed. Gating and the percentage of cells existing within gates was determined using FlowJo 8.7 software (Ashland, OR, USA).

Colorimetric MTT (tetrazolium) assay

MTT (3-(4,5-dimethylthiazol-2-yl)-2,5-diphenyl tetrazolium bromide (#475989; Calbiochem, USA) was dissolved in PBS (5 mg/ml) and filtered. 20 h after Nilotinib treatment, stock MTT solution (10X) was added to all assay wells, and plates were incubated at 37°C for 4 h. Warm lysis buffer (50% N,N-Dimethylformamide + 20% SDS in destilled water) was added to all wells and mixed thoroughly to dissolve the dark blue crystals. After a few minutes at room temperature to ensure that all crystals were dissolved, the plates were read in a Synergy H1 equipment using the software Gen5 (BioTek, USA), using a test wavelength of 570 nm, and a reference wavelength of 630 nm. Plates were normally read within 1 h of adding the lysis solution.