Additional file 1

Section 1. HYPER Iterative

The base of the HYPER Iterative algorithm can be written as the following objective function [1,2]:

f=argmaxf≥0ij-pijfj+icilnjpijfj-jγj×Ufj                                   1

γj=gNEC, snnsj×β                                                                                                                             2

Uf=x,y,z∇f                                                                                                                                     3

Where f is the estimated image. i and j are the indexes of the projection bins and the image pixels, respectively. ci are the measured emission data. pij is the system matrix indicating the counts emitted from the jth image pixel detected by the ith projection bin. γj is a parameter of regularized strength. U is the total variation penalization of the pixels in the neighborhood. snsj is the spatially varied sensitivity profile. NEC presents the noise equivalent counts. g is a function of NEC and snsj. β is a factor representing penalty strength which is normalized to a range of 0 to 1.0.

[1] Sui X, Tan H, Yu H, et al. Exploration of the total-body PET/CT reconstruction protocol with ultra-low 18F-FDG activity over a wide range of patient body mass indices. EJNMMI Phys. 2022;9(1):17.

[2] Xu L, Li RS, Wu RZ, et al. Small lesion depiction and quantification accuracy of oncological 18F-FDG PET/CT with small voxel and Bayesian penalized likelihood reconstruction. EJNMMI Phys. 2022;9(1):23.

Section 2.

The percent contrast recovery (CR) and background variability (BV) for each sphere were measured according to NEMA NU2-2012 protocol. The region of interest (ROI) was draw at the center of each hot sphere with a matched diameter to the sphere. The background ROI was drawn in the peripheral area of the phantom background at the central slice of the spheres, and ±1 cm and ±2 cm aside. A total of 60 background ROIs of each size, 12 ROIs on each of five slices shall thus be drawn. The locations of all ROIs shall be fixed between successive measurements. The average counts inn each background ROI shall be recorded. The percent contrast CRH, j for each hot sphere j and the percent background variability BVj are calculated by equation (4), (5) and (6):

CRH,j=CH,jCB,j-1aHaB-1×100 %                                                          4

BVj=SDjCB,j×100 %                                                                                                   5

SDj=k=1KCB,j,k-CB,j2K-1                                                                                       6

Where CRH,j is the percent CR of the sphere j, CH,j and CB,j are the average counts within a ROI on each sphere j and corresponding background ROIs. aH and aB are the activity concentration in the sphere and the background of the phantom, BVj is the percent BV measured by background ROIs from sphere j, SDj is the standard deviation of the background ROI counts for sphere j. K is equal to 60.