Supplementary Table S1. The detailed parameters of brain MR scanning

| MR sequences | TE (ms) | TR (ms) | Thickness/Space (mm) | FOV (cm) | Matrix | Nex | Slice |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ax T2 FSE | Minimum | 5000 | 5.0/1.0 | 24×24 | 288×288 | 1 | 22 |
| Ax T2 Flair | 140 | 6200 | 5.0/1.0 | 24×24 | 256×256 | 1 | 22 |
| 3D T1 GRE | Minimum | 2100 | 1.0/0.0 | 25×25 | 256×256 | 1 | 188 |
| Ax DWI b=1000 | Minimum | 5000 | 5.0/1.0 | 24×24 | 128×128 | 2 | 44 |
| 3D ASL | 10.7 | 4643.0 | 4.0/0.0 | 24×24 | 512×8 | 1 | 24 |
| Multivoxel 1H-MRS | 144 | 1000 | 10.0/0.0 | 24×24 | 18×18 | 1 | 1 |

The water peak and adjust the transmitter gain (TG) performed by a vendor-provided auto-prescan were used to calibrate the resonance frequency. The water suppression was performed by point-resolved spectroscopy (PRESS) sequence with CHESS in two-dimensional magnetic resonance spectroscopic imaging (2D MRSI). The volume of interest (VOI) was selected using the PRESS sequence and six outer volume saturation bands to suppress signals from subcutaneous lipids. Signal intensities were calculated by the sum of each peak with the metabolite peaks assigned to the following chemical shift ranges: Cr at 3.12–2.97 ppm, Cho at 3.34–3.19 ppm and NAA at 2.13–1.96 ppm.

Abbreviations: TE, Echo Time, TR, Repetition Time, DWI, Diffusion-Weighted Imaging, FOV, Field of View, FSE, Fast Spin Echo, GRE, Gradient Echo, ASL, Arterial Spin Labeling, 1H-MRS, 1H-Magnetic Resonance Spectroscopy