# Additional file 1 Principle of the three ELISA tests to detect bovine PrPSc and PrPres

The term PrPres is used when in the commercial test a digestion step with proteinase K is included to effectively breakdown PrPC. The removal of the N-terminal region of (the PrP units in) PrPSc is then a consequence, such that epitopes for e.g. SAF34, P4 and 12B2 are variably absent depending on the stringency of the digestion conditions. Epitopes in the remaining C-terminal part remain intact. When a digestion with proteinase K under stringent conditions is performed, N-terminal epitopes are removed for e.g. SAF34 (62QPHGGGW92) and 12B2 (101WGQGG105), but not for 9A2 (110WNK112) and other antibodies with specificity to the more C-terminal region of PrP downstream the 9A2 epitope.

The TeSeE test will detect presence of PrPres after subsequent steps of immobilisation by precoated antibody SAF34, denaturation, peroxidase-conjugated antibody Bar224 binding, incubation with substrate and spectrophotometry. Digestion with proteinase K is an elementary step which is performed under less stringent digestion conditions.

The HerdCheck test captures fibrillar proteins such as PrPSc and PrPres in prions using 96 wells titre plates with immobilised Seprion ligand on the wall. After binding of prion material to the ligand a protein denaturation treatment follows to allow access to PrP-specific monoclonal antibodies, incubation with peroxidase conjugated anti-mouse IgG, substrate addition and final spectrophotometric detection. The specificity of the PrP specific antibody(ies) used is (are) not disclosed. No proteinase K treatment is applied.

The CediTect test measures unfolding of PrPres aggregates after adsorption to 96 well polyvinylidenefluoride filter-plates (PVDF) that can be washed by filtration for subsequent steps. Two wells with the same sample are differently treated to establish a measure for protein unfolding by comparing non-denatured (n) using PBS and denatured (d) state using 4 M guanidine-thiocyanate. Further subsequent steps with intermediate washing are incubations respectively with primary antibody (94B4 or 9A2), rabbit anti-mouse IgG conjugated to alkaline phosphatase, and Luminol. Chemoluminescence detection was carried out with a Centro LB 960 Luminometer (Berthold, Germany). After denaturation PrPres epitopes become highly accessible for PrP-specific primary antibodies yielding high d/n ratios in BSE positive cases.