Exploring the potential for biomethane production by willow pyrolysis using life cycle assessment methodology

Elham Ahmadi Moghaddam, Niclas Ericsson, Per-Anders Hansson and Åke Nordberg

Department of Energy and Technology, Swedish University of Agricultural Sciences (SLU), P.O. Box 7032, 750 07, Uppsala, Sweden

Supplementary Material

Table S1. Agricultural operations and inputs in maize cultivation

| Annual operations | Addition | Per ha & yr |  |
| --- | --- | --- | --- |
| Planting | Seeds | 25 kg1 |  |
| Plowing |  |  |  |
| Weed control |  |  |  |
|  | Herbicide 2 | 0.8 kg of a.i.3 |  |
|  | Insecticide | 0.1 kg of a.i. |  |
| Fertilization4 |  |  |  |
|  | Nitrogen | 20 kg |  |
|  | Phosphorus | 3 kg |  |
|  | Potassium | 47 kg |  |
| Digestate | Digestate | 26 t |  |
| Total fuel consumption machinery | Diesel | - T |  |

1 [1].

2 [2].

3Active ingredient.

4 Digestate supplied 129, 23, and 72 kg/ha and yr of N, P, and K, respectively, mineral fertilizer supplied 20, 3, and 47 kg/ha and yr, respectively [3].

Table S2. Yearly agricultural operations and inputs in willow cultivation1

| Year | Operation | Addition | Per ha & yr |  |
| --- | --- | --- | --- | --- |
| Preparation |  |  |  |  |
| 0, 22, (44) | Weed control2 | Roundup | 5 L | Applied a.i.3 = 2.4 kg/ha |
| 0, 22 | Plowing |  |  |  |
| 1, 23 | Planting4 |  | 13 000 plants |  |
| 1, 23 | Weed control2 | Cougar | 1 L | Applied a.i3. = 0.6 kg/ha |
| 1, 23 | Harrowing |  |  | Mechanical weed control |
| Cutting Cycle 1 |  |  |  |  |
| 2 | Fertilization4 | Phosphorus | 22 kg |  |
| 2 | Fertilization4 | Potassium | 73 kg |  |
| 3 | Harvest |  |  | Yield = 20 000 kg DM/cycle 1 |
| Cutting Cycle 2-7 |  |  |  |  |
| 4,7,10,13,16,19 | Fertilization4 | Nitrogen | 48 kg |  |
| 5,8,11,14,17, 20 | Fertilization | Nitrogen | 76 kg |  |
| 6,9,12,15,18,21 | Fertilization | Nitrogen | 67 kg |  |
| 5,8,11,14,17, 20 | Fertilization | Phosphorus | 22 kg |  |
|  |  | Potassium | 73 kg |  |
| 6,9,12,15,18,21 | Harvest transport |  |  | 2 km |
| 6,9,12,15,18,21 | Harvest |  |  | 24 km |
|  |  |  |  | Yield = 30 000 kg DM/cycle 2-7 |
| 22, 44 | Stump removal |  |  |  |
| Annual input | Biochar application5 |  | 1 t |  |
| Annual operation | Biochar transport6 |  |  | 24 km |
| Annual operation | Biochar spreading6 |  |  |  |

1Rotation of 22 years.

2[4]

3Active ingredient.

4[5].

5[6].

6[7].

Table S3. Primary energy (PE)1 use and greenhouse gas (GHG) emissions from production of different inputs to agricultural operations

|  | PE use | Units | GHG emissions | Units |
| --- | --- | --- | --- | --- |
| Maize seed | 243 | MJ/ha | 20.0 | kg CO2-eq./ha |
| Willow seedlings | 809 | MJ/ha | 735.0 | kg CO2-eq./ha |
| Chemicals |  |  |  |  |
| Roundup | 96 | MJ/lit | 5.3 | kg CO2-eq./kg of a.i |
| Cougar | 119 | MJ/lit | 5.0 | kg CO2-eq./kg of a.i |
| Maize herbicide | 356 | MJ/ kg of a.i. | 25.0 | kg CO2-eq./kg of a.i. |
| Maize insecticide | 358 | MJ/ kg of a.i. | 26.0 | kg CO2-eq./kg of a.i. |
| Fertilizers2 |  |  |  |  |
| Nitrogen | 35.0 | MJ/kg | 8.0 | kg CO2-eq./kg |
| Phosphorus | 8.0 | MJ/kg | 3.0 | kg CO2-eq./kg |
| Potassium | 5.0 | MJ/kg | 0.5 | kg CO2-eq./kg |
| Other |  |  |  |  |
| Fuel consumption (diesel)3,4 | 48.9 | MJ/lit3 |  |  |

1Defined as the energy needed to produce one unit of useful energy, like electricity or heat. Included in the PE are extraction of fuel, transportation, and conversion, transmission, and distribution losses. By converting useful energy to PE, it is possible to compare different energy carriers to each other [8].

2EU average [9].

3Diesel consumption in plowing, harrowing, pesticide application, harvest, field transport, stump removal, and by-product spreading in the field.

4[10].

Figure S1. Energy and mass inputs/outputs to the biomass conversion plant and gas upgrading in the willow scenario.

Table S4. Energy demand/supply of the pyrolysis plant in the willow scenario, calculation based on [11]

|  | Demand (MW) |  | Supply (MW) |
| --- | --- | --- | --- |
| Drying | - 2.0 | Cooling – after reformer | +1.2 |
| Pyrolysis | - 1.6 | Methanation | +0.9 |
| Grinding | - 0.2 | Cooling-final upgrading | +1.1 |
| Pyrolysis gas circulation | - 0.1 | Heat from combustion | +2.9 |
| Compression | - 0.6 | Heat recovery | +1.0 |
| Upgrading | - 0.16 |  |  |
| Total energy demand | 4 MW |  |  |
| Total energy supply | 7 MW |  |  |
| Net energy balance | 3 MW |  |  |
| Exported heat | 1 MW |  |  |
| Lost heat | 2 MW |  |  |

Appendix S5.

- The ICBM model and parameters:

The model was applied to calculate the SOC content according to carbon inputs and mineralization rates. The ICBM model has two carbon pools, ‘young’ (Y) and ‘old’ (O). All carbon from aboveground residues (ag), belowground residues (bg), and digestate application enters the young carbon pool (Y). The easily degradable C is released as CO2 by mineralization determined by the humification coefficient (h) and the resistant fraction is humified and stored in the old carbon pool (Eq. SM1). Dead biomass is not included as a separate pool. Live biomass is assumed to be harvested and taken away [12].

h*ky*re*y

Yss=iKy*re	                    	                Oss=hiKo*re               (Eq. SM1)

Parameter and variable descriptions:

Y: Young soil organic carbon pool.

O: Old soil organic carbon pool.

suffix ag: refers to the aboveground fraction of the young pool.

suffix bg: refers to the belowground fraction of the young pool.

h: Humification coefficient. Determines how much of the C broken down in a time step that ends up in the old pool and how much is returned to the atmosphere as CO2.

kY: Decay constant of young pool.

kO: Decay constant of old pool.

re: Decomposer activity factor. Affects rate of decomposition. Dependent on external factors such as soil temperature and moisture content.

- Table S5. Input values used in ICBM for modeling soil organic carbon (SOC) pools in the reference and willow scenarios, value calculated based on [13].

|  | Reference scenario1 | Willow scenario2 |  |  |
| --- | --- | --- | --- | --- |
| Initial SOC level3 (Mg/ha) | 130 | 130 |  |  |
| Input ag(Mg/ha) | 3.7 |  | 1st cycle | 2-7nd cycle |
|  |  | 1st year | 0.56 | 1.05 |
|  |  | 2nd year | 1.08 | 1.67 |
|  |  | 3rd year | 0.85 | 1.46 |
| Input bg (Mg/ha) | 1.1 |  |  |  |
|  |  | 1st year | 1.35 | 2.32 |
|  |  | 2nd year | 2.55 | 2.72 |
|  |  | 3rd year | 2.20 | 3.26 |
| Input digestate (Mg/ha) | 0.15 | - |  |  |
| ky | 0.8 | 0.8 |  |  |
| ko | 0.006 | 0.006 |  |  |
| hag | 0.1 | 0.1 |  |  |
| hbg | 0.3 | 0.3 |  |  |
| hdigestate | 0.3 | - |  |  |
| R | 1 | 1 |  |  |

1Inputs from aboveground residues (ag), belowground residues (bg) and digestate for maize (annual crop) were used to calculate steady state SOC level prior to year 0 and the reference scenario in the 44-year study period.

2Inputs from aboveground residues (ag) and belowground residues (bg) for willow (perennial) were used to calculate the yearly changes in SOC during the study period

3The initial SOC level refers to the first year of the study period (year 0)

Appendix S6. Biochar modeling in willow scenario (a)

The carbon fraction (C t) remaining in the soil from an initial biochar carbon application (C0) was modeled using eq. SM2 [14]:

Ct = C0 -( C0 expbm+1tm+1)                                                                  (Eq. SM2)

where b and m describe the degradation rate as a function of biochar properties and time. Parameters for coarse pine wood pyrolysis at 525°C were used for willow [14].

# References

1. Kim S, Dale BE, Keck P (2014) Energy requirements and greenhouse gas emissions of maize production in the USA. Bioenergy Res 7(2):753–764

2. Grassini P, Cassman KG (2012) High-yield maize with large net energy yield and small global warming intensity. Proc Natl Acad Sci 109(4):1074–1079

3. Gissén C, Prade T, Kreuger E, Nges IA, Rosenqvist H, Svensson SE et al (2014) Comparing energy crops for biogas production–yields, energy input and costs in cultivation using digestate and mineral fertilisation. Biomass Bioenergy 64:199–210. https://doi.org/10.1016/j.biombioe.2014.03.061

4. Nilsson, D., and Bernesson, 2008. Pelletering och brikettering av jordbruksråvaror, Report 001, ISSN 1654-9406, Uppsala 2008

5. Aronsson P, Rosenqvist H (2011) Gödslingsrekommendationer för Salix 2011. Rapport Sveriges lantbruksuniversitet, Uppsala

6. Williams, A.G., Audsley, E., Sandars, D.L., Determining the environmental burdens and resource use in the production of agricultural and horticultural commodities. Main Report. 2006. Cranfield University and Defra, Bedford.

7. Jílek L, Pražan R, Podpěra V, Gerndtová I (2008) The effect of the tractor engine rated power on diesel fuel consumption during material transport. Resource Agric Eng 54:1–8

8. SEA (2006). Effektivare primärenergianvändning – En uppföljning av måluppfyllelse avseende EG-direktivet om effektivare slutanvändning av energi och om energitjänster mellan åren 1991 till 2004. (More efficient primary energy use - A follow-up of goal fulfillment regarding the EC directive on more efficient end use of energy and on energy services between 1991 and 2004). Report ER 2006:32. Swedish Energy Agency

9. Yara, The Fertilizer Industry Handbook (2017). Available via http://yara.com/doc/245620_Fertilizer_Industry_Handbook_2017_with_notes.pdf

10. Ecoinvent, 2017. Ecoinvent Centre, ecoinvent data v3.3. Swiss Centre for Life Cycle Inventories, Dübendorf, Switzerland.

11. Görling M, Larsson M, Alvfors P (2013) Bio-methane via fast pyrolysis of biomass. Appl Energy 112:440–447. https://doi.org/10.1016/j.apenergy.2013.01.002

12. Andrén O, Kätterer T (1997) ICBM: the introductory carbon balance model for exploration of soil carbon balances. Ecol Appl 7(4):1226–1236. https://doi.org/10.1890/1051-0761(1997)007[1226:ITICBM]2.0.CO;2

13.  Ericsson N, (2015) Time-dependent climate impact of short rotation coppice willow–based systems for electricity and heat production, (Doctoral dissertation), Swedish University of Agricultural Sciences (SLU), Uppsala. ISBN 978-91-576-8390

14.  Zimmerman AR (2010) Abiotic and microbial oxidation of laboratory-produced black carbon (biochar). Environ Sci Technol 44:1295–1301