Table S3 . Working conditions and specifications of ICP-AES in this experiment.

| ICP-AES | Properties |
| --- | --- |
| RF power | 0.7-1.8 Kw (1.2-1.3 kW for axial) |
| Auxilary gas flow rate (Ar) | 1.5 |
| Plasma gas flow rate (Pr) | 10.5-15 L/min (radial), 15 “(axial)” |
| Copy time | 3 s (max. 100 s) |
| Copy and reading time | 1-5 s (max. 60 s) |
| Viewing height | 5-12 mm |