Microbial activity response to hydrogen injection in thermophilic anaerobic digesters revealed by genome-centric metatranscriptomics

Alessandra Fontana; Panagiotis G. Kougias; Laura Treu; Adam Kovalovszki; Giorgio Valle; Fabrizio Cappa; Lorenzo Morelli; Irini Angelidaki; Stefano Campanaro

Additional file 1

NGS data statistics and 16S rRNA gene amplicon results (Table S1 to S2 and Fig. S1 to S2)

Differentially expressed genes not assigned to MAGs (Fig. S3)

Relative abundance of the 50 MAGs (Fig. S4)

Statistical analysis (Fig. S5)

Methanogenic pathways regulated by the discussed archaeal MAGs (Fig. S6 to S8)

Simulation results (Figures S9 to S19)

Mass balance calculations

Table S1 Statistics regarding raw and filtered data related to all NGS analyses obtained in the current study. Replicates samples for microbial analysis (a, b, c) have been collected in R1, R2 and R3 at three time-points in Phase I, meaning before H2 injection (I, II, III), and at one time point in Phase II, meaning after H2 injection (H). The total number of raw reads and the number of reads after pair-merging and filtering process have been reported per each replica for the three NGS methods applied in the present study: 16S rRNA gene amplicon sequencing (16S amplicons), total random metagenomics (TRS gDNA) and metatranscriptomics (RNA-seq).

|  | 16S amplicons |  | TRS gDNA |  | RNA-seq |  |
| --- | --- | --- | --- | --- | --- | --- |
| ID | Raw reads | Merged and filtered | Raw reads | Merged and filtered | Raw reads | Merged and filtered |
| R1-I-a | 256626 | 91200 | - | - | - | - |
| R1-I-b | 492408 | 139746 | - | - | - | - |
| R1-I-c | 362602 | 120683 | - | - | - | - |
| R1-II-a | 297924 | 89648 | - | - | - | - |
| R1-II-b | 275710 | 87090 | - | - | - | - |
| R1-II-c | 303426 | 104522 | - | - | - | - |
| R1-III-a | 268378 | 95562 | 10287776 | 8818995 | 20940340 | 20536432 |
| R1-III-b | 237382 | 83307 | 17192444 | 14768055 | 23052600 | 22613424 |
| R1-III-c | 220274 | 76683 | 11347582 | 9714979 | 18232671 | 17754036 |
| R1-H-a | - | - | 7804280 | 6587305 | 32812271 | 32203531 |
| R1-H-b | - | - | 8066664 | 6939638 | 30410352 | 29856461 |
| R1-H-c | - | - | 10235162 | 8662206 | 29543921 | 29013189 |
| R2-III-a | - | - | 7720826 | 6454683 | - | - |
| R2-III-b | - | - | 10219246 | 8561994 | - | - |
| R2-III-c | - | - | 9912878 | 8299553 | - | - |
| R2-H-a | - | - | 19631228 | 16360486 | - | - |
| R2-H-b | - | - | 18809546 | 15468619 | - | - |
| R2-H-c | - | - | 16585754 | 13716479 | - | - |
| R3-I-a | 601274 | 165286 | - | - | - | - |
| R3-I-b | 517310 | 150902 | - | - | - | - |
| R3-I-c | 288742 | 82693 | - | - | - | - |
| R3-II-a | 358664 | 115412 | - | - | - | - |
| R3-II-b | 357146 | 121341 | - | - | - | - |
| R3-II-c | 250220 | 83510 | - | - | - | - |
| R3-III-a | 345368 | 107050 | 12483892 | 10590424 | 30112439 | 29555098 |
| R3-III-b | 254714 | 73099 | 12757550 | 10848501 | 26909201 | 26429086 |
| R3-III-c | 453518 | 124249 | 11390552 | 9800708 | 32486849 | 31861174 |
| R3-H-a | - | - | 20172266 | 17496525 | 31533889 | 31006723 |
| R3-H-b | - | - | 11193188 | 9722547 | 29341280 | 28785157 |
| R3-H-c | - | - | 8862690 | 7641383 | 30806835 | 30240013 |

Table S2 Microbial community analysis of samples collected in R1 and R3 at multiple time-points. A comparison between the microbial species putatively identified using the 16S rRNA gene amplicon sequencing and the dominant MAGs reconstructed using total random sequencing (TRS) is showed. OTUs were obtained at three time-points in Phase I (I, II, III), while MAGs are only related to the third time point (R1-TRS, R3-TRS). Relative abundances averages and standard deviations of the OTUs in R1 (R1-I, R1-II, R1-III) and R3 (R3-I, R3-II, R3-III) are reported. The OTUs’ values determined at the third time point were coherent with MAGs relative abundances averages estimated using coverage values.

| 16S rRNA amplicon OUTs | R1-I | R1-II | R1-III |  | R3-I | R3-II | R3-III |  | TRS MAGs | R1-TRS | R3-TRS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D. tunisiensis | 35.04% | 45.56% | 34.04% |  | 66.83% | 45.05% | 42.60% |  | D. tunisiensis UC0050 | 17.32% | 27.78% |
| C. proteolyticus | 22.32% | 18.47% | 27.36% |  | 8.49% | 11.24% | 7.79% |  | C. proteolyticus UC0011 | 43.14% | 10.63% |
| A. hydrogeniformans | 16.50% | 15.72% | 10.58% |  | 3.63% | 1.43% | 5.26% |  | A. hydrogeniformans UC0046 | 17.10% | 7.71% |
| Clostridiales spp. | 19.10% | 15.76% | 23.80% |  | 0.29% | 19.34% | 17.04% |  | Clostridiales sp. UC0015 | 8.59% | 6.55% |
| A. mobile | 1.86% | 0.79% | 1.14% |  | 13.27% | 5.98% | 3.33% |  | A. mobile UC0048 | 1.65% | 4.68% |
| T. acetatoxydans | 2.46% | 1.36% | 1.44% |  | 1.95% | 0.52% | 0.01% |  | T. acetatoxydans UC0018 | 4.28% | 0.06% |
| Bacteroidales spp. | 0.72% | 0.44% | 0.20% |  | 1.94% | 10.92% | 16.05% |  | Bacteroidetes sp. UC0002 | 0.01% | 10.16% |
| Syntrophomonas sp. | 0.02% | 0.02% | 0.01% |  | 0.38% | 0.98% | 1.61% |  | Syntrophomonas sp. UC0014 | 0.10% | 4.87% |
| Anaerolineaceae sp. | 0.00% | 0.01% | 0.00% |  | 0.03% | 0.55% | 1.45% |  | Chloroflexi sp. UC0004 | 0.01% | 2.61% |
| S. schinkii | 0.00% | 0.00% | 0.00% |  | 0.25% | 0.17% | 1.12% |  | Syntrophaceticus sp. UC0017 | 0.06% | 3.65% |
| Haloplasmataceae sp. | 0.00% | 0.00% | 0.00% |  | 0.01% | 0.81% | 0.72% |  | Firmicutes sp. UC0013 | 0.02% | 1.91% |
| Anaerobaculum sp. | 0.48% | 0.81% | 0.43% |  | 0.30% | 0.05% | 0.31% |  | Anaerobaculum sp. UC0047 | 0.59% | 1.96% |
| Methanothermobacter sp. | 0.36% | 0.19% | 0.33% |  | 0.69% | 0.09% | 0.22% |  | M. wolfeii UC0008 | 1.09% | 1.02% |
| M. thermophila | 0.00% | 0.00% | 0.00% |  | 0.09% | 0.02% | 0.06% |  | M. thermophila UC0006 | 0.04% | 0.66% |

Several OTUs having variable relative abundances between different time-points in R3 were could not be classified at high taxonomical level. In particular, 4 OTUs were grouped as Clostridiales spp. and 2 OTUs were assigned to Bacteroidales spp. A possible explanation of these differences may be related to the limitation of 16S rRNA gene amplicon technique; in fact, information obtained from such a small fragment of DNA are not enough to discriminate the specific microbes that may be responsible for the anaerobic digestion process. Moreover, the major variations between time points were recorded in OTUs assigned to Tepidanaerobacter acetatoxydans, Haloplasmataceae sp. and Anaerolineaceae sp. (35-fold, 18-fold and 13-fold changes on average, respectively). These dynamic OTUs showed two opposite trends: T. acetatoxydans was clearly disappearing with time, while Haloplasmataceae sp. and Anaerolineaceae sp. were increasing in abundance. This finding indicates a potential functional redundancy in this particular AD system, meaning that the two OTUs presenting increased growth could substitute the role of T. acetatoxydans in the microbial consortium. Indeed, Haloplasmataceae and Anaerolineaceae families are known to include fermenting bacteria species which were found to be positively correlated to acetate concentrations in anaerobic digesters [1,2]. Therefore, it could be possible that these two microbes functionally replaced T. acetatoxydans in both reactor configurations, during the progressive increment of acetate levels. It can be assumed that such conditions created a more suitable environment for these two species, resulting in a faster growth.

References1. Li YF, Nelson MC, Chen PH, Graf J, Li Y, Yu Z. Comparison of the microbial communities in solid-state anaerobic digestion (SS-AD) reactors operated at mesophilic and thermophilic temperatures. Appl. Microbiol. Biotechnol. 2014;

2. Yi J, Dong B, Jin J, Dai X. Effect of increasing total solids contents on anaerobic digestion of food waste under mesophilic conditions: Performance and microbial characteristics analysis. PLoS One. 2014

Fig. S1 β-diversity calculated among the samples. Results obtained for β-diversity calculation (Whittaker method) were visualized as a heatmap. Correspondence between colors and β-diversity values is reported in the scale at the bottom of the matrix. Average β-diversity values were 0.12 for samples collected in R1, 0.12 for samples in R3 and 0.25 for comparisons between R1 and R3.

Fig. S2 Principal coordinate analysis (PCoA) plot representing variations of the OTUs based on phylogenetic distance and Unweighted UniFrac method. Replicate samples obtained from the two reactors (R1 and R3) at three time-points (I, II and III) are showed as uncorrelated linear principal coordinates. The two coordinates represented in the graph contribute to more than 70% of total variance (PCo1 58% and PCo2 14%).

Fig. S3 Number of total differentially expressed genes (not assigned to MAGs) per each COG category, in the single stage (A) and two-stage (B) reactors.

Fig. S4 Relative abundance of the 50 MAGs distributed in the different reactor configurations. R1 and R1H: single stage pre- and post-H2, respectively; R2 and R2H: acidogenic reactor of the two-stage pre- and post-H2, respectively; R3 and R3H: methanogenic reactor of the two-stage pre- and post-H2, respectively; a, b, c: replicates.

Fig. S5 Canonical correspondence analysis (CCA) testing significative correlations between relative abundance of the MAGs and reactor parameters, in the single stage reactor (A), acidogenic reactor of the two-stage configuration (B), and methanogenic reactor of the two-stage configuration (C). R1 and R1H: single stage pre- and post-H2, respectively; R2 and R2H: acidogenic reactor of the two-stage pre- and post-H2, respectively; R3 and R3H: methanogenic reactor of the two-stage pre- and post-H2, respectively; a, b, c: replicates).

Fig. S6 KEGG methane metabolism of Methanothermobacter wolfeii UC0008 in the single stage reactor (R1). Over and under expressed genes are labeled in red and green, respectively.

Fig. S7 KEGG methane metabolism of Methanothermobacter wolfeii UC0008 in the methanogenic reactor of the serial configuration (R3). Over and under expressed genes are labeled in red and green, respectively.

Fig. S8 KEGG methane metabolism of Methanosarcina thermophila UC006 in the methanogenic reactor of the serial configuration (R3). Over and under expressed genes are labeled in red and green, respectively.

SIMULATION RESULTS

Single stage reactor:

Fig. S9 Methane productivity in R1.

Fig. S10 Total VFA concentration in the digestate of R1.

Fig. S11 Butyric acid concentration in the digestate of R1.

Fig. S12 Concentration of different simulated microbial groups in R1. CARB_aci, PROT_aci and LIPID_aci are acidogenic bacterial groups; LCFA_ace, Pro_ace, But_ace and Val_ace are acetogenic bacterial groups; Ace_meth and Hyd_meth are methanogenic archaeal groups; SAOB are syntrophic acetate oxidixing bacterial groups. Initial concentrations were defined by fitting the initial experimental data points of other variables to their respective simulation curves. The results are not quantitative, providing only limited information about the succession dynamics of the microbial community.

Two-stage, acidogenic reactor:

Fig. S13 Total VFA concentration in the digestate of R2.

Fig. S14 Concentration of different simulated microbial groups in R2. CARB_aci, PROT_aci and LIPID_aci are acidogenic bacterial groups; LCFA_ace, Pro_ace, But_ace and Val_ace are acetogenic bacterial groups; Ace_meth and Hyd_meth are methanogenic archaeal groups; SAOB are syntrophic acetate oxidixing bacterial groups. Initial concentrations were defined by fitting the initial experimental data points of other variables to their respective simulation curves. The results are not quantitative, providing only limited information about the succession dynamics of the microbial community.

Two-stage, methanogenic reactor:

Fig. S15 Methane productivity in R3.

Fig. S16 Total VFA concentration in the digestate of R3.

Fig. S17 Butyric acid concentration in the digestate of R3.

Fig. S18 Hydrogen fraction in the effluent gas of R3.

Fig. S19 Concentration of different simulated microbial groups in R3. CARB_aci, PROT_aci and LIPID_aci are acidogenic bacterial groups; LCFA_ace, Pro_ace, But_ace and Val_ace are acetogenic bacterial groups; Ace_meth and Hyd_meth are methanogenic archaeal groups; SAOB are syntrophic acetate oxidixing bacterial groups. Initial concentrations were defined by fitting the initial experimental data points of other variables to their respective simulation curves. The results are not quantitative, providing only limited information about the succession dynamics of the microbial community.

MASS BALANCE CALCULATIONS

Reactions considered:

Acetogenesis                                    2CO2+4H2→CH3COOH+2H2O

Propionate formation                    CH3COOH+3H2+CO2→CH3CH2COOH+2H2O

Butyrate formation                        CH3CH2COOH+3H2+CO2→CH3CH2CH2COOH+2H2O

Hydrogenotrophic methanogenesis       4H2+CO2→CH4+2H2O

molesH2, injected /day = VH2, injected /day /Vmolar, ideal gas, 55°C = 2.45 L/day / 26.9 L/mol= 0.0911 mol/day

Single stage reactor:

- Methane increase (Δmethane) after H2= 0.24 L/day

molesΔmethane= 0.24 L/day / 26.9 L/mol = 0.0089 mol

molesH2, used for Δmethane = 0.0089 x 4 = 0.0357 mol (~40% of the H2 moles injected is utilized for the methane increase)

- Butyrate increase (Δbutyrate) after H2= 1.2 g/L

molesΔbutyrate= 1.2 g/L x 3 Lr = 3.6 g / 88.1 g/mol = 0.04086 mol

Hypothesis a): molesH2, used for Δbutyrate (from CO2) = 0.04086 mol x 10 mol H2 = 0.409 mol

Hypothesis b): molesH2, used for Δbutyrate (from acetate) = 0.04086 mol x 6 mol H2 = 0.245 mol

Hypothesis c): molesH2, used for Δbutyrate (from propionate) = 0.04086 mol x 3 mol H2 = 0.123 mol

The left 60% H2 moles injected (0.0554 mol) are not enough for the butyrate increment registered, considering all hypotheses (a, b and c). Thus, the utilization of indigenous H2 should be considered:

molesH2, BIOGAS = 0.48 L/day / 26.9 L/mol = 0.018 mol

molesH2, left for other metabolisms = 0.055 – 0.018 = 0.037 mol

Hypothesis a): 0.409-0.037 = 0.37 mol (too much indigenous H2 required to produce butyrate from CO2, not reasonable)

Hypothesis b): 0.245-0.037 = 0.21 mol (too much indigenous H2 required to produce butyrate from acetate, not reasonable)

Hypothesis c): 0.123-0.037 = 0.09 mol (most suitable hypothesis)

Considering hypothesis c) and the equation for lactose fermentation to acetate and butyrate (main VFA in R1):

C12H22O11+5H2O →8H2+4CO2+4CH3COOH

C12H22O11+H2O →4H2+4CO2+2CH3CH2CH2COOH

moles lactose in the feed/day = 5.6 g/ 342.3 g/mol = 0.0164 mol

moles H2 from lactose fermentation to acetate = 0.0164 x 8=0.13 mol

moles H2 from lactose fermentation to butyrate = 0.0164 x 4=0.07 mol

It is therefore possible to consider the utilization of indigenous hydrogen to explain the butyrate increment via propionate reduction (this last VFA is indeed the lowest in concentration).

Two-stage, acidogenic reactor:

- Butyrate increase (Δbutyrate) after H2= 1.3 g/L

molesΔbutyrate= 1.3 g/L x 0.6 Lr = 0.78 g / 88.1 g/mol = 0.0089 mol

Hypothesis a): molesH2, used for Δbutyrate (from CO2) = 0.0089 mol x 10 mol H2 = 0.089 mol (~97% of the H2 injected)

Hypothesis b): molesH2, used for Δbutyrate (from acetate) = 0.0089 mol x 6 mol H2 = 0.053 mol (~60% of the H2 injected)

Propionate reduction hypothesis has not been considered since propionate concentration was negligible in this reactor (0.020 ± 0.003 g/L).

Two-stage, methanogenic reactor:

molesH2, BIOGAS = 0.94 L/day / 26.9 L/mol = 0.035 mol

Hypothesis a): moles H2, from R2 = (0.091 – 0.089) mol = 0.003 mol (no injected H2 goes to R3)

Hypothesis b): moles H2, from R2 = (0.091 – 0.053) mol = 0.038 mol (~equal to the H2 moles in the output biogas; thus, probably not utilized for other metabolic reactions)

Thus, the H2 injected in R2 did not reach R3 (hypothesis a) or was not utilized (hypothesis b). Since the concentration of acetate slightly decreased in R2, and 30% of the injected hydrogen was found in the effluent gas of R3, hypothesis b) seems the most suitable.

- Butyrate increase (Δbutyrate) after H2= 0.7 g/L (most probably from R2 and/or acetogenesis inhibition in R3, given acetate concentration)

molesΔbutyrate= 0.7 g/L x 2.4 Lr = 1.68 g / 88.1 g/mol = 0.0191 mol

- Acetate increase (Δacetate) after H2= 1.7 g/L

molesΔacetate= 1.7 g/L x 2.4 Lr = 4.08 g / 60.05 g/mol = 0.068 mol

Most of these acetate moles probably derived from butyrate degradation:

CH3CH2CH2COOH+2H2O→2CH3COOH+2H2

molesΔacetate from butyrate oxidation = 0.0191 molΔbutyrate x 2= 0.038 (~60% of the acetate increment experienced)

Acetogenesis account for acetate increase:

molesΔacetate from acetogenesis = 0.029 mol/4 = 0.007 mol (~10% of the acetate increment experienced)

Thus, the left 30% of augmented acetate moles was probably due to an accumulation effect, which may have also inhibited the acetogenic pathway.