SOP _09_THICK_BLOOD_SMEAR_SAMPLING

- SOP code: SOP_09_Thick_blood_smear_sampling_V02_EN

- Area: Equatorial Guinea Mainland

- Version: V02

- Language: English

- Title: Operational procedures on the thick blood smear collection and analysis of samples

- Written by /date: Thuy-Huong Ta Tang 15/10/2019

- Revised by / date: Zaida Herrador 16/10/2019; Lidia Redondo 16/10/2019; Laura Reguero 17/10/2019; Marta García 17/10/2019

- Approved by / date and signature: Agustín Benito 19/10/2019

- Original version: Spanish

# OBJETIVES

To describe the procedure related to the collection of a sample to perform a thick blood smear for the parasitological diagnosis of microfilariae and their species identification.

# DEFINICIONES

Thick blood smear (TBS): thick layer of erythrocytes without haemoglobin (lysates).  Blood elements and parasites, if present, are more concentrated than in an equivalent area of a thin smear.  Thus, the thick smear is more efficient for the detection of parasites (increased sensitivity).

# APPLICABLE TO

Technicians, team supervisors, coordinators and coordination assistants.

- IMPLEMENTATION DATE

  - Team building: November 2019

  - Field work: November-December 2019.

- PROCEDURES

  - Sample collection: Thick blood smear (TBS)

Identify the sample by sticking the corresponding barcode on the slide with matte edge.

The person taking the sample should always wear gloves.

- Hold the patient's hand palm upwards and wipe the third fingertip with a cotton swab soaked in 70% alcohol. In the case of a child, a heel swab can be used.

- Dry the finger with a clean, dry cotton swab while holding it vigorously. With a sterile lancet, quickly prick the fingertip laterally.

- Discard the sterile, single-use lancet in an appropriate container.

- Obtain a drop of blood by pressing lightly on the finger and deposit it on the slide with a clean, grease-free, matte edge and avoid touching the slide to the finger.

- Once the drop of blood has been deposited on the slide, using the corner of another slide (without a dull edge), spread the blood to a diameter of about 1 cm in a circular motion. The thickness should be such that it is possible to read through the preparation.

- The thick blood smear drop IS NOT FIXED with methanol or heat. Allow to AIR DRY protected from dust and insects, normally 30-60 minutes.

Once dry, stored it in the slide holder.

- RELATED DOCUMENTS

- SOP_01_SAMPLING STRATEGY

- SOP_03_SURVEY CONDUCT

- SOP_10_STORAGE AND SHIPPING SAMPLE