Supplemental Table 2. Equipment, analytical methods and software used for data acquisition and analysis.

| Methodology | Equipment | Analysis | Software |
| --- | --- | --- | --- |
| Live-cell Imaging | Leica TCS SP8 confocal microscope running LAS X 3.5.2.18963 (Leica Microsystems, Wetzlar, Germany), 20×/0.75 dry objective and 488 or 522 nm excitation at airy 1 (456×456 µm, 1024×1024-pixel images, 32 slices with 1.3 µm spacing). Two different positions per well were acquired. | Tumor cells morphology Cells roundness and perimeter measurement (4 cells/image), using the polygon tool | ImageJ version 1.53f, RRID: SCR_003070 |
|  |  | Position relatively to the endothelium Construction of 3D and orthogonal representations (all cells/image) Criteria: above the endothelium level (out), partially or fully intercalated within the endothelial monolayer (stage 1 or stage 2, respectively) and below the endothelium (transmigrated) |  |
|  |  | In-plane distance migrated Mass center coordinates measurement, using the polygon tool, and distance calculated by Pythagorean theorem (all cells/image) |  |
|  |  | Endothelial monolayer integrity Measurement of HBMEC monolayer covered area in single and mixed culture, using the polygon tool |  |
| Transmigration | Olympus BX60 microscope equipped with Olympus U-RFL-T Mercury lamp and Hamamatsu Orca R2 cooled monochromatic CCD camera, using a 10×/0.4 objective. 15 fields/condition were acquired. | Transmigrated cells Count the number of cells, using the CellCounter tool (all cells/field) | ImageJ version 1.53f, RRID: SCR_003070 |
| Transendothelial Eletrical Resistance (TEER) | EndOhm™ chamber and resistance meter (EVOMX; World Precision Instruments, Inc, USA | Normalized TEER values TEER was calculated after deducting the values of empty insert and multiplying by the insert area (1.12 cm2), being expressed as percentage variation from control readings. | n/a |
| Paracellular Permeability to Sodium Fluorescein (NaF) | Varioskan LUX Multimode Microplate Reader (Thermo Fisher Scientific) | NaF levels Fluorescence measurements at excitation/emission: 460/515 nm, and the transendothelial permeability coefficient was calculated, as described 28 | n/a |
| Flow Cytometry | Cytek® Aurora (Aurora spectral cytometer, Cytekä Biosciences, USA), using SpectroFlow software. At least 25000 (endothelial and mixed culture) or 5000 (tumoral cultures) events were recorded. | Data cleaning Homogeneous time and singlet populations isolation Mean Fluorescence Intensity (MFI) Measurement of each adhesion marker mean intensity using the polygon tool in each cell population | FCS express version 7.10.0007, RRID: SCR_016431 |
| Immunocytochemistry | Olympus BX60 microscope equipped with Olympus U-RFL-T Mercury lamp and Hamamatsu Orca R2 cooled monochromatic CCD camera, using a 10×/0.4 objective. 15 fields/condition were acquired. 10 fields/condition were acquired. | Mean Fluorescence Intensity Cell body fluorescence was measured using Polygon tool (5 cells/field) | Icy version 1.8.6.0, RRID: SCR_010587 |
|  |  | Nuclei Strong Fluorescence Intensity Threshold of intensity application Criteria: strong when all the nuclei are homogeneously stained; moderate, when partially stained and negative, when is without staining | ImageJ version 1.53f, RRID: SCR_003070 |
|  |  | Actin organization and presence of cortactin puncti on tumor cells protrusions A line was drawn along a representative cell cytoplasm or protrusion, and the plot profile tool of protein fluorescence intensity was obtained. |  |
|  |  | Endothelial cell body and nuclei morphology Cell body: A line was drawn in the length and width of cellular body and the ratio of length/width was calculated, Nuclei: each nucleus was delineated using Polygon tool and their solidity was measured. Irregularity index=1-solidity |  |
|  |  | Tumor cells adhesion to endothelium Number of cells were counted, using the CellCounter tool (all cells/field) |  |
| Wound-healing Assay | Nikon ECLIPSE TS100, Jenoptik, using the 10× objective. 3 fields/condition were acquired. | Wound closure The distance between the ends of the wound was measured at 3 different sites (top, center and bottom), and the percentage of wound closure was determined using equation: % closure=100-( wound distance at 24 or 48 h × 100wound distance at 0 h) | ImageJ version 1.53f, RRID: SCR_003070 |
| Cell Viability | Varioskan LUX Multimode Microplate Reader (Thermo Fisher Scientific) | Absorbances at 570 (reduced form) and 600 nm (oxidized form) were measured and the percentage of viability was calculated: Cell viability %= A570-A600 transfected cellsA570-A600 control cells x 100 | n/a |
| RT-qPCR | QuantStudio™ 7 Flex Real-Time PCR System (Applied Biosystems, Thermo Fisher Scientific) | Reactions Settings 1 cycle 95°C for 2-3 min, 40 cycles 95°C for 5 sec, 60-65°C for 20-30 sec, followed by a dissociation/melting curve analysis. The quantifications were performed by the ∆∆Ct method and results were presented by normalization to GAPDH followed by fold-change to untreated cells. | Applied Biosystems QuantStudio 12K Flex Real-Time PCR System, RRID: SCR_021098 |

n/a, non-applicable