- Wire connection between the Arduino uno and the motor driver

Table S1. Pin mapping for Bluetooth communications

| Uno ↔ HC-06 |  |  | Uno ↔ Motor driver |  | Motor driver ↔ step-motor |  |
| --- | --- | --- | --- | --- | --- | --- |
| P0 | TXD | Turret | P2 | ENABLE- | /B | /B |
| P1 | RXD |  | P3 | ENABLE+ | B | B |
| GND | GND |  | P4 | DIRECTION- | /A | /A |
| 5V | VCC |  | P5 | DIRECTION+ | A | A |
|  |  |  | P6 | CLOCK- | V- | GND |
|  |  |  | P9 | CLOCK+ | V+ | 12V |
|  |  | Mirror | P7 | ENABLE- | /B | /B |
|  |  |  | P8 | ENABLE+ | B | B |
|  |  |  | P11 | DIRECTION- | /A | /A |
|  |  |  | P12 | DIRECTION+ | A | A |
|  |  |  | P10 | CLOCK- | V- | GND |
|  |  |  | P13 | CLOCK+ | V+ | 12V |

- Bluetooth communication code for operating step-motor

1. HC-06 Module Setup and Communication using Smartphones

#include <SoftwareSerial.h>

SoftwareSerial mySerial(2, 3);

void setup()

{

Serial.begin(9600);

Serial.println("Hi!");

mySerial.begin(9600);

mySerial.println("Hello!");

}

void loop()

{

if (mySerial.available())

{Serial.write(mySerial.read());}

if (Serial.available())

{mySerial.write(Serial.read());}

}

2. Controlling two step-motors

#include <PWM.h>

int control=0;

int pwm1 = 10;

int pwm2 = 9;

void setup()

{

Serial.begin(115200);

Serial.println("Hello!");

InitTimersSafe();

//Motor - Mirror

pinMode(pwm1, OUTPUT); // Motor Driver CLOCK+

pinMode(13, OUTPUT); // Motor Driver CLOCK-

pinMode(12, OUTPUT); // Motor Driver DIRECTION+

pinMode(11, OUTPUT); // Motor Driver DIRECTION-

pinMode(8, OUTPUT); // Motor Driver ENABLE+

pinMode(7, OUTPUT); // Motor Driver ENABLE-

digitalWrite(12, HIGH); //DIR+

digitalWrite(8, HIGH); //ENA+

digitalWrite(13, LOW); //CLK-

digitalWrite(11, LOW); //DIR-

digitalWrite(7, LOW); //ENA-

//Motor - Turret

pinMode(pwm2, OUTPUT); // Motor Driver CLOCK+

pinMode(6, OUTPUT); // Motor Driver CLOCK-

pinMode(5, OUTPUT); // Motor Driver DIRECTION+

pinMode(4, OUTPUT); // Motor Driver DIRECTION-

pinMode(3, OUTPUT); // Motor Driver ENABLE+

pinMode(2, OUTPUT); // Motor Driver ENABLE-

digitalWrite(5, HIGH); //DIR+

digitalWrite(3, HIGH); //ENA+

digitalWrite(6, LOW); //CLK-

digitalWrite(4, LOW); //DIR+

digitalWrite(2, LOW); //ENA-

}

void loop()

{

while (Serial.available())

control = Serial.read();

//Motor - Mirror

if(control == '1' || '2' || '3' || '4'){

pwmWrite(pwm1,191);

int frequency1 = 500;

bool success = SetPinFrequencySafe(pwm1, frequency1);

if(control == '1'){

Serial.println("M1 RIGHT");

digitalWrite(12, HIGH); //DIR+

delay (1000); }

else if(control == '2'){

Serial.println("M1 LEFT");

digitalWrite(12, LOW); //DIR+

delay (1000); }

else if(control == '3'){

Serial.println("M1 STOP");

digitalWrite(8, HIGH); //ENA+

delay (1000); }

else if(control == '4'){

Serial.println("M1 GO");

digitalWrite(8, LOW); //ENA+

delay (1000); }

}

//Motor - Turret

if(control == 'q' || 'w' || 'e' || 'r'){

pwmWrite(pwm2,25);

int frequency2 = 3500;

bool success1 = SetPinFrequencySafe(pwm2, frequency2);

if(control == 'q'){

Serial.println("M2 RIGHT");

digitalWrite(5, HIGH); //DIR+

delay (1000); }

else if(control == 'w'){

Serial.println("M2 LEFT");

digitalWrite(5, LOW); //DIR+

delay (1000); }

else if(control == 'e'){

Serial.println("M2 STOP");

digitalWrite(3, HIGH); //ENA+

delay (1000); }

else if(control == 'r'){

Serial.println("M2 GO");

digitalWrite(3, LOW); //ENA+

delay (1000); }

}

}

- Peak assignment for Raman spectra of silicon wafer, 4-nitrobenzenethiol, and rhodamine 6G

Table S2. Raman peak assignments of silicon wafer, 4-nitrobenzenethiol, and rhodamine 6Ga (numbers in cm-1)

| Peak No. | Si wafer | 4-nitrobenzenethiol | Rhodamine 6G |
| --- | --- | --- | --- |
| 1 | 519 |  |  |
| 2 |  |  | 604 |
| 3 |  |  | 680 |
| 4 |  |  | 761 |
| 5 |  |  | 865 |
| 6 |  |  | 960 |
| 7 |  |  | 1018 |
| 8 |  | 1085 |  |
| 9 |  |  | 1092 |
| 10 |  |  | 1266 |
| 11 |  | 1321 |  |
| 12 |  |  | 1356 |
| 13 |  |  | 1432 |
| 14 |  |  | 1491 |
| 15 |  | 1583 |  |

- All of the Raman and SERS spectra are obtained with ex = 632.8 nm.