Supporting information

Table S1. Sequences of the designed oligonucleotides

| Title | Sequences (5’ to 3’) |
| --- | --- |
| LT | GCA ACC TCG GAG ACT TAA ACT GGA GAC ACG ATG GCA ACC TCG GAG ACT TAT TTT TTT TTT CTC CGA GGT TAC CCC TAT CAC GAT TAG CAT TAA |
| miRNA-155 | UUA AUG CUA AUC GUG AUA GGG GU |
| smiR-155 | UUA AUG UUA AUC GUG AUA GGG GU |
| tmiR-155 | UUA UUG UUA UUC GUG AUA GGG GU |
| miRNA-141 | UAA CAC UGU CUG GUA AAG AUG G |
| miRNA-21 | UAG CUU AUC AGA CUG AUG UUG A |
| SP | AGG TTG CCA T GGG TAG GGC GGG TTG GGT ATGGCAA CCT |
| Primer | CGT AAG TCT CCG AGG |

Table S2. A brief comparison of the method with former ones.

| Title | Mechanism | Enzymes | Target miRNA | LOD | Signal mode | Label-free | Advantages and disadvantages | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| The method | Exo III assisted target recycling+ EXPAR | Polymerase, endonuclease; | miRNA-155 | 0.43 fM | Color change | √ | Advantages: high sensitivity; elevated specificity of EXPAR; label-free signal generation; high specificity. Disadvantages: the method could be further integrated in POCT application. |  |
| RACE | RCA | T4 ligase, phi29 polymerase, Cas9+ sgRNA | Let-7a, miRNA-155, miRNA-21 | 90 fM | Fluorescence | × | Advantages: high specificity. Disadvantages: low sensitivity; multiple enzymes; labeling moiety for signal generation; require essential equipment for result readout. | 1 |
| Enhanced RCA | RCA+ CRISPR-Cas12a | T4 ligase, phi29 polymerase, Cas9+ sgRNA | Let-7a | 47 fM | Fluorescence | × | Advantages: high specificity; improved sensitivity. Disadvantages: low sensitivity; multiple enzymes; labeling moiety for signal generation; require essential equipment for result readout. | 2 |
| DNAzyme | Target recycling | Hemin | miRNA-155 | 0.7 nM | Color change | √ | Advantages: direct signal readout. Disadvantages: low sensitivity; | 3 |

Note: RCA, rolling circle amplification; EXPAR, exponential amplification reaction; Exo III, exonuclease III; RACE, Rolling Circular Amplification (RCA)-Assisted CRISPR/Cas9 Cleavage.

References:

1.	Wang, R.;  Zhao, X.;  Chen, X.;  Qiu, X.;  Qing, G.;  Zhang, H.;  Zhang, L.;  Hu, X.;  He, Z.;  Zhong, D.;  Wang, Y.; Luo, Y., Rolling Circular Amplification (RCA)-Assisted CRISPR/Cas9 Cleavage (RACE) for Highly Specific Detection of Multiple Extracellular Vesicle MicroRNAs. Anal. Chem. 2020, 92 (2), 2176-2185.

2.	Zhang, G. Z., L.; Tong, J.; Zhao, X.; Ren, J.;, CRISPR-Cas12a enhanced rolling circle amplification method for ultrasensitive miRNA detection. Microchem. J. 2020, 158 (2020), 105239.

3.	Shahsavar, K.;  Shokri, E.; Hosseini, M., Sensitive colorimetric detection of miRNA-155 via G-quadruplex DNAzyme decorated spherical nucleic acid. Mikrochim. Acta 2022, 189 (9), 357.