Online Supplement

Flow-controlled ventilation versus pressure-controlled ventilation in moderate to severe ARDS patients: a randomized crossover physiological study

Julien P. Van Oosten 1, Juliette E. Francovich1, Dolf Weller1,2, Wim Rietdijk1,3, Nico Goedendorp1, Peter Somhorst1, Corstiaan A. Den Uil2, Diederik A.M.P.J. Gommers1, Annemijn H. Jonkman1, Henrik Endeman1,4

Author affiliations:

1)	Intensive Care, Erasmus Medical Center, Rotterdam, The Netherlands

2)	Intensive Care, Maasstad Hospital, Rotterdam, The Netherlands

3) 	Department of Hospital Pharmacy, Erasmus Medical Center, Rotterdam, The

Netherlands

4)	Intensive Care, OLVG, Amsterdam, The Netherlands

EXTENDED RESULTS SECTION

Population and characteristics

In total, 15 patients were recruited for the study, of whom 10 patients were included in the analysis; their main characteristics are presented in Table 1. Five patients were excluded because of reaching a P/F ratio > 200mmHg between enrolment and start of study (n=3), a major pneumothorax discovered at the start of study making EIT measurements unreliable (n=1) and hemodynamic instability (n=1). The median P/F ratio at enrolment was 159 mmHg, the median SOFA score was 11 and in total 3 patients were on veno-venous extracorporeal membrane oxygenation (VV-ECMO) during the study, and 1 patient was in prone position.

No serious adverse events were reported, however in one patient the study was stopped after 60 minutes on FCV because of severe hypercapnia resulting in a pH < 7.20 despite FCV optimization (adverse event). The patient with an ARDS of unknown origin was stable for 90 minutes on PCV (PEEP 10 cmH2O, Ppeak 24cmH2O, RR 30min-1), resulting in a tidal volume of 252 ml and PaCO2 of 6.71 kPa. During step 1 FCV was initiated with the same PEEP and Ppeak settings, but this resulted in a PaCO2 of 7.26 kPa (flow 20L/min, RR 28 min-1). During step 2 we were not able to optimize FCV as intended, because compliance decreased while increasing the driving pressure. Despite a Ppeak of 27cmH2O, maximum flow of 20L/min (resulting in a tidal volume of 228ml, RR of 28 min-1) the PaCO2  increased to 7.94. To prevent further respiratory acidosis FCV was stopped. In this patient we used the measurements at 60 minutes of FCV as step 3.

Switch from PCV to FCV with similar settings (PCV vs. FCV step 1)

FCV with ‘similar’ PCV settings did not affect overall MPRS (12.1 (10.6-14.6) vs. 14.8 (11.1-18.4) J/min, p=0.121) and dissipated energy (0.23 (0.14-0.27) vs. 0.28 (0.13-0.40) J/L, p=0.363). However, FCV resulted in a lower MPL (5.2 (4.0-6.4) vs. 6.8 (4.9-8.5) J/min, p=0.008) and dissipated energy (0.19 (0.08-0.21) vs. 0.25 (0.13-0.29) J/L, p=0.003) of the lung (values for FCV and PCV, respectively). For all results comparing FCV step 1 and PCV, see Supplemental Tables 1 and 2.

The EELV increased by 16.1 ml (1.6-25.2, p=0.007) during FCV compared to PCV (Supplemental Table 2). Although the GI index did not change (Supplemental Table 2), FCV with similar PCV settings showed a trend towards a more homogeneous spatial ventilation distribution with increased participation of the dorsal lung regions using EIT (Supplemental Figure 2). However, this was accompanied by a decrease in compliance of most regions of interest in 8 patients, particularly in the ventral/non-dependent lung parts, suggesting overdistension (Supplemental Table 2). In the other 2 patients the compliance did not change or slightly increased.

Supplemental Table 1. Results PCV vs. FCV with PCV settings (step 1)

|  | PCV Median (IQR) | FCV step 1 Median (IQR) | P-value |
| --- | --- | --- | --- |
| Respiratory parameters |  |  |  |
| Inspiratory TV/IBW (mL) | 4.9 (4.1-5.6) | 4.8 (3.6-6.1) | 0.783 |
| Expiratory TV/IBW (mL) | 4.9 (4.2-5.6) | 4.7 (3.7-6.1) | 0.869 |
| ΔP (cmH2O) | 10.4 (8.6-15.0) | 11.6 (9.2-14.6) | 0.502 |
| PEEP set (cmH2O) | 13.0 (9.5-16.5) | 13.0 (9.5-16.5) | 1.000 |
| PEEP total (cmH2O) | 14.7 (11.2-16.9) | 14.9 (12.1-16.8) | 0.388 |
| Ppeak set (cmH2O) | 27.0 (24.0-32.0) | 25.5 (24.0-30.3) | 0.128 |
| Ppeak measured (cmH2O) | 25.7 (23.8-31.7) | 26.5 (25.9-31.7) | 0.053 |
| Pplateau (cmH2O) | 24.3 (23.3-29.9) | 24.3 (23.7-30.7) | 0.194 |
| Pmean (cmH2O) | 18.5 (15.9-22.9) | 19.2 (17.6-23.7) | 0.052 |
| I:E ratio | 0.65 (0.57-0.81) | 0.94 (0.84-0.96) | 0.002 |
| Cstatic RS (ml/cmH2O) | 32.7 (20.7-41.2) | 30.2 (18.1-41.7) | 0.441 |
| Resistance (cmH2O/L/s)# | 14.8 (12.6-19.8) | 8.6 (7.2-10.6) | 0.007 |
| RR (x/min) | 23.0 (15.2-30.0) | 18.9 (14.3-25.7) | 0.122 |
| MV (L/min) | 7.6 (5.1-9.7) | 6.3 (5.2-8.4) | 0.117 |
| End inspiratory PL (cmH2O) | 12.0 (9.7-13.9) | 11.5 (10.4-14.0) | 0.485 |
| End expiratory PL (cmH2O) | 4.0 (1.9-5.1) | 3.3 (1.9-4.5) | 0.471 |
| ΔPL (cmH2O) | 7.6 (6.2-11.7) | 8.6 (6.4-13.2) | 0.965 |
| Cstatic lung (ml/cmH2O) | 43.4 (27.1-58.1) | 42.6 (26.5-59.1) | 0.685 |
| Cstatic chest wall (ml/cmH2O) | 162 (136-234) | 163 (144-229) | 0.803 |
| MPRS (J/min)$ | 14.8 (11.1-18.4) | 12.1 (10.6-14.6) | 0.121 |
| MPL (J/min) | 6.8 (4.9-8.5) | 5.2 (4.0-6.4) | 0.034 |
| Dissipated energy RS (J/L) | 0.28 (0.13-0.40) | 0.23 (0.14-0.27) | 0.363 |
| Dissipated energy lung (J/L) | 0.25 (0.13-0.29) | 0.19 (0.08-0.21) | 0.003 |
| Gas exchange parameters |  |  |  |
| P/F ratio* (mmHg) | 149 (131-194) | 150 (124-180) | 0.417 |
| PaO2 (kPa) | 11.6 (10.9-13.0) | 12.0 (11.5-12.8) | 0.545 |
| PaCO2 (kPa) | 6.1 (5.6-6.4) | 6.4 (6.1-7.2) | 0.017 |
| Ventilatory ratio | 1.4 (0.8-1.9) | 1.3 (0.8-1.6) | 0.365 |
| Hemodynamic parameters |  |  |  |
| Arterial-venous delta CO2 (kPa) |  | 0.64 (-0.21-0.73) | 0.50 (0.40-0.66) | 0.617 |
| ScvO2 (%) |  | 81.4 (79.9-86.2) | 81.9 (79.6-86.9) | 0.887 |
| Pulserate (beats/min) |  | 69 (59-99) | 69 (65-92) | 1.000 |
| Mean arterial pressure (mmHg) |  | 79.0 (73.8-91.3) | 76.0 (70.0-84.0) | 0.920 |
| Dose norepinephrine (µg/kg/min) |  | 0.10 (0.06-0.38) | 0.10 (0.05-0.42) | 0.363 |

Abbreviations: ΔP: airway driving pressure; ΔPL: transpulmonary driving pressure; Cstatic RS: Static Compliance Respiratory System; Cstatic lung: Static Compliance of the Lung; FCV: Flow-Controlled Ventilation; IBW: Ideal Body Weight (men 50/women 45.5 + (0.91x(height in cm – 152.4)); I:E ratio: inspiratory:expiratory ratio; IQR: Inter Quartile Range; MV: Minute Volume; MPL: Mechanical Power of the lung; MPRS: Mechanical Power of the respiratory system; PCV: Pressure-Controlled Ventilation; PEEP: Positive End Expiratory Pressure; P/F ratio: PaO2/FiO2 ratio; PL: Transpulmonary Pressure; RS: Respiratory System; ScvO2: Central Venous Oxygen Saturation; TV: Tidal Volume

#Resistance as measured by the Drager (PCV) and Evone (FCV) ventilators.

*For the calculation of the P/F ratio the three VV-ECMO patients were excluded.

$Primary endpoint of study

Supplemental Table 2. EIT results PCV vs. FCV with PCV settings (step 1); values represent median (IQR)

| Supplemental Table 2a. Changes in EIT parameters during FCV as compared to PCV* |  |  |  |
| --- | --- | --- | --- |
|  |  | FCV step 1 | P-value |
| Change in EELV (ml) |  | 16.1 (1.6-25.2) | 0.007 |
| Global change in ΔZ (%) |  | 0.1 (-19.7-9.5) |  |
| Regional change in ΔZ (%) ROI ventral ROI mid-ventral ROI mid-dorsal ROI dorsal |  | -2.0 (-21.9-5.0) -1.3 (-18.5-10.2) 0.8 (-12.0-18.6) 5.4 (2.7-34.1) | 0.379 0.999 0.328 0.102 |
| Global change in static compliance (%) |  | -10.0 (-13.8-1.1) |  |
| Regional change in static compliance (%) ROI ventral ROI mid-ventral ROI mid-dorsal ROI dorsal |  | -12.0 (-22.5- -3.3) -12.9 (-18.1-0.5) -9.2 (-14.0- 9.0) -2.1 (-11.3-8.3) | 0.070 0.115 0.257 0.424 |
| Supplemental Table 2b. Absolute EIT parameters reflecting lung and ventilation homogeneity |  |  |  |
|  | PCV | FCV | P-value |
| GI (%) | 40.6 (39.9-49.5) | 40.4 (39.9-48.4) | 0.603 |
| RVDi (%) | 1.70 (1.20-4.32) | 2.4 (1.7-4.6) | 0.620 |

Abbreviations: a.u.: arbitrary units; ΔZ: tidal impedance change; EIT: Electrical Impedance Tomography; EELV: end-expiratory lung volume; FCV: Flow-Controlled Ventilation; GI: Global Inhomogeneity index; PCV: Pressure-Controlled Ventilation; ROI: Region Of Interest; RVDi: Regional Ventilation Delay index.

*Changes in ΔZ and static compliance are expressed as percentage change between FCV step 1 and PCV at baseline, since both are expressed in arbitrary units which makes direct comparisons between patients unreliable.

Supplemental Figure 1.

Tube adapter (green) with connector (red) which enables the measurement of intratracheal pressures while allowing the Evone ventilator to operate.

Supplemental Figure 2.

Continuous regional impedance distribution averaged over all patients and per region of interest, in an average inspiration at PCV (left) and step 1 on FCV (right). Shaded areas represent 95% confidence intervals.