Supplementary File

Supplementary tables

| Feedstock / Pretreatment | Dominant Inhibitors (typical range) | Primary Detoxification Module(s) | Representative Engineering | Key Conditions / Observations | References |
| --- | --- | --- | --- | --- | --- |
| Corn stover (high solids /steam explosion) | Furfural, HMF, phenolic monomers, acetic acid from hemicellulose deacetylation. | In situ bioconversion by engineered oxidoreductases Adaptive laboratory evolution and stress response engineering. | Overexpression of NAD(P)H-dependent oxidoreductases converting furans to less toxic alcohols. Integration of efflux pump systems to lower intracellular concentration of inhibitors. | Combined metabolic engineering and ALE can significantly increase detoxification rates and maintain robust fermentation | (Jönsson, Alriksson and Nilvebrant, 2013a; Ujor and Okonkwo, 2022a; Shan et al., 2023) |
| Sugarcane bagasse (alkaline or ammonia fiber explosion) | Ferulic acid, vanillin, HMF (~0.2 g L⁻¹), acetic acid (~3 g L⁻¹) | Laccase/peroxidase enzymatic pretreatment + redox shunt | Addition of fungal laccase (5–10 U mL⁻¹) before fermentation reduces total phenolics by 60–80%; co-expression of pntAB or zwf enhances NADPH regeneration in Z. mobilis | Improves ethanol productivity 1.4–1.8× and reduces lag phase | (Jönsson, Alriksson and Nilvebrant, 2013a; Adebule et al., 2026) |
| Hardwood (organosolv or dilute-acid poplar, birch) | Syringaldehyde, vanillic acid, coniferyl alcohol, lignin-derived oligomers | Laccase or DyP peroxidase detox; Efflux and membrane engineering (hopanoid modulation) | Surface-immobilized laccase (2–5 U mL⁻¹) or DyP-type peroxidase in situ; Z. mobilis mutants with tuned hpnF/shc expression (hopanoid biosynthesis) for phenolic tolerance | 50% reduction in total phenolics; 2.3× increase in cell viability; improved membrane order (fluorescence anisotropy assays) | (Bollag, Shuttleworth and Anderson, 1988; Yi et al., 2015; Brenac et al., 2019a, 2019b) |
| Wheat straw / rice husk | HMF (~0.2–0.4 g L⁻¹), furfural (~1 g L⁻¹), ferulic acid, syringic acid | Combined enzyme–redox module + efflux activation | Co-expression of ffr + fdh and adaptive regulation of marA homologs; ALE under 0.8 g L⁻¹ furfural for 250 h yields upregulation of global stress genes | 3× increase in detoxification rate; stable performance in mixed-hydrolysate medium | (Jilani and Olson, 2023; Yao et al., 2023) |
| Sugarcane bagasse / Corn stover (alkaline or AFEX pretreatment) | Phenolics (ferulic/para-coumaric acids), furans (furfural, HMF), acetic acid | In situ enzymatic detoxification (laccases, peroxidases) Microbial metabolic engineering for efflux and stress response Chemical detoxification when needed (e.g., overliming, activated carbon) | Laccase/peroxidase treatments remove phenolics from hydrolysates before fermentation. Engineering NAD(P)H-dependent reductases enhances furans conversion | Enzymatic detoxification increases fermentability and ethanol yields, though sugar loss should be minimized. Strains adapted or engineered for higher efflux and redox capacity show improved performance in mixed hydrolysates. | (Jönsson, Alriksson and Nilvebrant, 2013b; Sjulander and Kikas, 2020; Ujor and Okonkwo, 2022b; Shan et al., 2023) |
| Softwood (pine, spruce) | Guaiacyl-type phenolics (guaiacol, vanillin), resin acids | Peroxygenase or P450-driven oxidation + efflux | Engineered Agrocybeperoxygenase or bacterial P450 BM3 variants co-expressed in host; integration with MFS pumps (acrB homologs) | Up to 70% detox of guaiacol; improved ethanol yield by 1.5× | (Ibarra et al., 2023; Fernández-Sandoval et al., 2024) |

Table S1. Feedstock-specific mapping of lignin-derived inhibitors and microbial detoxification strategies. The table summarizes dominant fermentation inhibitors typically generated during pretreatment, along with primary enzymatic, redox, and membrane-based detoxification strategies. Representative genetic engineering targets, enzyme systems, and adaptive evolution approaches are highlighted, together with key operating conditions and observed improvements in microbial tolerance, viability, and ethanol productivity.

| Feedstock | Carbohydrate | Content | Reference |
| --- | --- | --- | --- |
| Corn Stover | Cellulose | 35.00% | (Ruan et al., 2019) |
|  | Hemicellulose | 20% |  |
|  | Lignin | 12% |  |
| Wheat Stover | Cellulose | 39% | (Adewuyi, 2022) |
|  | Hemicellulose | 30% |  |
|  | Lignin | 16% |  |
| Rice Husk | Cellulose | 36% | (Isikgor and Becer, 2015a) |
|  | Hemicellulose | 29% |  |
|  | Lignin | 20% |  |
| Coconut Husk | Cellulose | 35% | (Salaenoi et al., 2024) |
|  | Hemicellulose | 25% |  |
|  | Lignin | 16% |  |
| Coconut Shell | Cellulose | 33.61% | (M et al., 2019) |
|  | Hemicellulose | 29.27% |  |
|  | Lignin | 36.51% |  |
| Giant Reed | Cellulose | 46.50% | (Vasmara et al., 2023) |
|  | Hemicellulose | 29% |  |
|  | Lignin | 25.90% |  |
| Jatropha | Cellulose | 22.29% | (Wever, Heeres and Broekhuis, 2012) |
|  | Hemicellulose | 23.84% |  |
|  | Lignin | 47.60% |  |
| Sugarcane Bagasse | Cellulose | 45% | (Isikgor and Becer, 2015a) |
|  | Hemicellulose | 32% |  |
|  | Lignin | 25% |  |
| Soyabean Straw | Cellulose | 35% | (Martelli-Tosi et al., 2016) |
|  | Hemicellulose | 17% |  |
|  | Lignin | 21% |  |
| Barley straw | Cellulose | 39.00% | (Pradhan et al., 2025) |
|  | Hemicellulose | 26% |  |
|  | Lignin | 18% |  |
| Sorghum residue | Cellulose | 49.43% | (Andrade Alves et al., 2019) |
|  | Hemicellulose | 19.18% |  |
|  | Lignin | 30.42% |  |
| Oat hulls | Cellulose | 35% | (Debiagi, Faria-Tischer and Mali, 2020) |
|  | Hemicellulose | 28% |  |
|  | Lignin | 15% |  |
| Sunflower husk | Cellulose | 46.40% | (Islamova et al., 2023) |
|  | Hemicellulose | 31.60% |  |
|  | Lignin | 17.00% |  |
| Potato waste | Cellulose | 55% |  |
|  | Hemicellulose | 12% | (Soni et al., 2023) |
|  | Lignin | 14% |  |
| Banana peels | Cellulose | 9.60% | (Acevedo et al., 2021) |
|  | Hemicellulose | 9.40% |  |
|  | Lignin | 12% |  |
| Citrus peels | Cellulose | 13.60% | (Ververis et al., 2007) |
|  | Hemicellulose | 6.10% |  |
|  | Lignin | 2% |  |
| Sugarbeet pulp | Cellulose | 50% | (Yasin, Gangan and Panchal, 2025) |
|  | Hemicellulose | 35% |  |
|  | Lignin | 25% |  |
| Almond shells | Cellulose | 38.48% | (Li et al., 2018) |
|  | Hemicellulose | 28.82% |  |
|  | Lignin | 29.54% |  |
| Switchgrass | Cellulose | 40.08% | (Tóth et al., 2023) |
|  | Hemicellulose | 38.41% |  |
|  | Lignin | 12.41% |  |
| Bamboo | Cellulose | 50% | (Alves et al., 2010) |
|  | Hemicellulose | 20% |  |
|  | Lignin | 23% |  |
| Douglas fir | Cellulose | 44% | (Isikgor and Becer, 2015b) |
|  | Hemicellulose | 11% |  |
|  | Lignin | 27% |  |
| Grasses | Cellulose | 40% | (Isikgor and Becer, 2015b) |
|  | Hemicellulose | 50% |  |
|  | Lignin | 30% |  |
| Eucalyptus | Cellulose | 54% | (Alves et al., 2010) |
|  | Hemicellulose | 18.40% |  |
|  | Lignin | 21.50% |  |
| Barley Hulls | Cellulose | 34% | (Isikgor and Becer, 2015b) |
|  | Hemicellulose | 36% |  |
|  | Lignin | 19% |  |
| Spruce | Cellulose | 45.50% | (Isikgor and Becer, 2015b) |
|  | Hemicellulose | 22.90% |  |
|  | Lignin | 27.90% |  |

Table S2. The table summarizes the relative contents of cellulose, hemicellulose, and lignin (expressed as percentage of dry biomass). highlighting compositional variability across feedstocks that influences pretreatment requirements, enzymatic hydrolysis efficiency, and downstream biofuel yields.

| Feedstock | Bioethanol Yield (L/Ton) | Conversion technology | References |
| --- | --- | --- | --- |
| Citrus Peels | 586 | Continuous Immobilized Fermentation | (Mahato et al., 2021) |
| Sugarcane Bagasse | 329 | Fed-batch SSF | (Hemansi and Saini, 2023) |
| Potato Waste | 284 | Organosolv pretreatment, Enzymatic hydrolysis, ethanolic fermentation, and anaerobic digestion | (Soltaninejad, Jazini and Karimi, 2022) |
| Switch Grass | 276 | Liquid hot-water and low-moisture ammonium hydroxide (LMA) pretreatments, Enzymatic hydrolysis, Separate hydrolysis and fermentation (SHF) | (Dien et al., 2018) |
| Corn Stover | 259 | Simultaneous saccharification and fermentation (SSF) | (Molaverdi et al., 2021) |
| Barley Straw | 246 | Separate hydrolysis and co-fermentation (SHCF) | (Díaz, Moya and Castro, 2022) |
| Sorghum Residue | 213 | alkali (NaOH) pretreatment, Enzymatic Degradation, co-culture fermentation | (Punia and Kumar, 2025) |
| Giant Reed | 205 | SHF | (Viola et al., 2015) |
| Wheat Straw | 185 | Simultaneous saccharification and fermentation | (Erdei et al., 2010) |
| Oat Hulls | 179 | Chemical pretreatment with biological conversion methods (enzymatic hydrolysis and fermentation | (Baibakova et al., 2017) |
| Banana Peels | 173.7 | SSF | (Sarkar et al., 2020) |
| Soyabean Molasses | 162.7 | Direct fermentation (submerged alcoholic fermentation using S. cerevisiae) | (Siqueira et al., 2008) |
| Rice Straw | 127 | SHF | (Belal, 2013) |
| Sugarbeet Pulp | 103.5 | Direct fermentation of sugar-rich substrate using S. cerevisiae | (Panella and Kaffka, 2010) |
| Sunflower Husks | 101.4 | SHF using S. cerevisiae | (Havrysh et al., 2023) |
| Acacia Wood | 90 | SSF using S. cerevisiae | (Ko et al., 2012) |
| Jatrpoha | 88.5 | Sequential hydrolytic and fermentation | (Santos, Macedo and Santos, 2014) |
| Coconut Husk | 86 | Combined pretreatment and fermentation using Z. mobilis | Manuscript in progress |
| Eucalyptus Wood | 83 | SSF using S. cerevisiae | (Ko et al., 2012) |
| Almond Shell | 73.5 | SHF using S. cerevisiae | (Kacem et al., 2016) |
| Oil Palm | 66.5 | SHF | (Farid et al., 2019) |

Table S3. Reported bioethanol yields from a range of lignocellulosic and agro-industrial feedstocks. Bioethanol yields are expressed as liters per metric ton of dry biomass (L ton⁻¹) and reflect variability arising from feedstock composition, pretreatment severity, hydrolysis efficiency, and fermentation performance reported in the literature.

Note: All biofuel yields are reported on a feedstock mass basis and were calculated by converting reported concentrations or mass yields into liters per metric ton (L ton⁻¹). Conversions were performed using standard density values (ethanol = 0.789 kg L⁻¹) and established stoichiometric relationships for sugar fermentation.

| Feedstock | Butanol Yield (L/ton feedstock) | Conversion technology | Reference |
| --- | --- | --- | --- |
| Jatropha cake | 556 | SHF + ABE fermentation using c. beijerinckii | (Jiang et al., 2012) |
| Banana peels | 194 | SHCF using co-culture of Saccharomyces cerevisiae and Pichia sp. | (Mishra et al., 2020) |
| Sugarcaen Molasses | 189 | Direct ABE fermentation using Clostridium beijerinckii | (Thanapornsin et al., 2018) |
| Cassava | 185 | ABE fermentation using C. beijerinckii BA101 | (Lépiz-Aguilar et al., 2013; Huang et al., 2019) |
| Palm Oil | 178 | SHF + ABE fermentation Using Clostridium acetobutylicum | (Komonkiat and Cheirsilp, 2013) |
| Sugarcane | 175 | # SHF + ABE fermentation Using Clostridium acetobutylicum GX01 | (Pang et al., 2016) |
| Barley | 173 | SHF + ABE fermentation using Clostridium beijerinckii | (Qureshi et al., 2010) |
| Fruit residue | 173 | ABE fermentation by Clostridium acetobutylicum DSM 792 | (Bardone et al., 2016) |
| Soybean hulls | 172 | SHF + ABE fermentation | (Amaro Bittencourt et al., 2021) |
| Wheat Straw | 170 | ABE using Clostridium beijerinckii P260 | (Qureshi, Saha and Cotta, 2007) |
| potato waste | 168 | SHF + ABE fermentation | (Abedini, Amiri and Karimi, 2020) |
| Rice Straw | 167 | ABE using clostridial acetobutylicum | (Ranjan, Khanna and Moholkar, 2013) |
| Sorghum | 150 | Hybrid pervaporation process | (Cai et al., 2013) |
| food waste | 149 | Direct fermentation using C. sacchroperbutylacetonicum N1-4 | (Wang et al., 2023) |
| Switchgrass | 136 | Hydrothermolysis pretreatment - ABE | (Liu et al., 2015) |
| Corn (starch) | 136 | ABE fermentation (batch fermentation) using Clostridium beijerinckii BA101 | (Qureshi and Blaschek, 2001) |
| Physaria fendleri (Yellow Top) | 131 | SHF + ABE fermentation using Clostridium beijerinckii | (Qureshi et al., 2019) |
| Corn Stover | 98 | SHF + ABE fermentation (ionic liquid pretreatment) using Clostridium saccharobutylicum | (Ding et al., 2016) |
| Sugarcane Bagasse | 58 | SHF + ABE fermentation (acid hydrolysis) using Clostridium beijerinckii | (Jonglertjunya et al., 2014) |
| Algae | 57 | SHF + ABE fermentation (enzymatic hydrolysis of microalgal biomass) | (Yang et al., 2023) |

Table S4. Reported bioethanol yields from a range of lignocellulosic and agro-industrial feedstocks. Bioethanol yield is expressed as liters per metric ton of dry biomass (L ton⁻¹) and reflect variability arising from feedstock composition, pretreatment severity, hydrolysis efficiency, and fermentation performance reported in the literature.

Note: All biofuel yields are reported on a feedstock mass basis and were calculated by converting reported concentrations or mass yields into liters per metric ton (L ton⁻¹). Conversions were performed using standard density values (ethanol = 0.81 kg L⁻¹) and established stoichiometric relationships for sugar fermentation.

| Feedstock | Biohydrogen yield (kg/Ton) | Conversion technology | References |
| --- | --- | --- | --- |
| Switch grass | 30 | Thermochemical gasification | (Sarkar et al., 2014) |
| Giant reed | 18.2 | Dark fermentation | (Vasmara et al., 2025) |
| Almond shell | 15.7 | Hydrothermal gasification | (Safari, Javani and Yumurtaci, 2018) |
| Potato waste | 15.67 | photo fermentation | (Das and Basak, 2025) |
| Corn stover | 12.7 | Dark–photo co-fermentation (DF–PF hybrid process) | (Zhang et al., 2020) |
| Sunflower husks | 7.41 | Dark fermentation using E. coli | (Vanyan et al., 2025) |
| Soyabean straw | 5.42 | Alkaline Pretreatment, Dark fermentation using Clostridium butyricum | (Han et al., 2012) |
| Sorghum residue | 5.39 | Acidic Pretreatment, Fermentation Using Rumincoccus albus | (Ntaikou et al., 2008) |
| Wheat stover | 4.02 | Acidic pretreatment, (SSF) Dark fermentation using mix culture | (Nasirian et al., 2011) |
| Orange peels | 1.87 | Dark fermentation (ultrasound-assisted pretreatment) | (Rouabhia, Álvarez-Gallego and Fernández Güelfo, 2025) |
| Sugarbeet pulp | 1.31 | Dark fermentation (ultrasound-assisted pretreatment) | (Rouabhia, Álvarez-Gallego and Fernández Güelfo, 2025) |
| Banana peels | 0.79 | Dark fermentation | (Abd-Alla et al., 2025) |
| Jatrpoha | 0.78 | Dark fermentation | (Kumar and Lin, 2014) |
| Sugarcane bagasse | 0.564 | Dark fermentation | (Fangkum and Reungsang, 2011) |
| Oil palm | 0.423 | Dark fermentation | (Atif et al., 2005) |
| Rice husk | 0.255 | Dark fermentation | (Lay et al., 2019) |
| Barley straw | 0.027 | # sequential dark and photofermentation | (Özgür and Peksel, 2013) |

Table S5. Comparative biohydrogen yields from diverse lignocellulosic residues and energy crops. Biohydrogen production is reported as kilograms of hydrogen per metric ton of dry feedstock (kg ton⁻¹), reflecting differences in biomass composition, pretreatment strategy, microbial pathway, and process configuration (e.g., dark fermentation, photofermentation, or integrated systems).

Note: All biohydrogen yields are reported on a dry feedstock mass basis and were calculated by converting reported volumetric or molar hydrogen production values into kilograms per metric ton (kg ton⁻¹) of biomass. Conversions were performed using standard gas properties at standard temperature and pressure (STP), where 1 mol H₂ = 2.016 g and 1 Nm³ H₂ = 0.0899 kg.

| Feedstock | Biodiesel Yield (L/ton) | Conversion technology | references |
| --- | --- | --- | --- |
| Rapeseed/Canola | 1,125 | Base catalyzed Transesterification | (Demirbas, 2009; Gong et al., 2014) |
| Palm oil | 1085 | heterogeneous catalyst Transesterification | (Anguebes-Franseschi et al., 2016) |
| Sunflower | 1080 | Transesterification | (Granados et al., 2007) |
| Animal fat | 1057 | Acid-catalyzed esterification followed by transesterification | (Gandure et al., 2017a; Toldrá-Reig, Mora and Toldrá, 2020) |
| Castor | 1045 | Alkali-catalyzed transesterification | (Keera, El Sabagh and Taman, 2018; Zulqarnain et al., 2021) |
| Waste cooking oil | 1023 | Esterification followed by transesterification | (Gandure et al., 2017; Veljković et al., 2021) |
| Neem | 1,020 | Alkali-catalyzed transesterification (ethanolysis) | (Hamadou et al., 2020; Iqbal et al., 2024) |
| Mustard | 434 | Co-solvent assisted alkali-catalyzed transesterification | (Fadhil, Saleh and Altamer, 2020) |
| Karanja (Pongamia) | 366 | Alkali-catalyzed transesterification | (Dwivedi, Jain and Sharma, 2011; Vijay and Chandra, 2024) |
| Jatropha | 342 | Transesterification | (Otieno et al., 2022) |
| cotton seed | 220 | Transesterification | (Onukwuli et al., 2017; Venkatesan, John and Sivamani, 2017) |
| Rice bran oil | 190 | Alkali-catalyzed transesterification | (Ju and Ramjan Vali, 2005; Bello and Oluboba, 2014) |
| Soybean | 161 | base-catalyzed transesterification | U.S. Department of Energy. How Much Energy Does It Take to Make a Gallon of … (DOE biodiesel yield data) |
| Corn stover | 127 | Microbial lipid production + transesterification | (Dai et al., 2019; Langholtz et al., 2022) |

Table S6. Comparative biodiesel yields from oil-bearing crops, animal fats, waste oils, and selected lignocellulosic feedstocks. Biodiesel production is expressed as liters per metric ton of feedstock (L ton⁻¹), illustrating the strong dependence of yield on lipid content, oil extractability, and feedstock type.

Note: All biodiesel yields are reported on a feedstock mass basis and were calculated by converting reported oil yields or fatty acid methyl ester (FAME) contents into liters per metric ton (L ton⁻¹) of dry biomass. Conversions were performed using standard density values for biodiesel/FAME (ρ ≈ 0.88–0.90 kg L⁻¹, typically 0.88 kg L⁻¹ at 15–20 °C).

Table S7.

| Section | Key AI/ML Applications & Examples | References |
| --- | --- | --- |
| Feedstock & Pretreatment | ML models optimize pretreatment; hydrochar characterization | (Tušek et al., 2024) |
| Fermentation Optimization | Predictive models reduce experimentation; biodiesel yield improved (84–98%) | (Helleckes et al., 2022; Awogbemi and Kallon, 2023) |
| Strain Engineering | Hybrid/training ML models enhance metabolic pathway design | (Butean et al., 2025) |
| Downstream Processing | AI enables predictive separation control (e.g., distillation) | (Butean et al., 2025) |
| Integration & Supply Chain | AI optimizes supply chain, integration, reduces carbon footprint | (Chauhan et al., 2025) |
| Carbon Capture & Valorization | AI accelerates material discovery, improves CCS efficiency 10–30%, optimizes CO₂ conversion | (States Department of Energy, 2024; Khalili et al., 2025; Tawalbeh et al., 2025) |

Table S7. Artificial Intelligence and Machine Learning Applications Across the Biofuel Value Chain. Summary of key AI and machine learning applications in modern biorefinery systems, highlighting representative use cases and performance metrics for feedstock, pretreatment, and fermentation optimization.