Table S1 MR imaging parameters

| Sequence | Imaging Plane | TR(ms)/TE(ms) | Slice Thickness(mm) | Gap (mm) | Field of View(mm) | Acquisition matrix (phase × frequency) | Number of Excites | b-values (sec/mm2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GE signa excite HD 3.0T |  |  |  |  |  |  |  |  |
| FS FSE pelvic T2WI | Axial | 5400/106.5 | 5 | 1 | 400 | 320*256 | 2 | - |
| FS FSE retroperitoneal T2WIa | Axial | 4900/100.2 | 5 | 1 | 400 | 320*256 | 2 | - |
| FSE T2WI | Axial oblique | 5200/134.6 | 4 | 1 | 220 | 320*256 | 1 | - |
| DWI | Axial | 4700/64.4 | 5 | 1 | 400 | 128*128 | 2 | 0, 800 |
| Multi-phase contrast enhanced MRI | Sagittal | 3.8/1.8 | 3 | 0 | 300 | 320*192 | 0.7 | - |
| GE Discovery HD750 3.0T |  |  |  |  |  |  |  |  |
| FS FSE pelvic T2WI | Axial | 4734/90.3 | 5 | 1 | 380 | 320*256 | 2 | - |
| FS FSE retroperitoneal T2WIa | Axial | 4734/87.9 | 5 | 1 | 380 | 320*256 | 2 | - |
| FSE T2WI | Axial oblique | 6241/121.6 | 4 | 0.4 | 200 | 320*256 | 1 | - |
| DWI | Axial | 4000/56 | 5 | 1 | 380 | 128*128 | 6 | 0, 800 |
| Multi-phase contrast enhanced MRI | Sagittal | 3.8/1.8 | 3 | 0 | 340 | 320*192 | 0.7 | - |

aRetroperitoneal T2WI sequence was performed from the renal hilum level to the first slice of the pelvic axial T2WI sequence. Axial oblique T2WI, multi-phase contrast enhanced MRI, and DWI were used to the extraction of radiomics features. Retroperitoneal T2WI and pelvic axial T2WI were used to evaluate the retroperitoneal and pelvic lymph node status.

FSE fast spin echo, FS fat suppression, TR repetition time, TE echo time, T2WI T2-weighted imaging, DWI diffusion-weighted imaging, MRI magnetic resonance imaging

Appendix E1. Radiomics score (Rad-score) calculation formula for PFS estimation

Rad-PFS = 0.005 * T2WI_original_glszm_LargerAreaLowGrayLevelEmphasis + 0.059 * T2WI_original_glszm_SizeZoneNonUniformity - 0.283 * ADC_original_shape_Sphericity + 0.133 * ADC_original_firstorder_Kurtosis - 0.105 * ADC_original_firstorder_Mean - 0.590 * ADC_original_glcm_ClusterShade - 0.342 * ADC_original_glcm_DifferenceVariance + 0.134 * ADC_original_glcm_Imc1 + 0.243 * ADC_original_glcm_Idmn + 0.314 * Arterial-phase_original_gldm_LowGrayLevelEmphasis + 0.055 * Delayed-phase_original_firstorder_TotalEnergy + 0.117 * Delayed-phase_original_glszm_ZoneEntropy

Appendix E2. Radiomics score (Rad-score) calculation formula for OS estimation

Rad-OS = -0.114 * ADC_original_firstorder_Maximum - 0.502 * ADC_original_glcm_ClusterProminence - 0.426 * ADC_original_glcm_DifferenceVariance - 0.127 * ADC_original_glcm_JointEntropy + 0.211 * ADC_original_glcm_Imc1+0.211 * Arterial-phase_original_shape_Maximum2DDiameterColumn + 0.094 * Arterial-phase_original_firstorder_Skewness + 0.456 * Arterial-phase_original_glszm_LowGrayLevelZoneEmphasi + 0.093 * Arterial-phase_original_gldm_LowGrayLevelEmphasis + 0.354 * Delayed-phase_original_firstorder_TotalEnergy + 0.008 * Delayed-phase_original_glrlm_RunLengthNonUniformity

Appendix E3. Risk score calculation formula for PFS estimation

Risk score(PFS) = 0.329 * T stage + 0.361 * LNM position + 0.789 * Rad-PFS

Appendix E4. Risk score calculation formula for OS estimation

Risk score(OS) = 0.447 * T stage + 0.278 * LNM position + 0.832 * Rad-OS